3GPP TSG SA WG2 #166AH-E	            S2-2500207

20 - 24 January 2025, Electronic 	(revision of S2-25aaaa)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1539 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Update to DC interworking with MTSI UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editorial changes, inconsistencies, and linguistic enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial changes and linguistic enhancements. Inconsistenceis relates to the interworking event specified in claue AD.2.5.2 Application Data Channel requires interworking is established; Detected when an Application Data Channel that requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel.. This event is detected when an IMS is in the process of being established, and a Notify is sent out as per clause AC.7.9.3. However, clause AC.7.9.4 includes 2 notifications related to the same event, one initiated by the IMS AS when the session is in the process of being established and the second one when the session is established as this is needed for this use case. The second Notify is not aligned with the event description. As is stands, looks like the event should report a Notify when the session is in the process of being established and when the session is established. Although only one is needed per use case, this is the one way to support the 2 cases with a single event otherwise 2 distinctive events are required, and the AS has to subscribe to both. Comments are inserted in the text. There is also additional inconsistency since it is unclear in AC.7.9.4 how the communication between UE#1 and the DC AS is established to enable interworking. The text includes the comments relevant to this aspect. |
| Consequences if not approved: |  | Difficult readability, and inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.9.0, AC.7.9.1, AC.7.9.2, AC.7.9.3, AC.7.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AC.7.9.0	General

This clause describes the IMS data channel setup signalling procedures to support interworking with MTSI UE as defined in TS 26.114 [76].

When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, a bootstrap data channel is set up between the originating DCMTSI UE and the originating network (i.e. DCSF and MF), and an MMTEL session for the other media (e.g. audio, video, any MMTEL media other than data channel) may be set up with the terminating side based on the SDP response from the terminating MTSI UE. This is described in clause AC.7.9.1.

The originating DCMTSI UE may initiate an application data channel associated with the MMTEL session between the DCMTSI UE and the MTSI UE (e.g. for real-time screen sharing). The data channel media sent from the originating DCMTSI UE is transcoded to MMTEL media and transferred to the terminating MTSI UE via the associated RTP stream; equally the MMTEL media sent from the terminating MTSI UE is transcoded to data channel media and transferred to the originating DCMTSI UE via the associated application data channel stream. This is described in clause AC.7.9.2.

A DCMTSI UE that requires interworking with an MTSI UE, may initiate an application data channel setup with the DC AS for data channel application configured to support interworking with an MTSI UE. The DC AS subscribes to the IMS AS to be notified about any application data channel event for data channel applications that require interworking with an MTSI UE. This is described in clause AC.7.9.3.

A DC AS may initiate data channel with a terminating DCMTSI UE  when it requires interworking with an MTSI UE that originated the IMS session. The DC AS subscribes to the IMS AS to be notified when any IMS session is established between any MTSI UE and the DCMTSI UE as the callee. In this case the callee is configured as an interworking provider (e.g. customer service center). This is described in clause AC.7.9.4.

# * * * * Next change * * * *

### AC.7.9.1	Bootstrap Data Channel Setup Signalling Procedure

Figure AC.7.9.1-1 depicts a signalling flow diagram for establishing a bootstrap data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE. In Figure AC.7.9.1-1, the procedure starts with the INVITE including SDP offer which includes audio and data channel media and represent the minimum SDP offer to initiate this procedure, however it is not limited only for that media (e.g. the video offer may be included).

Figure AC.7.9.1-1: Bootstrap Data Channel set up Signalling Procedure for IMS data channel interworking with MTSI UE

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains SDP offers for audio, and the bootstrap data channel with bootstrap DC stream ID.

2.	Data channel management procedure in originating network is executed as per steps 2~10 in Figure AC.7.1-1.

3-4.	IMS AS sends the INVITE request to remote network with SDP including data channel media for terminating network and UE #2.

5-7.	UE#2 and terminating network returns bootstrap DC negotiation result to originating network. The port number of bootstrap data channel in the SDP answer is set to zero implying that the terminating UE #2 does not support IMD DC or is not acceping the establishment of bootstrap data channel.

8.	IMS AS requests MF to update the local DC media resources accordingly.

9-11.	When IMS AS sends the bootstrap DC negotiation result to DCSF, since the port of bootstrap data channel for UE #2 is set to zero, the DCSF knows that the terminating network or UE #2 does not support IMS DC or does not want to use IMS data channel in this session.

12-18.	These procedures are same as step14~20 in Figure AC.7.1-1.

19. The bootstrap data channels have been established only between originating MF and UE#1.

20.	The subsequent procedure in clause AC.7.1-1 applies.

# * * * *Next change * * * *

### AC.7.9.2	Signalling Procedure of Application Data Channel Interworking via MF

Figure AC.7.9.2-1 depicts a signalling flow diagram for establishing the application data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, providing interworking of application data channel to MTSI UE via MF. This procedure enables the transcoding to be performd by the MF, upon instructions by the DCSF, between the data channel media and the MMTEL media such as audio and video e.g. Real-time Screen Sharing.

Figure AC.7.9.2-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via MF

1-3.	UE#1 initiates a re-INVITE request to establish an application data channel in which the application data channel and data channel application information (for instance, application ID) is included. The IMS AS validates user subscription data to determine whether this request should be notified to the DCSF as stated in Clause AC.7.2.

4.	After receiving the notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information as application ID) in the notification and/or DCSF service specific policy.

5-7.	Since the DCSF is aware the terminating network or UE #2 does not support or does not want to use IMS data channel in this session, the DCSF determines that interworking of the application data channel media to MTSI UE is needed and possible, based on the application information, i.e. appliation id. The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for the application data channels on originating side and video media on terminating side. The DCSF also includes the instruction on how to transcode data channel media to video media. The IMS AS invokdes Nmf_MediaResourceManagement to the MF to allocate resource and transcode data channel media to video media according to DCSF instruction.

NOTE:	The media instruction enables the MF to perfrorm transport protocol translation from DC protocol (UDP/DTLS/SCTP) on one side and video media (RTP/AVPF) on the other.

8-9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1. The DCSF stores the media resource information and responds to the request received in step 3.

10-11.	After the media resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF.

12-14.	After terminating network side negotiation, UE#2 and terminating network returns 200 OK for the re-INVITE.

15-16.	DCSF is notified about the terminating network media negotiation result.

17-21.	The IMS AS sends 200 OK for the re-INVITE initiated by UE#1 and UE#1 returns ACK.

22.	UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes the application data channels with the MF.

23-24.	The IMS AS reports media negotiation result to the DCSF, and reports that anchoring video stream is successful. The DCSF responds  to enable MMTel AS to continue the session.

25.	The IMS AS sends a Media Resource Management Update Request to the MF to associate application data channel stream between UE#1 and the MF with the allocated video streams between UE#2 and the MF.

The application data channels between the MF and UE#1 are associated with the RTP video streams between the MF and UE#2. The RTP stream of UE#1's (e.g. screenshare sequence) is received by UE#2 from video streams.

# * * * * Next change * * * *

### AC.7.9.3	Signalling Procedure of Application Data Channel Interworking via DC AS for a Terminating MTSI UE

Figure AC.7.9.3-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to DC application server, providing interworking of application data channel, supported by the DC AS, to a MTSI UE via the DC application server.

In this case, the originating DCMTSI UE sends the data channel application media to the DC AS, which performs the necessary interworking actions on the application media for the terminating MTSI UE.

Figure AC.7.9.3-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via DC application server

The steps in the call flow are as follows:

1. DC AS subscribes via the NEF for the event “Application Data Channel requires interworking is established” to be notified when any application DC for a DC application that requires interworking is established.

1a. The DC AS subscribes to the NEF by using Nnef_ImsEE_Subscribe Request (event ID: Application Data Channel requires interworking is established, DC Interworking Information). The DC Interworking Information includes the application ID(s) of the DC application which requires interworking with MTSI UE.

1b. After performing the necessary authorization, the NEF responsds to DC AS via Nnef_ImsEE_Subscribe Response.

1c. The NEF locates the IMS AS(s) via NRF.

1d. The NEF subscribes to all the IMS ASs as requested by DC AS in step 1a, via Nimsas_ImsEE_Subscribe Request.

1e. The IMS AS responds to NEF via  Nimsas_ImsEE_Subscribe Reponse.

2. IMS session and bootstrap data channel have been established as per clause AC.7.1. Selected DC application(s) have been downloaded to UE#1.

The DCSF knows the UE#2 or terminating network does not support IMS DC, or is not willing to use IMS DC based on procedure in clause AC.7.9.1. Hence, the bootstrap data channel is established only between the UE#1 and the originating network.

3-5.	The UE#1 initiates a P2P application DC. Steps 1-2 of clause AC.7.2.1 apply.

6. After receiving the session event notification from the IMS AS, the DCSF determines that IMS DC interworking for the requested DC application is to be performed based on the application information, i.e. appliation id, the remote network or UE#2 not supporting IMS DC, or not willing to use IMS DC. The DCSF changes P2P application DC to P2A application DC.

6a. The DCSF informs the IMS AS that IMS DC interworking action is required for the requested DC application and the UE#2 side Information. The UE#2 side Information may include the SIP URI of the UE#2 or Called ID.

6b. The IMS AS notifies the NEF that the DC Interworking Required event is detected with the UE#2 side Information via Nimsas_ImsEE_Notify Request.

6c. The NEF responds to the IMS AS via Nimsas_ImsEE_Notify Response.

6d. The NEF notifes the DC AS that the DC Interworking Required event is detected accompanied by the UE#2 side Information via Nnef_ImsEE_Notify Request.

6e. The DC AS responds to the NEF via Nnef_ImsEE_Notify Response.

7.	The DCSF instructs the MF via the IMS AS to establish the media resource of the P2A application DC and the DC connection is established between UE and the DC AS. During P2A connection establishment, the DC AS considers the information received in step 6d. Steps 5-14 of clause AC.7.2.2 apply.

8-9.	IMS AS sends the re-INVITE to remote network side and UE#2, which does not include bootstrap data channel information and application data channel media components in the SDP.

10.	Steps 17-26 of clause AC.7.2.2 apply.

11.	UE#1 uses the established application data channel to provide DC application specific information e.g. sends the file (e.g. a photo) to the DC AS,  which will be sent to UE#2 from DC AS.

12.	DC AS performs interworking actions based on the notification received in step 6d and the data received in step 11.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 11 and sends the hyperlink to UE#2 through a SMS, where the user of UE#2 can download the file using the hyperlink if he/she wants.

# * * * * Next change

### AC.7.9.4	 Signalling Procedure of Application Data Channel Interworking via DC AS for originating MTSI UE

Figure AC.7.9.4-1 depicts a signalling flow diagram for establishing an application data channel from a DC application server to DCMTSI UE, providing interworking of application data channel, via DC application server, to an IMS session initiated by an MTSI UE towards a DCMTSI UE.

The DC AS initiates the procedure for application data channel setup between the serving network of the DCMTSI UE and the DCMTSI UE (terminating side). The terminating DCMTSI UE sends the data channel application media to the DC AS. The DC AS performs the necessary interworking actions on the application media towards the originating MTSI UE.

Figure AC.7.9.4-1: Application Data Channel set up signalling procedure for IMS data channel interworking with originating MTSI UE via DC application server

The steps in the call flow are as follows:

1. DC AS subscribes via the NEF for the event “Application Data Channel requires interworking is established” to be notified when an application DC for a DC application supported by the DC AS that requires interworking is established.

1a. The DC AS subscribes to the NEF via Nnef_ImsEE_Subscribe Request (event ID: Application Data Channel requires interworking is established, DC Interworking Information). The DC Interworking Information includes the UE#2 side Information which is configured as an interworking provider e.g. the service number of the customer service center.

1b. After performing the necessary authorization, the NEF responds to DC AS via Nnef_ImsEE_Subscribe Response.

1c. The NEF locates all the IMS AS(s) via NRF.

1d. The NEF subscribes to all the IMS AS(s) as requested by DC AS in step 1a, via Nimsas_ImsEE_Subscribe Request.

1e. The IMS AS responds to NEF via Nimsas_ImsEE_Subscribe Reponse.

2. The UE#1, an MTSI UE, initiates an IMS session with the UE#2 e.g. an enterprize service number.

3. The IMS AS determines that IMS DC interworking for the DC application served by the DC AS is to be performed based on the SDP offer received in step 2, e.g. the Called ID that matches the service number included in the UE#2 side Information.

3a. The IMS AS notifies the NEF that the DC Interworking Required event is detected with the UE#1 side Information by using Nimsas_ImsEE_Notify Request. The UE#1 side Information may include the SIP URI of the UE#1 or Calling ID.

3b. The NEF responses to the IMS AS by using Nimsas_ImsEE_Notify Response.

3c. The NEF notifies to the DC AS that the DC Interworking Required event is detected with the UE#1 side Information by using Nnef_ImsEE_Notify Request.

3d. The DC AS responds to the NEF via Nnef_ImsEE_Notify Response.

4. The IMS AS sends the INVITE to remote network side and UE#2, and the IMS session establishment procedure resumes.

5. After IMS session establishment, the IMS AS notifies the NEF that the IMS Session is established for the UE where IMS DC interworking is to be performed.

5a. The IMS AS notifies the NEF that the IMS Session is established with the UE#1 side Information by using Nimsas_ImsEE_Notify Request.

5b. The NEF responds to the IMS AS via Nimsas_ImsEE_Notify Response.

5c. The NEF notifies the DC AS that the IMS Session is established with the UE#1 side Information by using Nnef_ImsEE_Notify Request. The notification of IMS Session establishment is explicitly subscribed in step 3d.

5d. The DC AS responds to the NEF by using Nnef_ImsEE_Notify Response.

6. DC AS requests the serving network of the UE#2 (terminating network) to add a local data channel for UE#2, based on the procedure of Adding Data Channel to an existing IMS Session as described in clause AG.2.1. The IMS AS of the serving network of the UE#2 does not send the INVITE to UE#1.

7. P2A Application Data Channel is set up between the serving network and the UE#2 based on the procedure described in the clause AC.7.2.2. During P2A connection establishment, the DC AS considers the information received in step 3c.

8. UE#2 uses the established application data channel to provide DC application specific information (e.g. customer service menu) to the DC AS, which will be sent to UE#1 from DC AS.

9. DC AS performs interworking actions based on the notification received in step 3c and the data received in step 8.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 8 and sends the hyperlink to UE#1 through a SMS, where the user of UE#1 can download the file using the hyperlink if he/she wants.

# * * * * End of changes * * * *