3GPP TSG-SA2 Meeting #166AH-E	S2-2500209

# January 20-24,   2025, Electronic				 							  (revision of S2-25CCCCC)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1541 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Update to Network Capability Exposure Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Resolve Editor notes, and make some corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Expand the procedure updating an existing IMS session for a Data Channel UpdateAdd the procedures of IMS data channel capabilies exposure. - Complete the procedure for creating a stand alone IMS DC - Complete the procedure for terminating an IMS DC - Correct the Session delete procedue. The deletion request need not be reported to the DCSF. There is no reason why the DCSF would reject a session deletion request. Also unclear what will the DC AS do in case of a rejection. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing and incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.2.2, AG.1, AG.2.1, AG.2.2, AG.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. 

Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |
| ExternalSessionUpdateEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| ExternalSessionCreateEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |

# * * * * Next change * * * *

Annex AG (normative):
IMS Capability Exposure Framework in IMS

# AG.1	Description

The following scenarios utilizing IMS data channel capability exposure services are depicted in clause AG.2:

-	Adding, removing or updating the data channel to an existing IMS session, for which Nimsas_SessionManagement_Update is used

-	Creating a standalone data channel session, for which Nimsas_SessionManagement_Create is used.

-	Terminating a standalone data channel session, for which Nimsas_ImsSessionManagement_Delete is used.

The procedures can be triggered via the NEF on the originating or terminating side.

# * * * * Next change * * * *

# AG.2	IMS Data Channel capability exposure procedures

## AG.2.1	Updating an existing IMS session for a Data Channel

Figure AG.2.1-1 shows the procedure of adding, removing or updating bootstrap data channel(s) or application data channel(s) to an existing IMS session.

Figure AG.2.1-1: Procedure of adding data channel(s) to an existing IMS Session

0.	DC AS may determine to add or remove data channel from an existing IMS session based on local policy or event notification of the related IMS Session to which the DC AS subscribesd.

NOTE 1:	It is assumed that the  non 3GPP APIs (e.g. OMA specified) can be used for non-DC session notification in step 0. The IMS Framework in Annex AD can also be used for that purpose. Hence the received notification can include different parameters depending on the deployed method for receiving notifications.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF for adding or removing bootstrap data channel(s) or application data channel(s) to the exisiting IMS session. The request includes the session ID for the session to be updated, calling ID, called ID, media operative set including the instructions for the updated media e.g BDC, ADC. Optionally the request may include AS instance (s) for the subscribers in the session. 

The DC AS may include a Notification Target Address and Correlation ID in the Nnef_ImsSessionManagement_Update request to be notified of the progress of the session update request.

2.	Unless provided in the incoming request to the NEF, the NEF uses the Nhss_ImsUECM_Get service operation to retrieve the IMS AS instance serving the targeted user and discovers the IMS AS instance from NRF if needed.

3.	If the NEF aurhorizes the request, the NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 to add, remove, or update bootstrap data channel(s) or application data channel(s) in the specific session. The NEF includes the information received from the DC AS in its request to the NEF.

4.	The IMS AS-2 validates the user subscription data (for the targeted user) and checks the IMS DC capability of UE and determines whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription for IMS data channel or does not have the IMS DC capability, then the request is rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the requested IMS data channel (bootstrap or application data channel) is already established, then the request is rejected with an appropriate cause and the following steps are skipped.

5. 	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6.	The IMS AS-2 implictly sends an event notification to the DCSF2 which instructs IMS-AS-2 what to do:

6a.	IMS AS-S2 sends the Nimsas_SessionEventControl_Notify request to the DCSF-2 with the event set to ExternalSessionUpdateEvent, to add BDC, ADC as an example. The notification includes the necessary parameters based on the information extracted from paratmeters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

NOTE X:	IMS AS-2 extracts from the Media operation set parameter the needed information to construct the Media Info list parameter included in the Nimsas_SessionEventControl_Notify request.

6b. 

The DCSF-2 instructs the IMS AS-2 to set up the necessary media resources for the BDC, and ADC as per clauses AC.7.1 and AC.7.2 where applicable.

6c.	IMS AS-2 may interact with the MF-2 to allocate data channel resources. The procedures depicted in clause AC.7 are reused for interaction with MF-2.

6d.	IMS AS-2 returns Nimsas_MediaControl_MediaInstruction response to DCSF-2.

6e.	DCSF-2 returns Nimas_SessionEventControl_Notify response to the IMS AS-2.

NOTE 2:	The type of data channel established is dependent on the instructions provided by DCSF. In this example BDC, and ADC are included.

7.	The IMS AS-2 generates a re-INVITE request(s) towards UE-A and UE-B in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions:

7a.

The IMS AS-2 sends the re-INVITE request to UE-A to set up BDC with the DSCF-2, and ADC with UE-B in this example. The MF-2 allocated media resources are also included in the SDP offer.

7b.	 UE-A returns SIP 200 OK to IMS AS-2.

7c.	UE-A downloads the necessary application over the BDC.

7d.	  The IMS AS-2 sends the re-INVITE request to UE-B to set up ADC with UE-A in this example. The MF-2 allocated media resources are also included in the SDP offer.

7e.	 UE-B returns SIP 200 OK to IMS AS-2.

NOTE 3: The IMS DC session update procedures described in clauses AC.7.1 and AC.7.2 are followed.

The DC media negotiation is completed, as per clause AC.7.1 and AC.7.2  and corresponding data channel(s) are established.

9. 	The IMS AS-2 notifies the DCSF-2 for the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicited in the Figure.

If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Update request in Step 1, the IMS AS notifies the NEF for the progress of the session update, based on Table AA.2.4.4.5-1,  with the Nimsas_SessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session update with the Nnef_SessionManagement_Notify request service operation.

# * * * * Next change * * * *

## AG.2.2	Establishing IMS session with standalone Data Channel

Figure AG.2.2-1 shows the establishment procedure of a standalone data channel session.

Figure AG.2.2-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF to create a standalone DC session with UE A. The request includes calling ID, called ID, media information set including the instructions for the  BDC, or ADC associated with the standalone IMS DC session. Optionally, the request may include AS instance (s) for the subscribers in the session. 

The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session update.

2.	If needed, the NEF queries the HSS for the address of the IMS AS serving the called number e.g. service number of the enterprise, if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF rejects the Nnef_ImsSessionManagement_Create request from the DC AS with an error response and the subsequent steps are skipped.

3.	If the NEF aurhorizes the request, the NEF sends a Nimsas_SessionManagement_Create request to the IMS AS-2 to create a standalone DC session.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE to determine whether the data channel request should be notified to DCSF-2. If the service number does not have subscription for IMS data channel, then the request shall be rejected with appropriate cause. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Create response to the NEF.

5. 	The NEF returns a successful Nnef_ImsSessionManagement_Create response to the DC AS.

6. 		The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from paratmeters as received in Step 3.

The DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The procedures in clause AC.7 are reused to interact with MF.

NOTE :		Step 6 follows the clause AG.2.1

7.

The IMS AS-2 sends the INVITE request(s) according to the IMS DC session establishment procedures described in clauses AG.2.1, AC.7.1 and AC7.2.

8.	The DC media negotiation is completed and corresponding standalone IMS DC is established.

9. 	The IMS AS2 notifies the DCSF-2 for the IMS session events with the Nimsas_SessionEventControl_Notify service with the event set to SessionEstablishmentRequestEvent. This is not depicited in the Figure.

If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Create request in Step 1, the IMS AS notifies the NEF  with the progress of the session creation with the Nimsas_ImsSessionManagement_Notify service operation, based on Table AA.2.4.4.x-1.

10.	The NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify service operation.

# * * * * Next change * * * *

## AG.2.3	Terminating standalone Data Channel session

Figure AG.2.3-1 shows the terminating procedure of a standalone data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

Figure AG.2.3-1: Terminating of a standalone data channel Session

1.	The DC AS sends a Nnef_ImsSessionManagement_Delete request to the NEF requiring terminating a standalone DC session with UE A. The request incudes the Session ID to be terminated.

2.	Unless provided in the incoming request to the NEF, the NEF queries the HSS for the address of the IMS AS-2 serving the specific UE if needed.

3.	The NEF sends a Nimsas_ImsSessionManagement_Delete request to the IMS AS-2 requiring to terminate the standalone data channel session identified by the Session ID.

4.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Delete response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Delete response to the DC AS.

6.	The IMS AS-2 sends the BYE request according to existing IMS procedures to terminate the data channel session.

7.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request including the event SessionTerminationEvent to notify  DCSF-2 that a session is terminated.

8.	If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Delete request in Step 1, the IMS AS notifies the NEF for the progress of the session deletion with the Nimsas_SessionManagement_Notify service operation, based on Table AA.2.4.4.5-1.

9.	The NEF notifies the DC AS for the progress of the session deletion with the Nnef_SessionManagement_Notify service operation.

# * * * * End of changes * * * *