3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500223

# Jan. 20 – 24, 2025, E-meeting															(Revision of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5903 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. the encrypted traffic handling is not limited to XR traffic, the title of clause 5.37 has indicated the traffic to be supported, so it applies to its subclause: “5.37 Support for high data rate low latency services, eXtended Reality (XR) and interactive media services”. 2. Only security protection for QUIC-aware and UDP option solution needs to be defined in TS33.501, but not for all the solutions. 3. remove some references for MoQ to make the text cleaner, since reference in one place is enough, no need to add reference everywhere. 4. some other clarifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. remove the limitation to only XR traffic. 2. make it clear that the following only applies to QUIC-aware and UDP option “The media related information is encrypted, and integrity protected between the UPF and the AS, as defined in TS 33.501 [29].” 3. remove some references for MoQ. 4. other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The content of TS is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.1, 5.37.9.1, 5.37.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

### 5.37.5	PDU Set based Handling

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

-	Media over QUIC (MoQ) Transport, as defined in IETF draft-ietf-moq-transport [201] (as further described in clause 5.37.9.2).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

NOTE 2:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

For end-to-end encrypted traffic, PDU Set Information is received as media related information from the Application Server, see clause 5.37.9.

NOTE 3:	Identification of PDU Set information for end-to-end encrypted traffic is supported in the DL direction only.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 4:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 5:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for a UE registered in 3GPP access for single access PDU Session with IP PDU Session Type, for a UE registered in untrusted or trusted non-3GPP accesses for single access PDU Session with IP PDU Session Type, and for a 5G-RG registered in W-5GAN for single access PDU Session with IP PDU Session Type. The support of PDU Set based handling in 5G-RG is specified in TS 23.316 [84].

# * * * * Next change * * * *

5.37.9	Support of transferring media related information over N6

5.37.9.1	General

This clause describes the procedure to support transferring of media related information (i.e. information related to the media being transmitted, e.g. PDU Set Information, Data Burst Size, etc, either explicitly provided by the AS or inferred from protocol headers or payload) over N6. Such support necessary is when the end-to-end connection for media traffic between the UE and Application Server is fully encrypted thus UPF not being able to identify, for example, PDU set information.

On-path N6 signalling methods are used to transfer media related information over N6 where media related information is sent together with the encrypted payload allowing media related information to be received synchronously with the media packet. The following on-path N6 signalling methods are supported:

-	Using Media over QUIC (MoQ), as defined in IETF draft-ietf-moq-transport [201], whose usage is further defined in clause 5.37.9.2.

-	Use of connect-UDP for Using Proxying-UDP-in-HTTP, as defined in IETF RFC 9298 [170] and QUIC-Aware Proxy, as defined in draft-ietf-masque-quic-proxy [200], whose usage is further defined in clause 5.37.9.3

-	Using UDP-Options, as defined in IETF draft-ietf-tsvwg-udp-options [202], whose usage is further defined in clause 5.37.9.4.

When Media over QUIC is used the UPF identifies PDU Set Information and End of Data Burst Indication from media related information specified in [201].

When Proxying-UDP-in-HTTP RFC 9298 [170] or QUIC-Aware Proxy [200] or UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], is used, the following media related information are supported:

-	PDU Set Information (as described in clause 5.37.5.2).

-	End of Data Burst Indication (as described in clause 5.37.8.3).

-	Expedited Transfer Indication (as described in clause 5.37.9).

-	Data Burst Size (as described in clause 5.37.9).

-	Time to Next Burst (as described in clause 5.37.9).

The mechanisms defined in clause 5.37.9 address only UE to XRM server (AS) communication and do not address UE-UE XRM communications. The mechanisms defined in clause 5.37.9 support only downlink media related information delivery to the UPF.

When QUIC-Aware Proxy [200] or UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], is used, The media related information is encrypted and integrity protected between the UPF and the AS, as defined in TS 33.501 [29].

The mechanisms defined in clause 5.37.9 address only UE to XRM server (AS) communication and do not address UE-UE XRM communications. The mechanisms defined in clause 5.37.9 support only downlink media related information delivery to the UPF.

Editor's note:	Other parameters related to KI#4 are FFS.

# * * * * Next change * * * *

5.37.9.2	Usage of Media Over QUIC in order to Handle end-to-end encrypted flows

When a PSA UPF that supports MoQ relay functionality has been selected, encrypted Metadata associated with traffic can be transported between the UPF and the AS via Media over QUIC (MoQ), as defined in IETF draft-ietf-moq-transport [201].

A PSA UPF supporting the MoQ relay functionality can be selected by SMF with the consideration of the DNN and S-NSSAI provided by UE.

NOTE 1:	The application client on the UE uses MoQ Transport protocol and establishes a MoQ connection using the address of MoQ Relay between the application client and MoQ relay in UPF as defined in MoQ.

The SMF may get the address of MoQ Relay from UPF during N4 session management procedure as defined in the clause 4.4.1 of TS 23.502 [3] or the SMF may get MoQ relay address related to the PSA UPF from the NRF as described in clause 5.2.7.3.2 of TS 23.502 [3]. In the former case, the SMF indicates the PSA UPF to provide MoQ relay address via N4 Session Establishment/Modification Request and the PSA UPF returns MoQ relay address via N4 Session Establishment/Modification Response.

NOTE 2:	The application client can obtain the MoQ relay address via application layer-based schemes.

In the case the EASDF based DNS procedure as described in clause 6.2.3.2.2 of TS 23.548 [48] is used, after SMF has received FQDN in DNS message report from EASDF, SMF determines the FQDN is for MoQ traffic and gets the address of MoQ relay and then SMF provides DNS message handling rule with the address of MoQ relay to instruct EASDF to return the address of MoQ Relay in PSA UPF by setting the Forwarding Action as Respond directly to the DNS request according to TS 23.548 [130].

NOTE 3:	It is assumed an application client can send a DNS query to resolve FQDN.

NOTE 4:	The FQDN can be the FQDN for MoQ traffic and preconfigured in SMF or it can be the FQDN of the MoQ Relay and preconfigured in SMF and application client. It is assumed there is SLA between operator and application service provider for using MoQ relay functionality provided by operator.

NOTE 5:	In some other case, the application client can also obtain the MoQ relay address from the MoQ application server via the query during the MoQ session establishment between the application client and the MoQ application server as in MoQ, which is out of the scope of 3GPP. The MoQ application server can get the MoQ relay address via local configuration.

NOTE 6:	Different MoQ application servers can correspond to different MoQ relays that are deployed on the same PSA UPF.

The AF provided Protocol Description indicating Media over QUIC Transport can be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45]. Based on the PCC rule, the SMF instructs the PSA UPF to perform PDU Set Information identification. Upon receiving the MoQ SUBSCRIBE message, the MoQ relay can establish a connection to the MoQ application server if there does not exist an established connection yet.

For the downlink service data flow using MoQ Transport, the PSA UPF can identify the PDU Set Information from the MoQ Metadata as defined in clause 7 of MoQthat is extracted by the MoQ relay and provide the available PDU Set Information to the RAN in the GTP-U header.

NOTE 7:	The mappings between the MoQ Metadata and the PDU Set Information can be based on local configuration, or operator-determined implementations.

# * * * * End of changes * * * *