3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500225

# Electronic Meeting, 20th Jan – 24th Jan, 2025	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0272 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification for policy and same private UE IP addresses issues in HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In clause 6.7.2.2, The NOTE 2 on how V-EASDF differentiate DNS query when different UEs share the same private IP address should be moved to the EAS discovery procedure. 2. For HR PDU session, the H-SMF handles QFI allocation and configures the QER on PSA UPF. But in HR-SBO case, N4 rules on L-PSA UPF are configured by V-SMF directly which does not know the mapping relationship between QFIs and SDF filters. Further, in existing TS, how QER on L-PSA UPF is configured by V-SMF is not clear yet. A simple way to solve this issue is binding all offloaded traffic in VPLMN to a specific QoS Flow based on SLA with HPLMN. For example, default QoS Flow, and the V-SMF is aware the default QoS flow per TS 29.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Move NOTE 2 from clause 6.7.2.2 to clause 6.7.2.3 and some clarification is added to make the description clear. 2. Clarify that all offloaded traffic in VPLMN are bound to a specific QoS Flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description on the case of UE private IP address overlap and configuration of VPLMN L-PSA is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2, 6.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.2.2	PDU Session establishment for supporting HR-SBO in VPLMN

Figure 6.7.2.2-1: Procedure for PDU Session establishment supporting HR-SBO in VPLMN

1.	During the Registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the UE is roaming and if the AMF had received in SMF Selection Subscription data from UDM the HR-SBO allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

If the V-SMF supporting the HR-SBO receives the HR-SBO allowed indication from AMF, the V-SMF may:

-	select UL CL/BP UPF and L-PSA UPF based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL CL/BP UPF and L-PSA UPF can be co-located in the single V-UPF.

-	select a V-EASDF;

-	obtain the V-EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI, HPLMN ID and the UE IP address set to unspecified address or to a mapped address as specified in clause 7.1.2.2 to obtain the V-EASDF IP address;

-	may obtain V-EASDF DNS security information based on local configuration or via interaction with 7EASDF; and

NOTE 2:	The network needs to ensure that EASDF can disambiguate the DNS traffic of different UEs that would be allocated with same private UE IP address. This can be done by implementation and/or deployment specific means, e.g. tunnelling on N6, network instance or UE source IP address mapping. To disambiguate the DNS traffic of different UEs allocated with same private UE IP address, the V-SMF can update the Local PSA-UPF with N4 PDR and FAR including either N6 traffic routing information or network instance (as described in clause 5.6.12 of 23.501 [2]) to forward the DNS traffic to the V-EASDF. When N6 traffic routing tunnel is used, the V-SMF can configure V-EASDF with the N6 traffic routing information towards the Local PSA-UPF. Alternatively, the V-SMF can update the local UPF to translate the UE source IP address with the replaced IP address in the IP pool of the Local PSA UPF.

-	send the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and optionally send the IP address and (if exists) DNS security information of V-EASDF/Local DNS Server/Resolver to the H-SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

The H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the HR-SBO is authorized, the H-SMF requests and retrieves the optional VPLMN Specific Offloading Policy from H-PCF by SM Policy Association Establishment/Modification with the HR-SBO support indication as indicated in clause 5.2.5.4.2 of TS 23.502 [3].. The H-SMF generates VPLMN Specific Offloading Information (i.e. IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN in VPLMN, and/or Authorized DL Session AMBR for Offloading) based on the VPLMN Specific Offloading Policy. Each VPLMN Specific Offloading Policy may be provided with an Offload Identifier. The Offload Identifier is assigned by H-PCF, and is unique in the HPLMN.

NOTE 3:	To ensure that the Offload Identifier is unique within the HPLMN, PCF instances can be configured to assign Offload Identifier with different ranges.

If HR-SBO is authorized for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following information:

-	optionally, VPLMN Specific Offloading Information that may include FQDN range, IP range, session AMBR for the local part of DN and charging policy.

The VPLMN specific Offloading Information may refer to either allowed or not allowed traffic for HR-SBO (the latter is being used when the HPLMN would like to ensure that certain traffic are not allowed for HR-SBO). The VPLMN Specific Offloading Information for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both. The VPLMN Specific Offloading Policy can be configured in the H-SMF and tagged with Offload Identifier(s), and which VPLMN Specific Offloading Information to be sent to V-SMF can be indicated by these Offload Identifier(s) received from H-PCF.

H-SMF may send the Offload Identifier(s) alone or together with the VPLMN Specific Offloading Information:

-	If the given V-SMF has already received the VPLMN Specific Offloading Information corresponding to certain Offload Identifier(s), this could be indicated to the H-SMF in any subsequent request to another HR-PDU Session from the same V-SMF, and the H-SMF will in this case send only the Offload Identifier(s) as a response;

-	If the VPLMN Specific Offloading Information for a given Offload Identifier is changed, for each V-SMF using the Offload Identifier, the H-SMF chooses one existing HR-SBO PDU Session using the Offload Identifier to update VPLMN Specific Offloading Information and corresponding Offload Identifier to the V-SMF via PDU Session Modification procedure as described in clause 4.3.3.3 of TS 23.502 [3];

NOTE 4:	V-SMF keeps track for which HPLMN each Offload indicator is related to since the Offload Indicators are unique per HPLMN.

NOTE 5:	An Offload identifier can include a version number. In this case, a VPLMN Specific Offloading Information provided by H-SMF with a higher version number will overwrite the one with lower version number.

-	During PDU Session Release procedure as described in clause 4.3.4.3 of TS 23.502 [3], if the PDU Session is the last HR-SBO PDU Session using a given Offload Identifier on the V-SMF, the V-SMF may remove the Offload Identifier and corresponding VPLMN Specific Offloading Information based on roaming agreements.

NOTE 6:	In this Release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

-	the V-EASDF IP address (corresponding to clause 6.7.2.3) or Local DNS Server/Resolver IP address (corresponding to clause 6.7.2.4) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.5) as DNS server address to be sent to the UE via PCO; and

-	the DNS security information of V-EASDF/Local DNS Server/Resolver to be sent to the UE via PCO, if the UE indicate DNS server security information indicator in PDU Session Establishment Request and if supported in V-EASDF/Local DNS Server/Resolver (see TS 24.501 [11] and TS 33.501 [12]);

-	optionally, the DNS server address provided by HPLMN to be used for DNS requests related with traffic not to be subject to HR-SBO, including to configure V-EASDF corresponding to clause 6.7.2.3, or configure the UPF in VPLMN to perform IP replacement as described in clause 6.7.2.5;

NOTE 7:	In this Release, only public IP address can be used as the DNS server address provided by HPLMN.

-	optionally, the HPLMN address information (e.g. H-UPF IP address on N6) to be used by V-EASDF to build EDNS Client Subnet option for target FQDN of the DNS query which is not authorized for HR-SBO as described in clause 6.7.2.3;

-	the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not).

The H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If the request for HR-SBO is not authorized and DNS context has been created, the V-SMF delete the DNS context from the selected V-EASDF, and the subsequent steps related to the EASDF in this procedure are skipped.

If the request for HR-SBO is not authorized, the DNS server address provided by HPLMN is configured to UE as DNS server address. Step 3 to step 4 are skipped.

The detailed information of VPLMN Specific Offloading Policy is described in clause 6.4 of TS 23.503 [4].

NOTE 8:	The VPLMN Specific Offloading Policy can be prior configured in HPLMN based on the service level agreement between the VPLMN and HPLMN.

3.	The V-SMF configures the V-EASDF with the DNS handling rules using the VPLMN Specific Offloading Information received from H-SMF or corresponding to Offload Identifier(s) received from H-SMF.

The V-SMF optionally configures the V-EASDF with the DNS server address provided by the HPLMN as default DNS server (corresponding to clause 6.7.2.3), after the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] if they are received from H-SMF in the step 2. If V-SMF has not received the DNS server address provided by HPLMN from H-SMF in step 2, a default DNS server may be configured to V-EASDF.

If HPLMN address information is received, the V-SMF may also configure the V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS query which is not authorized for HR-SBO.

If the V-SMF has interacted with the V-EASDF in step 2, then the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF configures the UL CL UPF and PSA UPF selected in the step 2 to forward DNS messages to V-EASDF.

The V-SMF also configures the local PSA UPF to ensure the offloaded traffic is bound to a specific QoS Flow (e.g. default QoS Flow) based on SLA with HPLMN.

If the Authorized DL Session AMBR for Offloading is provided from the H-SMF, the V-SMF configures the Authorized DL Session AMBR for Offloading on the UL CL UPF in the VPLMN and additionally local PSA(s) terminating the N6 interface toward the local part of DN in the VPLMN.

At N4 session establishment for a PDU Session working in HR SBO mode, the SMF in VPLMN provides to any UPF in VPLMN acting as (local) PSA for that PDU Session and capable of enforcing NAT on N6 traffic: the HPLMN ID of UE, andthe DNN/S-NSSAI of the PDU Session in HPLMN for the PDU Session.

4A. 	EAS Discovery procedure with V-EASDF is performed as described in clause 6.7.2.3.

4B. 	EAS Discovery procedure with Local DNS Server/Resolver is performed as described in clause 6.7.2.4.

4C. 	EAS discovery procedure with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5.

# * * * * Second change * * * *

6.7.2.3	EAS Discovery Procedure with V-EASDF for HR-SBO

Figure 6.7.2.3-1: Procedure for EAS Discovery with V-EASDF for HR-SBO roaming scenario

1.	The DNS query sent by the UE reaches the UL CL UPF in VPLMN selected in step 2 of clause 6.7.2.2. The UL CL UPF forwards it to Local PSA UPF then to V-EASDF, or to H-UPF as described below.

NOTE X:	The network needs to ensure that EASDF can disambiguate the DNS traffic of different UEs that would be allocated with same private UE IP address. This can be done by implementation and/or deployment specific means, e.g. tunnelling of DNS traffic on N6, network instance for DNS traffic or UE source IP address mapping for DNS query. To disambiguate the DNS traffic of different UEs allocated with same private UE IP address, the V-SMF can update the Local PSA-UPF with N4 PDR and FAR including either N6 traffic routing information or network instance (as described in clause 5.6.12 of 23.501 [2]) to forward the DNS traffic to the V-EASDF. When N6 traffic routing tunnel is used, the V-SMF can configure V-EASDF with a N6 traffic routing information for the UE towards the Local PSA-UPF to forward DNS response to the Local PSA UPF, the N6 traffic routing information will be used by Local PSA UPF to determines for which UE the DNS response corresponds to. Alternatively, the V-SMF can update the local PSA UPF to translate the UE source IP address of the DNS query with an IP address for the UE from the IP pool of the Local PSA UPF and the local PSA UPF forwards DNS query with new source IP address to V-EASDF; for DNS reponse for the DNS query received from V-EASDF, local PSA UPF replaces the destination IP address of the DNS response to UE’s IP address and forward the DNS reponse with UE’s IP address to UE.

If the target FQDN of the DNS query is not part of the FQDN authorized by the H-SMF in step 2 of clause 6.7.2.2, the following a) or b) may be performed:

a)	Based on SMF instruction in step 3 of clause 6.7.2.2, the V-EASDF proceeds to step 12 of clause 6.2.3.2.2 where it sends the DNS query which may include the HPLMN address information as the EDNS Client Subnet option. The DNS query is sent to the DNS server address according to the DNS message handling rules provided by the V-SMF or to the default DNS server configured in the V-EASDF. Upon receiving the DNS response, the procedure proceeds immediately to step 5.

NOTE 1:	If HPLMN DNS or the default DNS server does not support the EDNS Client Subnet option, it cannot be ensured that an AS close to H-UPF will be resolved.

b)	The UL CL/BP UPF sends the DNS query to the DNS server address provided by HPLMN via V-UPF (if exists) and H-UPF (through N9), by modifying the packet's destination IP address (corresponding to V-EASDF) to the DNS server address provided by HPLMN on UL CL or H-UPF. For the corresponding DNS response received by H-UPF, the H-UPF or UL CL modifies the packets' source IP address to that of the V-EASDF.

This assumes that the UL CL is able to detect FQDN(s) in traffic sent to the IP address of the EASDF. It is thus incompatible with usage of DoT (DNS over TLS) or DoH to protect the DNS traffic exchanged between the UE and the PLMN.

If the VPLMN Specific Offloading Information only includes IP range, the V-SMF can configure the V-EASDF to resolve all DNS queries using a VPLMN address (e.g. an IP address associated with the L-PSA UPF in the VPLMN) as EDNS Client Subnet option.

The rest of the procedure assumes the target FQDN of the DNS query is part of the FQDN authorized by the H-SMF in step 2 of clause 6.7.2.2.

2.	The steps 8 to 15 of the procedure in the clause 6.2.3.2.2 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

If VPLMN Specific Offloading Information does not include FQDN range and the EAS IP address of the DNS response is not part of the IP range(s) authorized in step 2 of clause 6.7.2.2, the following may be performed:

-	If the V-EASDF is not configured with the DNS server address provided by the HPLMN as default DNS server, The V-SMF indicates the V-EASDF to construct and to send another DNS query with the same FQDN and the HPLMN address information as the EDNS Client Subnet option, to the DNS server address as described in step 1) bullet a). Otherwise V-SMF indicates the V-EASDF to construct and send another DNS query with the same FQDN to the DNS server provided by HPLMN.

3.	The V-SMF selects UL CL/BP and local PSA in VPLMN based on the V-EASDF notification, EAS Deployment Information in the VPLMN, VPLMN Specific Offloading Information and UE location. The V-SMF may perform insertion or change of UL CL/BP and local PSA in VPLMN.

The V-SMF configures the UL CL/BP and local PSA for the traffic to be offloaded to the local part of DN based on the VPLMN Specific Offloading Information received from H-SMF.

If the Authorized DL Session AMBR for Offloading is provided from the H-SMF, the V-SMF configures the Authorized DL Session AMBR for Offloading on the UL CL/BP in the VPLMN and additionally the local PSA(s) terminating the N6 interface toward the local part of DN in the VPLMN.

In the case of UL CL, the V-SMF configures the traffic detection rules and traffic routing rules on the UL CL UPF based on the EAS Deployment Information and the EAS addresses included in VPLMN Specific Offloading Information.

If there is no other V-UPF between the selected UL CL/BP in this step and H-UPF, the V-SMF sets up user plane between this UL CL/BP and H-UPF via the interaction with H-SMF. Otherwise, the V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

The V-SMF sets up user plane between the selected UL CL/BP in this step and RAN (if no other V-UPF exists between RAN and this UL CL/BP) or the V-UPF (if exists between this UL CL/BP and RAN).

NOTE 2:	If the selected UL CL/BP and local PSA in this step is the UL CL/BP and PSA selected by V-SMF in the step 2 of clause 6.7.2.2, the insertion of UL CL/BP and local PSA in VPLMN will not be performed in this step.

NOTE 3:	In the home routed roaming scenario, the V-UPF selected during PDU Session establishment procedure can be deployed at a central area within VPLMN. In this case, the V-UPF is located in the user plane path between UL CL/BP UPF in VPLMN and PSA-UPF in HPLMN. In some deployments, the UL CL/BP UPF can be collocated with the V-UPF.

If the EAS IP address is not part of the IP address authorized by H-SMF, the UL CL/BP V-UPF is instructed by V-SMF to send the packet from UE to the H-UPF.

4.	The steps 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

5.	V-EASDF sends the DNS Response to the UE.

# * * * * End of changes * * * *