3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500256

# Electronic meeting, 2025-01-20 – 2025-01-24

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0274 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting and improving text for KI#1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correcting and improving the text regarding KI#1 for WI EDGE_Ph3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting and improving the text. Editorial corrections of HR-SBO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not fully correct text and less readable, which may lead to erroreous implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.2.1, 6.10.2.2, 6.10.2.3, 6.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

## 6.10	Support of the Local Offloading Management

### 6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The I-SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF, and does not do DNS message handling for the locally offloaded traffic towards the local part of DN.

NOTE 3: The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

### 6.10.2	Procedure

#### 6.10.2.1	General

This clause describes the procedures related to Local Offloading Management.

#### 6.10.2.2	I-SMF insertion during the PDU Session establishment

Figure 6.10.2.2-1: Procedure for PDU Session establishment supporting Local Offloading Management

1.	During the Registration procedure, the AMF receives the Local Offloading Management allowed indication per DNN/S-NSSAI as part of SMF selection subscription data from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

If Local Offloading Management is allowed for the requested DNN/S-NSSAI in the step 1 of clause 4.3.2.2.1 of TS 23.502 [3], the AMF discovers SMF(s) that supports Local Offloading Management. The AMF may use the current location of the UE as query parameter in the discovery. For the discovered SMF(s), the AMF executes as follows:

-	If the UE is not within the local offloading management service area of any of the candidate SMF(s), then an I-SMF for local offloading management is not needed and step 2a is executed.

-	If the UE is within the local offloading management service area of at least one of the candidate SMF(s), then AMF selects an I-SMF for local offloading management supporting the local offloading management service area  and step 2b is executed.

NOTE x:	I-SMF could be inserted due to other reasons than Local Offloading Management.

2a.	Same as the step 1-6 defined in clause 6.2.3.2.2, and stteps 3 and 4 are skipped.

2b.	The AMF selects an I-SMF that serves the UE location and supports Local Offloading Management. The PDU Session Establishment procedure as described in clause 6.7.2.2 is used with the following differences:

-	This procedure applies for non-roaming PDU session and all NFs are located in HPLMN.

-	The V-SMF and V-UPF are replaced by I-SMF and I-UPF, respectively.

-	The H-SMF and H-UPF are replaced by SMF and UPF(PSA), respectively.

-	The AMF sends a Local Offloading Management allowed indication to the I-SMF in the step 3a of clause 4.3.2.2.2 of TS 23.502 [3] with V-SMF replaced by I-SMF.

If the I-SMF receives the Local Offloading Management allowed indication from AMF, the I-SMF:

-	Selects UL CL/BP UPF and L-PSA UPF based on UE location information.

-	Selects an EASDF.

-	Obtains the EASDF IP address based on local configuration.

-	May obtain EASDF DNS security information based on local configuration or via interaction with EASDF.

-	Sends  Nsmf_PDUSession_Create Request (Local Offloading Management support indication, IP address and (if exists) DNS security information of EASDF and I-SMF supported DNAI(s)). See step 6 of the procedure in figure 4.3.2.2.2-1 of TS 23.502 [3] with V-SMF, H-SMF replaced by I-SMF, SMF respectively.

When the SMF receives the Local Offloading Management support indication and allows I-SMF to perform Local Offloading Management based on SM subscription data, the SMF:

-	Skips the EASDF Discovery and Selection procedure and DNS message handling.

-	Provides Local Offloading Management support indicator to PCF, whereby PCF provides the Local Offloading Management Policy, i.e., FQDN/IP range(es) allowed to be routed to the local part of DN, or Offload Identifier, to SMF,  as described in clause 6.4 of TS 23.503 [4] Each Local Offloading Management Policy may also be provided with an Offload Identifier which is assigned by PCF.

-	Sends the received Local Offloading Management Policy and/or Offload Identifier to the I-SMF.

-	Sends IP address and optionally DNS security information of the EASDF received from the I-SMF to the UE via PCO.

-	Sends Local Offloading Management Allowed indication to the I-SMF.

NOTE:	Local Offloading Management Allowed indication can be part of SMF selection subscription data and Session Management Subscription data. It is up to UDM implementation to make sure the data is consistent.

3.	The I-SMF configures the EASDF with the DNS handling rules using the Local Offloading Control Information received from SMF.

If the I-SMF has the the EASDF IP address  locally configured, the I-SMF invokes Neasdf_DNSContext_Create Request to configure the EASDF.

The I-SMF configures the UL CL UPF to forward IP packets as per the Local Offloading Management Policy or Offload Identifier (which should include forwarding of IP packets to the EASDF)to the local PSA UPF.

4.	EAS Discovery procedure with EASDF is performed as described in clause 6.10.2.3.

#### 6.10.2.3	EAS Discovery Procedure with EASDF for Local Offloading Management

The EAS Discovery Procedure with EASDF for Local Offloading Management follows the same procedure as defined in clause 6.7.2.3, with the following differences:

-	General:

-	VPLMN is replaced by HPLMN;

-	V-EASDF is replaced by EASDF;

H-UPF is replaced by UPF;

-	H-SMF is replaced by SMF;

-	V-SMF is replaced by I-SMF;

-	V-UPF is replaced by I-UPF;

-	VPLMN Specific Offloading Information is replaced by Local Offloading Control Information.

-	Step 1:

-	Clause 6.7.2.2 is replaced by clause 6.10.2.2.

-	For option a), the SMF configures EASDF with the DNS server in the central part of the DN for FQDNs that are unauthorized in the local part of the DN and EASDF may provide an EDNS Client options related to the PSA UPF in messages sent to this DNS server.

-	Step 3:

-	NOTE 2 and NOTE 3 are not applicable.

************** 2nd change **************

#### 6.7.2.2	PDU Session establishment for supporting HR-SBO in VPLMN

Figure 6.7.2.2-1: Procedure for PDU Session establishment supporting HR-SBO in VPLMN

1.	During the Registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSSAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the UE is roaming and if the AMF had received in SMF Selection Subscription data from UDM the HR-SBO allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

If the V-SMF supporting the HR-SBO receives the HR-SBO allowed indication from AMF, the V-SMF may:

-	select UL CL/BP UPF and L-PSA UPF based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL CL/BP UPF and L-PSA UPF can be co-located in the single V-UPF.

-	select a V-EASDF;

-	obtain the V-EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI, HPLMN ID and the UE IP address set to unspecified address or to a mapped address as specified in clause 7.1.2.2 to obtain the V-EASDF IP address;

-	may obtain V-EASDF DNS security information based on local configuration or via interaction with EASDF; and

NOTE 2:	The network needs to ensure that EASDF can disambiguate the DNS traffic of different UEs that would be allocated with same private UE IP address. This can be done by implementation and/or deployment specific means, e.g. tunnelling on N6, network instance or UE source IP address mapping. To disambiguate the DNS traffic of different UEs allocated with same private UE IP address, the V-SMF can update the Local PSA-UPF with N4 PDR and FAR including either N6 traffic routing information or network instance (as described in clause 5.6.12 of 23.501 [2]) to forward the DNS traffic to the V-EASDF. When N6 traffic routing tunnel is used, the V-SMF can configure V-EASDF with the N6 traffic routing information towards the Local PSA-UPF. Alternatively, the V-SMF can update the local UPF to translate the UE source IP address with the replaced IP address in the IP pool of the Local PSA UPF.

-	send the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and optionally send the IP address and (if exists) DNS security information of V-EASDF/Local DNS Server/Resolver to the H-SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

The H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the HR-SBO is authorized, the H-SMF requests and retrieves the optional VPLMN Specific Offloading Policy from H-PCF by SM Policy Association Establishment/Modification with the HR-SBO support indication as indicated in clause 5.2.5.4.2 of TS 23.502 [3].. The H-SMF generates VPLMN Specific Offloading Information (i.e. IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN in VPLMN, and/or Authorized DL Session AMBR for Offloading) based on the VPLMN Specific Offloading Policy. Each VPLMN Specific Offloading Policy may be provided with an Offload Identifier. The Offload Identifier is assigned by H-PCF, and is unique in the HPLMN.

NOTE 3:	To ensure that the Offload Identifier is unique within the HPLMN, PCF instances can be configured to assign Offload Identifier with different ranges.

If HR-SBO is authorized for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following information:

-	optionally, VPLMN Specific Offloading Information that may include FQDN range, IP range, session AMBR for the local part of DN and charging policy.

The VPLMN specific Offloading Information may refer to either allowed or not allowed traffic for HR-SBO (the latter is being used when the HPLMN would like to ensure that certain traffic are not allowed for HR-SBO). The VPLMN Specific Offloading Information for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both. The VPLMN Specific Offloading Policy can be configured in the H-SMF and tagged with Offload Identifier(s), and which VPLMN Specific Offloading Information to be sent to V-SMF can be indicated by these Offload Identifier(s) received from H-PCF.

H-SMF may send the Offload Identifier(s) alone or together with the VPLMN Specific Offloading Information:

-	If the given V-SMF has already received the VPLMN Specific Offloading Information corresponding to certain Offload Identifier(s), this could be indicated to the H-SMF in any subsequent request to another HR-PDU Session from the same V-SMF, and the H-SMF will in this case send only the Offload Identifier(s) as a response;

-	If the VPLMN Specific Offloading Information for a given Offload Identifier is changed, for each V-SMF using the Offload Identifier, the H-SMF chooses one existing HR-SBO PDU Session using the Offload Identifier to update VPLMN Specific Offloading Information and corresponding Offload Identifier to the V-SMF via PDU Session Modification procedure as described in clause 4.3.3.3 of TS 23.502 [3];

NOTE 4:	V-SMF keeps track for which HPLMN each Offload indicator is related to since the Offload Indicators are unique per HPLMN.

NOTE 5:	An Offload identifier can include a version number. In this case, a VPLMN Specific Offloading Information provided by H-SMF with a higher version number will overwrite the one with lower version number.

-	During PDU Session Release procedure as described in clause 4.3.4.3 of TS 23.502 [3], if the PDU Session is the last HR-SBO PDU Session using a given Offload Identifier on the V-SMF, the V-SMF may remove the Offload Identifier and corresponding VPLMN Specific Offloading Information based on roaming agreements.

NOTE 6:	In this Release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

-	the V-EASDF IP address (corresponding to clause 6.7.2.3) or Local DNS Server/Resolver IP address (corresponding to clause 6.7.2.4) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.5) as DNS server address to be sent to the UE via PCO; and

-	the DNS security information of V-EASDF/Local DNS Server/Resolver to be sent to the UE via PCO, if the UE indicate DNS server security information indicator in PDU Session Establishment Request and if supported in V-EASDF/Local DNS Server/Resolver (see TS 24.501 [11] and TS 33.501 [12]);

-	optionally, the DNS server address provided by HPLMN to be used for DNS requests related with traffic not to be subject to HR-SBO, including to configure V-EASDF corresponding to clause 6.7.2.3, or configure the UPF in VPLMN to perform IP replacement as described in clause 6.7.2.5;

NOTE 7:	In this Release, only public IP address can be used as the DNS server address provided by HPLMN.

-	optionally, the HPLMN address information (e.g. H-UPF IP address on N6) to be used by V-EASDF to build EDNS Client Subnet option for target FQDN of the DNS query which is not authorized for HR-SBO as described in clause 6.7.2.3;

-	the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not).

The H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If the request for HR-SBO is not authorized and DNS context has been created, the V-SMF delete the DNS context from the selected V-EASDF, and the subsequent steps related to the EASDF in this procedure are skipped.

If the request for HR-SBO is not authorized, the DNS server address provided by HPLMN is configured to UE as DNS server address. Step 3 and step 4 are skipped.

The detailed information of VPLMN Specific Offloading Policy is described in clause 6.4 of TS 23.503 [4].

NOTE 8:	The VPLMN Specific Offloading Policy can be prior configured in HPLMN based on the service level agreement between the VPLMN and HPLMN.

3.	The V-SMF configures the V-EASDF with the DNS handling rules using the VPLMN Specific Offloading Information received from H-SMF or corresponding to Offload Identifier(s) received from H-SMF.

The V-SMF optionally configures the V-EASDF with the DNS server address provided by the HPLMN as default DNS server (corresponding to clause 6.7.2.3), after the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] if they are received from H-SMF in the step 2. If V-SMF has not received the DNS server address provided by HPLMN from H-SMF in step 2, a default DNS server may be configured to V-EASDF.

If HPLMN address information is received, the V-SMF may also configure the V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS query which is not authorized for HR-SBO.

If the V-SMF has interacted with the V-EASDF in step 2, then the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF configures the UL CL UPF and PSA UPF selected in the step 2 to forward DNS messages to V-EASDF.

If the Authorized DL Session AMBR for Offloading is provided from the H-SMF, the V-SMF configures the Authorized DL Session AMBR for Offloading on the UL CL UPF in the VPLMN and additionally local PSA(s) terminating the N6 interface toward the local part of DN in the VPLMN.

At N4 session establishment for a PDU Session working in HR SBO mode, the SMF in VPLMN provides to any UPF in VPLMN acting as (local) PSA for that PDU Session and capable of enforcing NAT on N6 traffic: the HPLMN ID of UE, and the DNN/S-NSSAI of the PDU Session in HPLMN for the PDU Session.

4A. 	EAS Discovery procedure with V-EASDF is performed as described in clause 6.7.2.3.

4B. 	EAS Discovery procedure with Local DNS Server/Resolver is performed as described in clause 6.7.2.4.

4C. 	EAS discovery procedure with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5.

************ End of changes ************