3GPP TSG-SA WG2#166E	S2-2500289

EMeeting, 20-24 January 2025

Source: 	vivo

Title: 	Multiplexing with duplicated stream ID

Document for: 	Discussion

Agenda Item: 	19.2.2

Work Item / Release:	NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution discusses the case of duplicated stream ID in multiplexing case.

1	Discussion

There are 3 scenarios for multiplexing are introduced:

- multiplexing and de-multiplexing of different types of bootstrap data channels

- multiplexing and de-multiplexing of application data channels when both originating and terminating networks support data channel multiplexing

- multiplexing and de-multiplexing of application data channels when only originating networks support data channel multiplexing

For the multiplexing and de-multiplexing of application data channels when both originating and terminating networks support data channel multiplexing scenario, the stream id duplicated problem is missing, this contribute try to complete the solution.

For example, UE#1 has 2 application DCs targeting to DC AS#1, and UE#2 originally, e.g. with stream id 1001 and 1000. The streams of app#1 and app#2 are multiplexed between UE#1 and originating MF/MRF in one SCTP connection

Figure 1

UE#2 initiates a new P2P application DC to UE-2 with stream ID 1001 and request to multiplex with application DC for app#2.

When the DCSF of UE-1 receives the multiplexing request from UE-2, the DCSF determines that the stream id of app#3 is the same with the stream id of app#3, the 2 application DCs cannot be multiplexed between UE#1 and originating MF/MRF. So, the DCSF of UE-1 de-multiplexing app#3’s application DC.

2     Proposal

In case both originating and terminating networks support data channel multiplexing scenario, DCSF shall be able to de-multiplexing application data channels with same stream ID.