3GPP TSG-SA WG2 Meeting #166E	S2-2500293

# Emeeting, 20 - 24 January, 2025												 (revision of S2-2412907)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1520 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication with early media in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handling of early media for UE-satellite-UE communication has been discussed. S2-2411950 discuss the issue for early media handling, and proposes to sustain using ground-AGW for early media based on operator policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedure for supporting UE-Satellite-UE communication with early media in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early media may be terminated or interrupted when UE-satellite-UE communication is activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (all new text) ---

AE.5.x	At call setup with early media

During the call setup with both originating and terminating UEs using regenerative NR satellite access, the originating P-CSCF uses the IMS-AGW on ground to transfer early media then, after the IMS session is answered, releases the IMS-AGW on ground and selects the IMS-AGW on satellite to transfer regular media. See Figure AE.5.x-1 that describes the procedure.

In this procedure, the IMS AS for early media uses existing method for providing early media and updating the IMS session when answered. The example procedure assumes that the IMS AS for early media is configured to update media with both originating side and terminating side when the IMS session is answered, which enables the update of the IMS AGW on satellite in terminating network after IMS session answered.

Figure AE.5.x-1: IMS session establishment procedure with early media

1-10.	Steps 1-10 of clause AE.5.1 apply.

11.	Step 11 of clause AE.5.1 applies with the difference that the IMS AS serving early media allocates early media resources with MRF, as well as includes the P-Early-Media header field and a modified SDP answer for early media in the 18X response.

12.	Based on the identification of the satellite serving UE B and the P-Early-Media header field in step 11, P-CSCF A determines to perform UE-satellite-UE communication with early media. P-CSCF A selects an IMS-AGW A on satellite but does not release the IMS-AGW A' on ground in order to use it for early media.

12a-12b.	P-CSCF A interacts with IMS-AGW A' on ground for early media to allocate and configure transport resources for IMS access side as well as configure transport resources for IMS core network side.

13. Step 13 of clause AE.5.1 applies.

The early media is able to be sent from the MRF to the UE A via the IMS AGW A' on ground, the UPF A' on ground, and the UPF A on satellite.

14.	Step 14 of clause AE.5.1 applies with the difference that P-CSCF A includes in the SIP 18X response an SDP answer containing the transport address of IMS-AGW A' on ground obtained in step 12.

15-16.	Steps 15-16 of clause AE.5.1 apply with difference that P-CSCF A does not generate/modify SDP offer in step 16. Step 17 of clause AE.5.1 is not performed.

17-18.	Steps 18-19 of clause AE.5.1 apply and UE B is ringing.

19.	The UE B answers the IMS session and sends SIP 200 response for the INVITE.

20.	The IMS AS serving early media instructs the MRF to stop playing early media. UE A may play local ring tone when early media is stopped.

21.	The IMS AS serving early media sends SIP ACK and SIP reINVITE without SDP information towards UE B. UE B returns SIP 200 response with an SDP offer. P-CSCF B forwards the SIP 200 response with modified SDP offer that contains the transport address of IMS-AGW B on satellite obtained in step 9.

NOTE 1:	In order to reduce the latency, P-CSCF B can stop interaction with UE B in this step, i.e., does not forward SIP ACK and SIP reINVITE to UE B and returns SIP 200 response on behave of UE B.

22.	The IMS AS serving early media sends SIP UPDATE request with the SDP offer for regular media towards originating side.

23.	P-CSCF A releases the IMS AGW A' on ground and interacts with the IMS-AGW A on satellite selected in step 12 for regular media to allocate and configure transport resources for both IMS access side and IMS core network side.

24.	P-CSCF A forwards the SIP UPDATE request with modified SDP offer to the UE A, which contains the transport address of IMS-AGW A on satellite obtained in step 23. The UE A returns SIP 200 response for the UPDATE with an SDP answer.

25.	P-CSCF A sends SIP 200 response with modified SDP answer towards IMS core network side, which contains the transport address of IMS-AGW A on satellite obtained in step 23.

NOTE 2:	In order to reduce the latency, P-CSCF A can perform step 25 right after step 23.

26.	The IMS AS serving early media sends SIP 200 response for the INVITE to the originating side, P-CSCF A forwards the SIP 200 response to UE A.

After receiving the SIP 200 response, the UE A is able to send regular media and is ready to receive regular media. The regular media uses UE-satellite-UE communication path.

27.	UE A sends SIP ACK towards terminating side. The IMS AS serving early media forwards the SIP ACK with the SDP answer received in step 25 towards terminating side.

NOTE 3:	In order to reduce the latency, P-CSCF A can send SIP ACK on behave of UE A towards terminating side (27b) and send SIP 200 response for the INVITE to UE A (26b) simultaneously.

28.	P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the received SDP answer for regular media to configure transport resources for IMS core network side.

NOTE 4:	In order to reduce the latency, P-CSCF B can update the allocated IMS-AGW B on satellite for UE B (step 28) and forward the SIP ACK to UE B (step 29) simultaneously.

29.	P-CSCF B forwards the SIP ACK with modified SDP answer containing the transport address of IMS-AGW B on satellite obtained in step 6.

After receiving the SIP ACK, the UE B is able to send regular media.

## ---End of the Change---