3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500320

Electronic Meeting, 20-24 January 2025

# Title:	[Draft] Reply LS on Signalling feasibility of data set and parameter sharing

# Response to:	LS on signalling feasibility of dataset and parameter sharing (R1-2410922/S2-2500041)

# Release:	Release 19

# Work Item:	FS_NR_AIML_air

Source:	SA2

To:	RAN1, SA5

Cc:	RAN2, SA3

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks RAN1 for their LS regarding the Rel-19 two-sided AI/ML CSI compression study for AI/ML Air Interface in S2-2500041. SA2 discussed the following RAN1 required procedures and agreed that SA2, SA3 and SA5 also need to be involved to define a standardized end-to-end solution to support the inter-vendor training collaboration between NG-RAN side and UE side.

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

RAN1 concluded that the standardized signaling may be - over-the-air, or - other approaches

SA2 discussed the over-the-air solution proposed by RAN1, which included the procedure to share the model parameters and /or data set from NW- side to UE and from UE to UE-side.

There is no existing solution and no work planned in SA2 to discuss the solution of model parameters and /or data set sharing from NW-side to UE and from UE to UE-side. SA2 also has a concern about the UE impact in being used as a “relay” to transfer data from NW-side to UE-side since it is not clear which UE will be selected to perform this operation considering there may be implications to charging and to the time efficiency for the over-the-air solution.

SA2 also discussed the standard signalling solution of other approaches without UE’s involvement. For example: model parameters and /or data set are shared from NW- side to OAM and then exposed from OAM through some APIs to UE-side. SA2 would suggest SA5 to provide the feedback on the feasibility of standardized signalling for the other approaches without UE’s involvement.

2. Actions:

To RAN1:

ACTION: 	SA2 suggests RAN1 to take the above information into account.

To SA5:

ACTION: 	SA2 suggests SA5 to take a look and comment on the feasibility of a solution where model parameters and /or data set can be shared from NW- side to OAM and then exposed from OAM through some APIs to UE-side.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		7-11 April 2025	Gotheburg, Sweden