SA WG2 Meeting #166-AdHoc-e	S2-2500328

# E-meeting, 20 – 24 January 2025                               (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Resolution of Editor’s notes with other WGs dependencies

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to discuss the way-forward for the Editor’s notes having other WGs dependencies.

1	Discussion

In the TR 23.700-13 clause 8 (Conclusions), some Editor’s notes with other WGs dependencies (e.g. RAN3, SA3) need to be resolved to complete the study item. This paper proposes to discuss the way-forward proposals for these Editor’s notes.

1) Editor’s note on BS Reader selection

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

The above Editor’s note is about BS Reader selection and has RAN3 dependency. The BS Reader selection has been discussed also in RAN3 and the agreements are documented in TR 38.769 as followings:

In Topology 1, an A-IoT RAN node may serve one or more readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

NOTE 4:	How to perform reader selection needs further discussions.

NOTE 5: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

Both SA2 and RAN3 acknowledged that BS Reader/A-IoT RAN selection is required and coordination between the two working groups is needed. As RAN3 has decided to resolve this issue during normative phase, it is reasonable for SA2 to work on this together with RAN3 during the normative phase since no resolution can be expected from RAN3 in 2025 Q1. It is proposed to remove this Editor’s note and to add a NOTE mentioning BS Reader/A-IoT RAN selection will be resolved during the normative phase.

Proposal 1a: It is proposed to remove this Editor’s note and to add a NOTE mentioning BS Reader/A-IoT RAN selection will be resolved during the normative phase.

From SA2 point of view, both BS Reader selection and A-IoT RAN selection can be supported as proposed in the solutions of TR 23.700-13. The key point is whether BS Reader will be defined by RAN3 and whether BS Reader information can be exposed to CN, so the leading group for resolution of this issue should be RAN3 and SA2 should follow RAN3 decision to design the corresponding solution. It is proposed to send an LS to RAN3 to clarify SA2 assumption that RAN3 should determine whether BS Reader will be defined and whether BS Reader information can be exposed to CN.

Proposal 1b: It is proposed to send an LS to RAN3 to clarify SA2 assumption that RAN3 should determine whether BS Reader will be defined and whether BS Reader information can be exposed to CN.

2) Editor’s note on assistance information

Editor's note:	Other assistance information may be added later if necessary.

Looking through the TR 38.769, no further assistance information is identified by RAN2 besides the already concluded ones in TR 23.700-13, so this Editor’s note can be removed.

Proposal 2: It is proposed to remove the Editor’s note on assistance information.

3) Editor’s note on AIoT service type

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

As far as RAN2 has concluded in TR 38.769, whether more information on command type is required is left to normative phase. It is proposed to remove the above Editor’s note and to add an NOTE saying more information on command type will be added if required by RAN2.

-	The A-IoT service type (e.g., inventory, command)

NOTE 3:	It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available.

Proposal 3: It is proposed to remove the above Editor’s note and to add an NOTE saying more information on command type will be added if required by RAN2.

4) Editor’s note on Device ID privacy protection and authentication

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

As the Editor’s note self-explained, the Device ID privacy protection and authentication will be concluded by SA3. Furthermore, for the Device ID privacy protection, there are LSs exchange (S2-2411049 and S3-245139) between SA2 and SA3 and SA3 will keep SA2 informed once a conclusion has been reached. For the Device ID authentication, several solutions were documented in TR 33.713 and conclusions will be made by SA3. There is no further work expected for SA2 in the study phase, so it is proposed to remove this Editor’s note.

Proposal 4: It is proposed to remove this Editor’s note as no further work is expected for SA2 in the study phase.

5) Editor’s note on AIoT Device NAS layer for Inventory Request

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

From SA2 point of view, it seems no need to use AIoT Device NAS layer to carry on Inventory Request as Inventory Request message only contains Device information for BS Reader to paging the AIoT Device as the above first Editor’s note implies. It is uncertain whether there is any security reason to protect the Inventory Request using AIoT Device NAS layer as the above second Editor’s note explained. It is proposed to remove this Editor’s note and send an LS to SA3 to ask whether there is any security reason to protect the Inventory Request using AIoT Device NAS layer.

Proposal 5: It is proposed to remove this Editor’s note and send an LS to SA3 to ask whether there is any security reason to protect the Inventory Request using AIoT Device NAS layer.

6) Editor’s note on command-only operation

Editor’s note:	If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

RAN2 has agreed to support the command-only operation via two options as described in TR 38.769.

-	For the detailed use case of "command-only":

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [151] and [152]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID and/or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

For the second option, the feasibility depends on SA2 and SA3. There is no restriction for SA2 to support the second option and it is totally pending on SA3 conclusion. It is proposed to remove this Editor’s note and send an LS to SA3 to ask the feasibility of supporting command-only operation.

Proposal 6: It is proposed to remove this Editor’s note and send an LS to SA3 to ask the feasibility of supporting command-only operation.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

NOTE x:	Whether and how AIOTF selects BS Readers or AIoT RAN nodes in Topology 1 will be determined in normative phase in coordination with RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE y:	Further information on command type may be added in normative phase if requested by RAN2.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# * * * * Second change * * * *

8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines A-IoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the A-IoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, a TASK ID etc. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

# * * * * End of Changes * * * *