3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500339

20th – 24th January, 2025, Online

Source:	China Mobile

Title:	Removing one EN related to How addressing works for UP traffic via AMF in KI#1

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes methods for addressing works for UP traffic via AMF.

# Discussion

Serving AIOT NF can be decided by service location area info provided by AF in the downlink direction. Once serving AIOT NF is chosen, it can deliver its identifier info to AIOT RAN or via AMF if AMF is used in the process. Simultaneously, AIOTF sends an operation request with transaction ID to AIOT RAN. Thus, AIOT RAN can determine to respond to the requests to the correct serving AIOT NF with the same transaction ID for uplink traffic. Then, the serving AIOT NF can correctly handle the response traffic based on transaction ID, such as aggregation.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request;

-  Serving AIOTF Identifier.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request with transaction ID to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request based on transaction ID. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# * * * * Second change * * * *

#### 8.1.2.3	AIOT RAN and the AIOTF communicate indirectly via an AMF

When AIOT RAN and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with AIOT RAN via an AMF. NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 1:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with AIOT RAN.

-	NGAP between AIOT RAN and the AMF is enhanced to support Ambient IoT Services.

NOTE 2:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support:

-	On the Nz reference point, the AMF supports Services which are used by an AIoTF for Ambient IoT Operations e.g. to send AIoT requests towards AIOT RAN and to receive AIoT responses from the AIOT RAN.

-	On the N2 reference point, the AMF supports sending AIOT information to the AIOT RAN (e.g. operation requests) and receiving responses from the AIOT RAN.

-	The AMF routes the AIoT messages between the AIOT RAN (over N2 reference point) and the AIOTF (over Nz reference point).

NOTE 3:	Whether to enhance an existing service or define a new service will be determined in the normative phase.

-	Figure 8.1.2.3-1 below shows the aspects related to Topology 1 (indirect path via AMF) architecture in reference point representation with other NFs removed.

Figure 8.1.2.3-1: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2.3-2 below shows the aspects related to Topology 1 (indirect  Path via AMF) architecture in reference point representation with other NFs removed.

Figure 8.1.2.3-2: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2.3-3 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between AIOT RAN and AIoTF.

-	AIoT Data represents information exchanged between the AIoT Device and AF (application specific content) and AIoT Reader Control represents the requests and responses between the AIOTF and AIOT RAN.

Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

NOTE 4:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be determined by RAN WG3.

# * * * * End of changes * * * *