3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500340

20th – 24th January, 2025, Online

Source:	China Mobile

Title:	Removing two ENs related to Ambient device-related context storage in KI#1

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes storing and using ambient IOT device-related context methods.

# Discussion

There are mainly two categories of ambient device information.  One is subscription-like static ambient device information, and the other is dynamic ambient device information in the service operation.

The main contents in the subscription-alike static ambient device:

- Ambient Permanent Device ID

- Authentication-related data, such as pre-shared key.

- Allowed serving location list

- Device status

- Owner information, such as AF ID

……

The main contents in the dynamic information related to ambient devices:

- Last serving BS reader ID or Last serving location ID per permanent device ID

……

Where to store these device-related information：

- It is suggested that the storage is in UDR for subscription-like static ambient devices. UDM may not be involved in the architecture since fewer UDM features are used in the solution.

- Dynamic device info is stored in AIOTF.

- AIOTF can obtain static information from UDR if needed.

How to use this device-related information:

- Static ambient IOT device information is mainly used for authentication and authorization.

- Dynamic device information, such as the last-serving BS reader ID or the last-serving location ID, is used for the BS reader selection optimization procedure and calculation of the estimated ambient device number per BS reader or serving location to assist AIOT RAN in setting the initial Q value.

Proposal: The network may manage AIOT device subscription-like static information (e.g., device ID, credentials, allowing serving location) in UDR. This information can be used for authentication and authorization.

The network may manage AIOT device dynamic information (e.g., Last serving BS reader ID or Last serving location ID ) in AIOTF. This information can be used for BS reader selection and assist AIOT RAN in setting the initial Q value.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.  	The network may manage AIOT device subscription-like static information (e.g., device ID, credentials, allowing serving location) in UDR. This information can be used for authentication and authorization.

The network may manage AIOT device dynamic information (e.g., Last serving BS reader ID or Last serving location ID ) in AIOTF. This information can be used for BS reader selection and assist AIOT RAN in setting the initial Q value.

# * * * * End of changes * * * *