3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500344

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5927 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor’s note on whether the Energy Saving indicator is one value or multiple values. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR resolves this editora's note Editor's note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the resoning in DP in S2-2500227 the CR proceeds to consider the indicator to be a scalar with 16 values, whose value is set by the operator and associated to a configurable behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specificaion editor's note remain, feature incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving Indicator that the UE is subject to network energy saving operation. The Energy Saving Indicator is a scalar with up to 16 values which are defined by the network operator. The behaviour related to a value is configured in the NG-RAN and PCF that receive the Energy Saving Indicator and what the possible energy saving behaviours are is outside the scope of this specification.The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF.

Editor's note:	Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS.

Editor's note:	Whether the Energy Saving indicator is also stored in the UDR is FFS.

End of changes