3GPP TSG-WG SA2 Meeting #166-AH-e	S2-2500350

Electronic Meeting, 20th Jan – 24th Jan, 2025

Source:	vivo

Title:	Discussion on Multiplexed Media Identification Information for end-to-end ciphered traffic

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract: This paper discusses the Multiplexed Media Identification Information for end-to-end ciphered traffic

1. Introduction

In SA2 Meeting #166, support of Multiplexed Media Identification Information for end-to-end ciphered traffic is discussed (see S2-2411710 and S2-2411696).

This paper discusses the feasibility that whether the Multiplexed Media Identification Information for end-to-end ciphered traffic can be supported in UL/DL or not.

2. Discussion

Each media flow can be identified by the Multiplexed Media Identification Information for end-to-end ciphered traffic in APP layer (e.g. the Full Track Name (Track Namespace and Track Name), Track Alias, stream info...). However, steering uplink packets to different QoS flows is done in the modem in the UE. But UE is a system and also include different layers. Hence, the QUIC packet, even if inside the UE, is just an encrypted data for the modem, no matter the Multiplexed Media Identification Information for end-to-end ciphered traffic is provided in the packet filter in the QoS rule or not (i.e. Modem is not able to filter the encrypted QUIC packet)

Figure 2-1: Packet filter for encrypted QUIC packet

Observation 1: The QUIC packet is not visible to the modem, and the modem is not able to filter the encrypted QUIC packet.

In order to let modem correctly identifies the Multiplexed Media Identification Information for end-to-end ciphered traffic, the APP layer needs to provide the Multiplexed Media Identification Information for end-to-end ciphered traffic along with the QUIC packet to the OS, and then OS provides it to the modem to identify the ciphered packet.

That is:

-	An XRM application needs to know the modem has such capability to identify the Multiplexed Media Identification Information for end-to-end ciphered traffic.

-	An XRM APP layer then provides the Multiplexed Media Identification Information for end-to-end ciphered traffic to the OS

-	OS should support to provide the Multiplexed Media Identification Information for end-to-end ciphered traffic to the modem

-	5GS provides the Multiplexed Media Identification Information for end-to-end ciphered traffic in the QoS rule

The lack of any step above leads to a failure of identifying the multiplexed media.

Observation 2: Even if Multiplexed Media Identification Information for end-to-end ciphered traffic is provided in the QoS rule, the UE may not be able to identify the multiplexed media.

Proposal 1: For UL, Multiplexed Media Identification for end-to-end ciphered traffic is not supported.

For DL, the Multiplexed Media Identification Information for end-to-end ciphered traffic can be added into the metadata as concluded in KI#2, the UPF could use the Multiplexed Media Identification Information to identify the DL data from the application server.

Besides, as described in IETF draft-ietf-moq-transport-07, the Track Namespace, Track Name, and Track Alias are helpful to identify a Track:

A track is a sequence of groups (Section 2.3). It is the entity against which a subscriber issues a subscription request. A subscriber can request to receive individual tracks starting at a group boundary, including any new objects pushed by the publisher while the track is active. … In MOQT, every track has a track name and a track namespace associated with it. A track name identifies an individual track within the namespace. … OBJECT message headers carry a short hop-by-hop Track Alias that maps to the Full Track Name (see Section 6.15). Relays use the Track Alias of an incoming OBJECT message to identify its track and find the active subscribers for that track. Relays MUST forward OBJECT messages to matching subscribers in accordance to each subscription's priority, group order, and delivery timeout.

According to above, the following information can be used for identifying the Multiplexed Media for QUIC packet when MoQ is used at the UPF:

-	Track Namespace;

-	Track Name;

-	Track Alias.

AF may provide the Multiplexed Media Identification Information for end-to-end encrypted traffic to PCF. Upon receipt of the Multiplexed Media Identification Information for end-to-end encrypted traffic, PCF generate the corresponding PCC rules and instructs SMF to derive the corresponding N4 rules.

Proposal 2: For DL, the Multiplexed Media Identification for end-to-end ciphered traffic can be supported and the DL packet filter can be extended to support the Track Namespace, Track Name, Track Alias, and self-defined stream ID.

3. Conclusion

Proposal 1: For UL, Multiplexed Media Identification for end-to-end ciphered traffic is not supported.

Proposal 2: For DL, the Multiplexed Media Identification for end-to-end ciphered traffic can be supported and the DL packet filter can be extended to support the Track Namespace, Track Name, Track Alias, and self-defined stream ID.