3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500370

# 20 - 24 January 2024, Electronic meeting

Source:	Ericsson

Title:	Analysis on the number of LCS-UPP connections

Agenda item:	9.6.2

Document for:	DISCUSSION

# Introduction

In SA2#165 SA2 meeting CR has been postponed defining the number of LCS-UPP connections per UE. Now this discussion paper analyses the situation and proposes the possible way forward.

# Background

LCS-UPP has been defined in Rel-18 to use user plane connection between UE and LMF to transfer LPP and supplementary services instead of using control plane. Based on stage 3 work and specification it is possible to maintain only one LCS-UPP, but it was never stage 2 intention to limit number of connections to one. The problem with one LCS-UPP connection per UE is that if AMF needs to select an LMF and if we want to leverage the user plane connection then AMF needs to select that LMF which has LCS-UPP based on UE context. In this case other selection criteria like slice information, tracking area, etc. cannot be used if those selection criteria are mutually exclusive. If operator wants to deploy LMFs for different positioning purposes and uses cases, then allowing only one LCS-UPP per UE limits the possibilities of user plane positioning solution as well as selection of dedicated LMF(s) for other services when deployed together.

Observation #1: Based on the existing solutions if an operator wants to leverage user plane connection for positioning AMF is forced to select LMF which has LCS-UPP based on UE context.

# Discussion

Based on CT4 TS 29.518 AMF can have only one LcsUPContext for the UE:

Table 6.1.6.2.84-1: Definition of type LcsUpContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upConnectionStatus | UpConnectionStatus | O | 0..1 | UP Connection Status |
| servingLMFIdentification | LMFIdentification | O | 0..1 | Serving LMF ID |

Observation #2: AMF can store only one LcsUpContext.

If AMF selects that LMF which does not have LCS-UPP - but based on UE context there is an LMF which has LCS-UPP connection to the UE – then the selected LMF may initiate LCS-UPP towards the UE which shall be rejected by the UE since UE has already LCS-UPP with other LMF. This can cause extra delay in the positioning service.

Workaround could work if one LMF is registered in NRF with LCS-UPP capability, but operators might deploy multiple LMFs for different positioning requests, and there is no reason to limit only 1 LMF can have the LCS-UPP connection to the UE.

Observation #3: Selected LMF which capable of LCS-UPP is not aware that UE has already LCS-UPP connection with other LMF then it may try initiate LCS-UPP which can cause delay in positioning service.

Rel-18 has been frozen, Stage 3 may not extend functionalities in rel18. If no opportunity to change Rel-18, the proposal is to keep this behaviour in rel-18 and accept only one LCS-UPP connection per UE. Stage 3 for rel-19 is not frozen yet therefore the proposal is to allow one LCS-UPP connections per UE towards each LMF in rel-19.

Proposal #1: Discuss and decide if in rel-18 the number of LCS-UPP connection per UE should be kept as one.

Proposal #2: Define in rel-19 UE can have one LCS-UPP connection towards each LMF.

# 4. Proposals

Proposal #1: Discuss and decide if in rel-18 the number of LCS-UPP connection per UE should be kept as one.

Proposal #2: Define in rel-19 or rel-20 that UE can have one LCS-UPP connection towards each LMF.