3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500391

# E-meeting, 20 – 24 January 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5938 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs about subscription information and policy control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 5.51.5, the following ENs should be resolved. [EN#1] Editor's note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. [EN#2] Editor's note: Whether the Energy Saving indicator is also stored in the UDR is FFS. Clause 5.51.6, the following EN should be resolved. [EN#3] Editor's note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. This CR mainly proposes that subscription information may include energy saving subscription information composing of UE level energy saving subscription information and/or Slice level energy saving subscription information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.51.5 - Remove [EN#1] while adding Slice level energy saving subscription information obtained by SMF and forwarded to PCF. - Remove [EN#2] because storing the Energy Saving indicator also in the UDR is not needed. §5.51.6 - Regarding [EN#1], describe that Slice level energy saving subscription information obtained by SMF and forwarded to PCF. - Remove [EN#3]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved ENs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.51.5	Subscription information aspects

The Subscription data for a UE may include the following one or more energy saving subscription information:

-	an UE Energy Saving indicator that the UE is subject to network energy saving operation.

-	a Slice Energy Saving indicator to indicate that the slice is subject to network energy saving operation.

The AMF receives the UE Energy Saving indicator in the UE Subscription data. The AMF forwards the UE Energy Saving indicator to the NG-RAN and to the PCF.

The SMF receives the Slice Energy Saving indicator in the UE Subscription data. The SMF forwards the Slice Energy Saving indicator to the PCF.

Editor's note:	Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS.

* * * * Start of Next Change * * * *

5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the UE Energy Saving indicator as described in clause 5.51.5. Based on the UE Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy.

During SM policy association establishment procedure, the SMF sends to the PCF the Slice Energy Saving indicator as described in clause 5.51.5. Based on the Slice Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy.

* * * * End of Changes * * * *