3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500405

# Online, 20-24 January 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5940 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference and editorial corrections to TS 23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New text according to CRs S2-2412897 and S2-2412899 has added some references that are incorrect. The text for Use of connect-UDP limits AF provisioning through the NEF, but a direct interface between a trusted AF and the PCF is also possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Wrong references to clause 5.37.9, draft-ietf-tsvwg-udp-options and draft-ietf-moq-transport corrected. Text on AF provisioning via the NEF removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect references. Not possible to connect a trusted AF to the PCF without a NEF for using connect-UDP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 5.37.9.1, 5.37.9.3 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.37.9	Support of transferring media related information over N6

#### 5.37.9.1	General

This clause describes the procedure to support transferring of media related information (i.e. information related to the media being transmitted, either explicitly provided by the AS or inferred from protocol headers or payload, e.g. PDU Set Information, Data Burst Size, etc.) over N6. Such support is necessary is when the end-to-end connection for XR media between the UE and Application Server is fully encrypted thus UPF not being able to identify, for example, PDU set information.

On-path N6 signalling methods are used to transfer media related information over N6 where media related information is sent together with the encrypted payload allowing media related information to be received synchronously with the media packet. The following on-path N6 signalling methods are supported:

-	Using Media over QUIC (MoQ), IETF draft-ietf-moq-transport [201], whose usage is further defined in clause 5.37.9.2.

-	Use of connect-UDP for Using Proxying-UDP-in-HTTP, IETF RFC 9298 [170] and QUIC-Aware Proxy, draft-ietf-masque-quic-proxy [200], whose usage is further defined in clause 5.37.9.3

-	Using UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], whose usage is further defined in clause 5.37.9.4.

When Media over QUIC is used the UPF identifies PDU Set Information and End of Data Burst Indication from media related information specified in [201].

When Proxying-UDP-in-HTTP RFC 9298 [170] or QUIC-Aware Proxy [200] or UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], is used, the following media related information are supported:

-	PDU Set Information (as described in clause 5.37.5.2).

-	End of Data Burst Indication (as described in clause 5.37.8.3).

-	Expedited Transfer Indication (as described in clause 5.37.10).

-	Data Burst Size (as described in clause 5.37.10).

-	Time to Next Burst (as described in clause 5.37.10).

The mechanisms defined in clause 5.37.9 address only UE to XRM server (AS) communication and do not address UE-UE XRM communications. The mechanisms defined in clause 5.37.9 support only downlink media related information delivery to the UPF.

The media related information is encrypted and integrity protected between the UPF and the AS, as defined in TS 33.501 [29].

Editor's note:	Other parameters related to KI#4 are FFS.

# * * * * Second change * * * *

#### 5.37.9.3	Use of connect-UDP

When the traffic is UDP and encrypted end-to-end, the UPF may be configured by the SMF to establish a connection to an HTTP/3 AS proxy using connect-UDP according to IETF RFC 9298 [170], acting as an HTTP/3 client configured to use a UDP proxy. The AS then provides media related information within HTTP datagrams. The media related information in the HTTP datagrams is protected by the security of the QUIC connection established between the UPF and the AS proxy. The mechanism for exchanging HTTP Datagrams and the associated format is defined in IETF RFC 9298 [170] and IETF RFC 9297 [172].

The AF may provide an indication of support of connect-UDP protocol to deliver media related information for encrypted traffic together with the corresponding UDP proxy address and the corresponding QoS requirements. This is done using the AF session with QoS procedure (as defined in clause 4.15.6.6 of TS 23.502 [3]), Then:

-	The PCF generates PCC rules including On-path N6 signalling information for Connect-UDP as defined in TS 23.503 [3].

-	The SMF provides N4 rules to the UPF including the corresponding On-path N6 connection information.

-	In the uplink direction, PDR rules are used to detect the UDP traffic flow subject to usage of connect-UDP and the associated FAR rules include the request to establish a connection to the AS proxy address using the connect-UDP protocol.

-	In the downlink direction, PDR rules are used to detect the UDP traffic flows and QER rules are used to associate them with proper QoS related marking (e.g. including PDU set marking as defined in clause 5.37.5),

The connection is established by the UPF, at the latest at reception of an UL UDP packet matching an N4 rule with On-path N6 connection information, if not already established. UPF may use the same or different QUIC connections for different UEs depending on implementation. In order to reuse the connection, UPF sends CONNECT requests for different UEs over a single QUIC connection to the AS proxy, opening different QUIC streams to handle the traffic for the UEs.

If the traffic is carried over QUIC, UPF and AS proxy may agree on using the Forwarded Mode in IETF draft-ietf-masque-quic-proxy [200], which allows for forwarding of packets without requiring full re-encapsulation and re-encryption. In that case, the AS proxy provides the media related information within datagrams, together with the forwarded QUIC packets.

By default, UPF and AS proxy use Tunnelling Mode in IETF draft-ietf-masque-quic-proxy [200] to exchange XRM traffic. When Tunnelling Mode is used the AS provides media related information in HTTP Datagrams.

NOTE 1:	During the QUIC connection establishment between the UE and the AS proxy, the UPF can learn the end-to-end QUIC Connection IDs that are used in the end-to-end flows from UE to AS and those used in the flows from AS to UE. Based on the learnt Connection ID(s), the UPF can negotiate with AS proxy according to IETF draft-ietf-masque-quic-proxy [200]to get Virtual Connection IDs (VCID(s)) corresponding to the end-to-end Connection IDs. These virtual CIDs can then be used by UPF and AS to support the Forwarded Mode.

When Forwarded Mode is active and possible for UL direction, the UPF sends UL QUIC packet towards the AS proxy using the Virtual Connection IDs negotiated via QUIC aware proxying with HTTP, IETF draft-ietf-masque-quic-proxy [200]; such UL traffic contains only the UDP payload received from the UE.

When Forwarded Mode is active and possible for DL direction, media related information is included by the AS into the forwarded UDP payload packets via a packet transform defined by 3GPP. The UPF uses a 3GPP defined packet transform (see TS 29.561 [132]) to retrieve the media related information and the content of the original end-to-end QUIC packet.

During the negotiation between UPF and AS proxy to activate the Forwarded Mode, the UPF and the AS proxy agree on the 3GPP packet transform to apply in the DL direction.

NOTE 2:	The connect-UDP client in the UPF can only extract the full CIDs from e2e QUIC long-header packets to use these values in REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules defined in IETF draft-ietf-masque-quic-proxy [200]. Prior to forwarding QUIC packets with short header, the connect-UDP client (UL) and the proxy (DL) check whether the destination CID is identical to an already registered CID, if not, the packets are sent in tunnelled-mode.

NOTE 3:	The format of HTTP Datagrams and the format of Forwarded Mode Datagrams carrying media related information are defined in TS 29.561 [132].

# * * * * End of changes * * * *