3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500412

# Electronic meeting, 2025-01-20 – 2025-01-24

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5944 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology corrections for the support of multimodal services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The term “data flow” is used in the specification with two different meanings, when it comes to the support multi-modal services. - In clause 5.37.2, the term “data flow” refers to the different types of data that comprise a multi-modal service. However stage 3 (3GPP TS 29.514) makes use of the existing term “media flow” to describe multi-modal services - In clause 5.37.6, the term “data flow” is used with the meaning of “service data flow”. This may lead to incorrect implementation of the specification and inter-vendor operability problems |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the terminology “data flow” by “media flow” or by “service data flow” as appropriate |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification due to misleading terminology |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.2, 5.37.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

### 5.37.2	Policy control enhancements to support multi-modal services

A multi-modal service is a communication service that consists of several media flows that relate to each other and that are subject to application coordination. The media flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources (e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs).

For the single UE case, it is expected that those media flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those media flows are transmitted in a single PDU session.

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each media flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that the AF session is related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the service data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for media flows associated to a multi-modal service to the PCF. The PCF generates the authorized QoS Monitoring policy for each service data flow.

NOTE:	In order to start the QoS monitoring for the service data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those media flows from AF within a single request or, in case of multiple requests, within a short period of time.

In addition to the features that are provided for the case that the media flows are associated with a single UE, the following features are provided for the case where the media flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service may be selected by each of the involved UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF may use the same Multi-modal Service ID in the interactions with the PCF(s) for all the involved UEs that relate to a multi-modal service. The PCF may take this information into account (e.g. to apply a specific QoS policy) when processing each AF request independently. The media flows contribute to the service experience, but are still valid stand-alone, as they are transmitted over separate PDU Sessions to/from the involved UEs.

-	If multiple PCFs are involved, the PCFs take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCFs in this Release of the specification. Policy decisions are taken by each PCF separately on a per PDU Session basis.

************** 2nd change **************

### 5.37.6	UL/DL policy control based on round-trip latency requirement

For XR and other interactive media services that require very low Round-Trip (RT) latency, Uplink-Downlink policy control may be supported to meet the RT latency requirement. RT latency requirement is the upper bound for the sum of UL delay and DL delay of a single service data flow or two different service data flows between UE and N6 termination point at the UPF. PCF may support Uplink-Downlink policy control based on RT latency requirement based on an RT latency indication from AF (as defined in clause 6.1.3.27.2 of TS 23.503 [45]) during the AF session with the required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

The AF can provide an RT latency indication with a single direction delay requirement between the UE and the PSA UPF expressed as the QoS Reference parameter or individual QoS parameters (as defined in clause 6.1.3.22 of TS 23.503 [45]). The RT latency indication indicates the need to meet the RT latency requirement for the service data flow, i.e. doubling of the single direction delay requirement between the UE and the PSA UPF expressed by the QoS Reference parameter or individual QoS parameter.

PCF determines the service data flow's UL PDB and DL PDB based on the RT latency requirement. The UL PDB and DL PDB can be unequal, but their sum shall not exceed the RT latency requirement. The PCF shall generate two PCC rules, one for UL QoS flow for UL traffic of the service data flow and one for DL QoS flow for DL traffic of the service data flow, respectively. PCF shall assign the 5QIs for each of these two PCC rules according to the derived UL PDB and DL PDB.

To support UL and DL delay tracking, the QoS monitoring for UL packet delay and the DL packet delay (as defined in clause 6.1.3.21 of TS 23.503 [45]) shall be triggered respectively to request tracking the UL packet delay of the QoS flow used in UL and DL packet delay of the QoS flow used in DL independently. Based on the QoS monitoring results, the PCF may readjust the UL PDB and/or DL PDB under the consideration of the RT latency requirement to better fit the new situation.

The Uplink-Downlink policy control based on round-trip latency requirement for two unidirectional service data flows is described in clause 6.1.3.27.2 of TS 23.503 [45].

NOTE:	How the PCF derives the round-trip latency and takes policy decisions is up to the implementation.

************ End of changes ************