3GPP TSG-SA2 Meeting #166AH	S2- 2500416

Emeeting, Elbonia,  20 – 24th January, 2025	(was S2-251xxxx)

Source:	Nokia

Title:	Discussion paper on missing Optional LLDP TLVs

Document for:	Discussion

Agenda Item:	7.7

Work Item / Release:	Vertical_LAN

Abstract of the contribution: This paper explains the issue of missing optional TLVs and recommends including them.

# 1.	Discussion

## 1.1	Overview

In an industrial network, most of the communication takes place through Ethernet based networks. These networks have been deployed over time as factories/industries evolved and 5GS must adapt to such existing industrial ethernet networks to allow for a brownfield integration.

The Link Layer Discovery Protocol (LLDP, IEEE Std 802.1AB [3]) is a vendor-neutral network discovery protocol that plays a pivotal role throughout the network industry in enterprise, data centre and carrier networks helping network devices to advertise their identity and capabilities to neighbours. The content of a LLDP message is defined in a Protocol Data Unit (LLDPDU), which is made up of several consecutive TLVs (Type-Length-Values). [IEEE Std 802.1AB-2016] defines two different TLV categories in clause 8.3: the basic management set (TLV type 0 – TLV type 8) and the Organizationally Specific TLV (TLV type 127). The table below differentiates the LLDPDU types according to their usage into Optional and Mandatory TLV types.

| TLV type (*) | TLV Name | Usage in LLDPDU |
| --- | --- | --- |
| 0 | End Of LLDPDU | Optional |
| 1 | Chassis ID | Mandatory |
| 2 | Port ID | Mandatory |
| 3 | Time To Live (TTL) | Mandatory |
| 4 | Port Description | Optional |
| 5 | System Name | Optional |
| 6 | System Description | Optional |
| 7 | System Capabilities | Optional |
| 8 | Management Address | Optional |
| 9-126 | reserved | -- |
| 127 | Org. Specific TLVs | Optional |

(*) TLVs with type values 0-8 are members of the basic management set

Table 1: TLV type values [IEEE Std. 802.1AB- 2016] section 8.4.1

In addition to the Mandatory TLVs: Chassis ID TLV, Port ID TLV and TTL TLV, an LLDPDU can also contain Optional TLVs (OTLVs) whereby the transmission of OTLVs can be tailored based on network requirements. It is best practice to selectively enable or disable OTLVs to optimize LLDP's functionality for specific network needs.

[IEC/IEEE 60802] determines the minimum OTLV set for the operation of an Industrial Ethernet network:

- a single or multiple Management Address TLV(s), a single System Capabilities TLV as defined in [IEEE Std 802.1AB-2016]

- additional Ethernet Capabilities TLV as defined in [IEEE Std 802.3-2018].

According to [IEC/IEEE 60802] IA-stations announce themselves via LLDP to support discovery by a Topology Discovery Entity (TDE) whereby the resulting topology information is used by the CNC. LLDP announcements contain the management address and system capabilities for the discovery operation e.g. the announced system capabilities information enables the topology discovery entity to identify IA stations with multiple end station and bridge components. For further details refer to [IEC/IEEE 60802]. These TLVs are missing in current 3GPP specifications in [3GPP TS 23.501] and should be supplemented accordingly, without which the deployments are limited as described above; each deployment may require to use different TLVs; for example, management address might be required in one deployment while without system capabilities LLDP exchange may not be affectively used as required in a specific deployment.

Moreover, in a specific application case LLDP helps in the onboarding process the CNC to identify if a legacy end station needs communication via a proxy. During the process, the CNC finds out whether the connected station requires a proxy, and if so, which one through the LLDP information provided to the neighboring 5GS bridge.

# 2	Way forward

The current LLDP functionality of the 5GS bridge needs to include the missing LLDP TLVs

- Resolve the dependencies to [IEC/IEEE 60802] and other standards which use LLDP according to IEEE Std 802.1AB-2016, e.g. IEEE Std 802.1Q-2018 Annex D on “IEEE 802.1 Organizationally Specific TLVs” , .

In the 5GS bridge the dis-/assembly of the LLDPDUs is carried out at the u-plane. However, the YANG/MIB data models of the associated TLV(s) are processed in the AF. Currently, only the information for the Mandatory TLVs is exchanged between the AF and the u-plane via the existing UMIC/PMIC mechanism. Therefore, UMIC/PMIC mechanism shall be supplemented by an OtlvSet which consists of the concatenated list of the missing TLVs.

Accordingly, the corresponding CRs with CR No: 5945, 5946, 5947, 5948 are produced.

# 3.	References

[1]	3GPP TS 23.501: "System Architecture for the 5G System".

[2]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[3]	3GPP TS 24.519: "Time-Sensitive Networking (TSN) Application Function (AF) to Device-Side TSN Translator (DS-TT) and Network-Side TSN Translator (NW-TT) protocol aspects".

[4]	IEEE Std 802.1AB-2016: “Station and Media Access Control Connectivity Discovery”.