3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500427

Online, January 20 - 24, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#2 Conclusion Update of Device Subscription Data

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discuss and update KI#2 conclusion to update the EN about Device Subscription Data.

1. Introduction

In SA2#166, regarding the device subscription-like data, there are some interim conclusions reached while it is still open on where the device subscription data are stored within 5GC:

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the Subscription data or related data of an Ambient IoT Device is.

Editor’s note:	Where to store the AIoT device subscription is FFS.

There was a proposal to reuse the architecture which was developed for UE. That is, device subscription-like data are stored in UDM (i.e., the data are stored in UDR, while the access is via UDM). This proposal caused some concerns on the impacts on the UDM.

There was some discussions on that the device subscription-like data are stored in UDR without UDM impact.

In our view, the access of the device subscription-like data are mainly driven by device authentication. For device authentication, it depends on SA3’s conclusions. From SA2’s perspective, there are some options which can be foreseen:

- Option 1: Reuse the existing authentication framework. The AIOTF interacts with the AUSF. And the AUSF interacts the UDM for device subscription-like data in 5GC. For external data, the AUSF interacts with the NSSAAF towards external AAA.

- Option 2: Develop a new authentication framework. In this case, the AIOTF may interacts with UDR for the device subscription-like data in 5GC. For external data, the AIOTF interacts with the NSSAAF towards external AAA. Within option 2, there could also be other different options on whether AUSF and/or UDM are involved in the call flow.

[Observation-1] Based on the options of the authentication framework, there are different solutions for the storage of device subscription-like data.

There is also an EN in clause 5.5 of TR 33.713 “Key Issue#5: Authentication in Ambient IoT service”:

Editor’s Note: Secure storage and processing of credentials (on the AIoT device and network side) for solution not reusing existing authentication framework as per the note above is ffs.

[Observation-2] SA3 is also looking into the storage of credentials on the network side. And whether to reuse authentication framework is FFS.

[Proposal-1] SA2 needs to work together with SA3 about the storage of device subscription-like data.

[Proposal-2] Send an LS to SA3 and update the EN to work together with SA3.

In the interim conclusions, there also another 2 ENs which reflects the dependency with SA3:

Editor’s note:	SA WG2 will align security related materials in subscription data with SA WG3 decision later.

……

Editor’s note:	Items within the subscription data for 3rd party AF need to be figured out later.

As both SA2 and SA3 are in conclusion phase, and there are two meetings to discuss Ambient IoT. And in February meeting, SA2 and SA3 can have joint meeting if needed. Those ENs can be addressed and concluded in the study phase, without further delay towards normative phase.

[Proposal-3] Update those two ENs to reflect the proposal above.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.2.1:

* * * Start of Change * * * *

### 8.2.2	Subscription Management

The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network.

Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network:

-	It is used by the AIOTF to check whether the device is subscribed.

NOTE 1:	Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier.

-	It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data.

-	It contains at least a permanent Ambient IoT Device ID, etc.

Editor’s note:	SA WG2 will align security related materials in subscription data with SA WG3 decision.

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the Subscription data or related data of an Ambient IoT Device is.

Editor’s note:	Where to store the AIoT device subscription requires cooperation with SA3.

Subscription data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service.

-	it is stored at UDM within 5G network.

Editor’s note:	Items within the subscription data for 3rd party AF need to be figured out.

UE Subscription data is enhanced to include an indication that whether the UE is allowed to operate as a UE reader.

* * * End of Change * * * *