3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500489

# Online, 20th Jan 2025 - 24th Jan 2025	                                                  revision of S2-2xxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5248 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 AF traffic influence information for I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes that the AF traffic influence information can be delivered to the I-SMF to address the following case: Reporting UP path change to AF can be done by I-SMF to AF directly in order to reduce impact on the centrally deployed SMF. When Local Offloading Management is applied, the I-SMF can report UP path change to AF directly, i.e., not via SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose SMF to deliver AF traffic influence information to I-SMF Procedure update for reporting UP path change to AF when Local Offloading Management is applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclarified and misaligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.6.2, 4.23.6.3, 5.2.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

4.23.6.2	Policy Update Procedures with I-SMF

Figure 4.23.6-1 shows procedures related to provisioning of PCC rules containing traffic steering information related to an I-SMF.

Figure 4.23.6-1: Policy Update procedure

In cases where step 1a in figure 4.23.6-1 is triggered in response to PCF receiving AF request, below steps 3 and 4 are applicable, in addition to those steps as explained in clause 4.3.6.1.

Step 3:	SMF provides to I-SMF with DNAI(s) of interest for this PDU Session for local traffic steering. If PCC rule changes for traffic offloaded via ULCL/BP due to the AF request, the SMF provides the updated N4 information to the I-SMF. If PCC rule changes Local Offloading Policy, SMF provides Local Offloading Policy to I-SMF. If Local Offloading Management is applied, the SMF provides AF traffic influence information to the I-SMF.

Editor's note:	It is FFS whether SMF transfers transparently Local Offloading Policy to I-SMF.

Step 4: The procedure described in clauses 4.23.9.1, 4.23.9.2 and 4.23.9.3, from step 2 is executed.

>>>> Next Changes <<<<

4.23.6.3	Reporting UP path change to the AF

Figure 4.23.6.3-1 shows procedures related Reporting UP path change to the AF.

Figure 4.23.6.3-1: Reporting UP path change to the AF

1a.	I-SMF indicates that UP path change may occur for the PDU Session via Nsmf_PDUSession_Update Request as described in clause 4.23.9; the SMF responds to the I-SMF.

2a.	If early notification has been requested by a PCC rule on behalf of AF as described in clause 4.3.6.2, then the SMF notifies the AF accordingly by invoking Nsmf_EventExposure_Notify service operation as described in clause 4.3.6.3. In this case the SMF may wait for further instructions of the AF.

2b. If Local Offloading Management is applied and I-SMF manages the UP path change notification to AF and if early notification has been requested by AF, the I-SMF notifies the AF accordingly as SMF does in clause 4.3.6.3. In this case, the SMF does not send notification to the AF.

3.	SMF initiates Nsmf_PDUSession_Update Request with N4 information to control the local PSA and ULCL/BP as described in clause 4.23.9.

4.	I-SMF enforces the change of DNAI or addition, change, or removal of UPF as described in clause 4.23.9.

5	I-SMF answers back to the Nsmf_PDUSession_Update from the SMF.

6.	If late notification has been requested by a PCC rule on behalf of AF as described in clause 4.3.6.2, then the SMF notifies the AF accordingly by invoking Nsmf_EventExposure_Notify service operation as described in clause 4.3.6.3.

6b.	If Local Offloading Management is applied and I-SMF manages the UP path change notification to AF and if early notification has been requested by AF, the I-SMF notifies the AF accordingly as SMF does in clause 4.3.6.3.

>>>> Next Changes <<<<

5.2.8.2.2	Nsmf_PDUSession_Create service operation

Service operation name: Nsmf_PDUSession_Create.

Description: Create a new PDU Session in the H-SMF or SMF or create an association with an existing PDN connection in the home SMF+PGW-C.

Input, Required: SUPI, V-SMF ID or I-SMF ID, V-SMF SM Context ID or I-SMF SM Context ID, DNN, V-CN Tunnel Info or I-UPF Tunnel Info, addressing information allowing the H-SMF to request the V-SMF to issue further operations about the PDU Session or addressing information allowing the SMF to request the I-SMF to issue further operations about the PDU Session, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.18 of TS 23.501 [2]).

Input, Optional: S-NSSAI, Alternative S-NSSAI, PCO, Requested PDU Session Type, 5GSM Core Network Capability, Requested SSC mode, PDU Session ID, Number Of Packet Filters, UE location information, subscription get notified of PDU Session status change, PEI, GPSI, AN type, PCF ID, PCF Group ID, DNN Selection Mode, UE's Routing Indicator optionally with Home Network Public Key identifier or UDM Group ID for the UE, Always-on PDU Session Requested, Control Plane CIoT 5GS Optimisation Indication, information provided by V-SMF related to charging in home routed scenario (see TS 32.255 [45]), AMF ID, EPS Bearer Status, extended NAS-SM timer indication, DNAI list supported by I-SMF (from I-SMF to SMF), HO Preparation Indication. MA PDU request indication, MA PDU Network-Upgrade Allowed indication, Indication on whether the UE is registered in both accesses; QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite backhaul category, Notification of the SM Policy Association Establishment and Termination, PCF binding information, Disaster Roaming service indication, HR-SBO request indication, VPLMN EASDF/Local DNS Server/Resolver IP address, DNS security information of VPLMN EASDF/Local DNS Server/Resolver, [ECS Address Configuration Information associated with PLMN ID of visited network], Indication of UE supports non-3GPP access path switching, [URSP rule enforcement reports], NG RAN QoS monitoring capability (as defined in clause 5.45.1 of TS 23.501 [2]), EASDF IP address (from I-SMF to SMF), DNS security information of EASDF (from I-SMF to SMF), Local Offloading Management support indication (from I-SMF to SMF).

Output, Required: Result Indication and if success a SM Context ID and in addition: QFI(s), QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s), H-CN Tunnel Info or PSA UPF Tunnel Info, Enable pause of charging indication, Selected PDU Session Type and SSC mode.

Output, Optional: PDU Session ID, S-NSSAI, Cause, PCO, UE IP address, IPv6 Prefix allocated to the PDU Session, information needed by V-SMF in the case of EPS interworking such as the PDN Connection Type, EPS bearer context(s), linked EBI, Reflective QoS Timer, Always-on PDU Session Granted, information provided by H-SMF related to charging in home routed scenario (see TS 32.255 [45]), DNAI(s) of interest for this PDU Session (from SMF to I-SMF), indication of multi-homing support (from SMF to I-SMF). MA PDU session Accepted indication, Indication on whether Small Data Rate Control applies or not, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN DNS Server address, Local DNS Server/Resolver address in VPLMN, HPLMN address information (e.g. H-UPF IP address on N6), AF traffic influence information (from H-SMF to V-SMF in case AF interacts with HPLMN to influence HR-SBO session at VPLMN, from SMF to I-SMF in Local Offloading Management scenario described in TS 23.548 [74]), Internal Group Identifier(s). Local Offloading Management allowed indication (from SMF to I-SMF), Local Offloading Management information (from SMF to I-SMF).

The V-SMF SM Context ID in the Input provides addressing information allocated by the V-SMF (to be used for service operations towards the V-SMF for this PDU Session).

The I-SMF SM Context ID in the Input provides addressing information allocated by the I-SMF (to be used for service operations towards the I-SMF for this PDU Session).

See clause 4.3.2.2.2 clause 4.11.1.2.2 clause 4.11.1.3.3 and clause 4.24 for details on the usage of this service operation.

See clauses 4.22.2.2 and 4.22.3 for detailed usage of this service operation for ATSSS.

See clause 6.7 of TS 23.548 [74] for HR-SBO request indication, HR-SBO authorization result, VPLMN EASDF address, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN DNS Server address, HPLMN address information.

See clause 6.5.2.6 of TS 23.548 [74] for details on the EAS Configuration Address Information provisioning in roaming.

>>>> End of Changes  <<<<