3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500502

Electronic Meeting, 20-24 January 2025

# Title:	[Draft] Reply LS on relay discovery announcement

# Response to:	LS to SA2 on relay discovery announcement (R2-2411184/ S2-2500045)

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	RAN2

Cc:	-

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 0537 to TS 23.304

1. Overall Description:

SA2 would like to thank RAN2 for their LS (R2-2411184/ S2-2500045) relay discovery announcement.

SA2 has discussed the questions in the LS for SA2 and would like to provide the answers as follows:

- Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

SA2’s answer: For multi-hop UE-to-Network Relay discovery, both Model A and Model B discovery are supported, as documented in TS 23.304 clause 6.3.2.5. The 5G ProSe UE-to-Network Relay Discovery Announcement message is used in Model A discovery only, as in clause 6.3.2.5.2. The Intermediate Relay would need to establsh a connection with the parent relay before sending the Relay Discovery Announcement message (as stated in clause 6.3.2.5.2).

SA2 has agreed the attached CR to make the requirement clearer.

SA2 would like to ask RAN2 to take the above information in account.

2. Actions:

To RAN2:

ACTION: 	RAN2 to take the above answers from SA2 into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden