3GPP TSG-WG SA2 Meeting #166 Ad-Hoc-e	S2-2500521

20th – 24th January, 2024, Online

Source:	China Mobile

Title:	AMF function enhancement for Ambient IoT service

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT

Abstract: This paper proposes AMF function enhancements for Ambient IoT service.

1. Introduction/Discussion

This contribution is prepared to propose AMF function enhancements for Ambient IoT service in 4.5.7.

2. Text Proposal

It is proposed to approve the following changes.

FIRST CHANGE

- Architecture model and concepts

4.5	Functional Entities

4.5.7	  AMF

For indirect path of Topology1, the Access and Mobility Management function (AMF) should be enhanced in the following functionality to support Ambient IoT services.

- Support of AIoT RAN node recognition

- Support of AIoT RAN node information delivery to AIoTF

- Support of transport for service messages between AIoT RAN node and AIoTF.

END OF CHANGES