3GPP TSG-SA2 Meeting #166AH-e	S2-2500530

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5970 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on roaming support for Non-3GPP Device Identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Unclear whether QoS differentiation is supported while a UE is roaming. According to clause 6.1.3.31 of TS 23.503, there is a reference to clause 4.3.3.2 of TS 23.502 which is non-roaming/LBO procedure, i.e. home-routed roaming is not included. In addition, in case of LBO, V-PCF cannot connect H-UDR. Therefore, roaming cannot be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that while a UE is roaming, QoS differentiation for non-3GPP device is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether the 5GC can provide QoS differentiation for non-3GPP device behind a UE when the UE is roaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.52.1	General

This clause specifies the scenario of a non-3GPP device connecting through the UE. In this scenario QoS differentiation of traffic is applied to the traffic that originates from or is directed to the non-3GPP device. The non-3GPP device does not use NAS and is not authenticated by 5GC.

The support of identification of traffic for non-3GPP devices connecting behind a 5G-RG is specified in TS 23.316 [84].

The Non-3GPP Device Identifier is unique within the scope of the UE's SUPI.

In this release of specification, QoS differentiation for non-3GPP device connecting behind a UE while the UE is roaming is not supported.

* * * * End of Changes * * * *