3GPP TSG-SA2 Meeting #166AH-e	S2-2500532

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1478 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behaviour when Non-3GPP Device Identifier is not associated with the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Current description says that UE sends Non-3GPP Device Identifier and optionally address information (e.g. MAC address, VLAN tag ID, IP address). However, without address information, 5GC cannot provide QoS differentation for non-3GPP devices. 2. It is not clear how to handle non-3GPP device when the UE received cause indicating that Non-3GPP Device Identifier is not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that UE sends both Non-3GPP Device Identifier and address information. 2. Clarify how the UE handles non-3GPP device when the UE received cause indicating that Non-3GPP Device Identifier is not available. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot provide QoS differentiation for non-3GPP device. Unclear UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and the corresponding user plane address as specified in clause 4.3.3.2 of TS 23.502 [3]. When a Policy Control Request Trigger is met, the SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information which is retrieved from the UDR and/or based on operator policy. The PCF installs/updates the PCC Rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

If the Non-3GPP Device Identifier is not included within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, then the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE. In case of PDU Session Modification, the PCF shall reject PDU Session Modification. When PDU Session Modification is rejected, a cause code is used to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

NOTE:	When a UE received a cause code that notifying the Non-3GPP Device Identifier is not available for the UE, how to handle the non-3GPP device is up to UE implementation, e.g. keep connection with non-3GPP device without supporting differentiated QoS, disconnecting the non-3GPP device, binding to another Non-3GPP Device Identifier, etc.

* * * * End of Changes * * * *