3GPP TSG-SA2 Meeting #166AH-e	S2-2500535

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5972 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving remaining editor's notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SliceSel |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. There is an editor's note on whether Replaced S-NSSAI should be removed from the Configured NSSAI. It is not clearly described which cause is used when the AMF rejects UE's request but our understanding is that AMF rejects the Replaced S-NSSAI for the Registration Area of the UE. However, this may cause issue over non-3GPP access. In non-3GPP access, RA is associated with a single TA. If the AMF rejects Replaced S-NSSAI for the Registeration Area over non-3GPP access, the UE cannot use Replaced S-NSSAI anymore over non-3GPP access. Even in 3GPP access, if the UE does not move out of the RA, the UE cannot request Replaced S-NSSAI until the AMF removes the Replaced S-NSSAI from the Rejected S-NSSAI. Therefore, anyhow AMF need to remember Rejected S-NSSAI provided to the UE and remove the Replaced S-NSSAI from the Rejected S-NSSAI to resolve above issues. Then, it is questionable whether adding Replaced S-NSSAI into the Rejected S-NSSAI is better approach compared to updating Configured NSSAI. If Replaced S-NSSAI is removed from Configured NSSAI, the UE does not request Replaced S-NSSAI whenever the UE performs Mobility Registration update. Therefore, Removing from the Configured NSSAI is more cleaner approach. Some companies may claim that if UE moves to non-supporting AMF, Configured NSSAI cannot be updated. However, if there is non-supporting AMF, this feature would not work e.g. when a UE moves from a supporting AMF to a non-supporting AMF, the non-supporting AMF provides Replaced S-NSSAI into Allowed NSSAI. Therefore, it is reasonable assumpetion that all AMFs supports this feature. 2. There is an editor's note on Network Slice Replacement Type. The detailed value of the type can be defined in stage-3 and it can be removed from the spec. If needed, SA2 can align with stage-3 spec when CT WGs complete stage-3 work. 3. There is an editor's note on PCRT. This can be removed since TS 23.503 already defines reporting Change of Allowed NSSAI. If the AMF includes Alternative S-NSSAI in the Allowed NSSAI, the PCF knows that AF requested modification has been completed. 4. It is not clear how to release PDU Sessions using the Replaced S-NSSAI. Since Replaced S-NSSAI is removed from the Allowed NSSAI, the PDU Session can be locally removed in the UE and AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that all AMFs supporting Replace S-NSSAI and Alternative S-NSSAI are assumed to support AF requested Network Slice Replacement. 2. Removes EN on Network Slice Replacement Type 3. Removes EN on PCRT to notify outcome of Network Slice Replacement with clarification that PCF uses exiting PCRT to determine outcome of AF requested Network Slice Replacement. 4. Removes EN on whether Replaced S-NSSAI should be removed from the Configured NSSAI with clarification that Replaced S-NSSAI is removed from the Configured NSSAI. 5. Clarify that AMF and UE locally removes PDU Sessions associated with Replaced S-NSSAI when AMF triggers AF requested Network Slice Replacement. 6. Clarify that the PCF need to assign lower precedence to the RSD with the Replaced S-NSSAI compared to the RSD with the Alternative S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.5.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 5.15.5.2.2a	AF Requested modification of the Set of Network Slice(s) for a UE

In non-roaming cases, an authorized AF or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The request in either case is sent to the PCF for the UE handling the impacted subscriber. Similarly, an authorized AF or an AF subject to authorization via the NEF, may request to terminate the replacement then request to use the Replaced S-NSSAI instead of the Alternative S-NSSAI for a UE again. The AF may subscribe to Notification on the outcome of the AF requested Network Slice Replacement.

Only a single AF can request the modification of the Set of Network Slice(s) for one replaced S-NSSAI.

NOTE 1:	It is assumed that all AMFs supporting Replaced S-NSSAI and Alternative S-NSSAI in the network supports AF requested Network Slice Replacement. How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

The PCF for the UE based on operator policies considers the AF request. Consequently the PCF for the UE triggers the Network Slice Replacement at the AMF as defined in TS 23.503 [45].The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503 [45]. The Slice replacement management includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by AF. The PCF uses existing policy control request triggers, i.e. "Change of Allowed NSSAI" as defined in clause 6.1.2.5 of TS 23.503 [45] to detect whether the AMF completed AF requested Network Slice Replacement. When the PCF detects complete of AF requested Network Slice Replacement, the PCF notifies the outcome to the AF if the AF subscribed to the notification. The AMF handles the procedure as following:

-	The AMF shall not handle the Network Slice Replacement procedure as described in clause 5.15.19.

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF shall notify the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further notify the AF.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present).

-	The AMF removes the Replaced S-NSSAI from the Allowed NSSAI and removes the Replaced S-NSSAI from the Configured NSSAI by a UE Configuration Update procedure.

-	The AMF and the UE locally release the PDU sessions that were using the Replaced S-NSSAI.

-	The AMF stores in the UE context the access and mobility management policy information as received from PCF and transfer to new AMF during inter AMF mobility.

NOTE 2:	Operator policies in the PCF ensures that the UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session. The precedence value of RSD with the Replaced S-NSSAI should be lower than the precedence value of RSD with the Alternative S-NSSAI, which enables the UE to prioritize using the Replaced S-NSSAI when both Replaced S-NSSAI and Alternative S-NSSAI are included in the Allowed NSSAI.

The trusted AF or a NEF may send request to PCF for the UE to stop the Network Slice Replacement. In this case the PCF for the UE sends access and mobility management policies to the AMF, as described in TS 23.503 [45]. The AMF removes the Alternative S-NSSAI from the Allowed NSSAI and adds the Replaced S-NSSAI to the Configured NSSAI by a UE Configuration Update procedure. The UE may request to register the Replaced S-NSSAIs again next time the UE sends a Requested NSSAI.

* * * * End of Changes * * * *