3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500542

20th – 24th January 2025, E-meeting	(revision of S2-25xxxxx)

Source:	Lenovo

Title:	KI#1, Nx transport layer association establishment in Topology 1

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to introduce a new statement in clause 8.1.2.2 “AIOT RAN and the AIOTF communicate directly” to document the Nx transport layer association.

# 1	Discussion

The clause 8.1.2.2 “AIOT RAN and the AIOTF communicate directly” doesn’t describe the Nx transport layer association. However, this is a critical aspect, on which an agreement is needed, since the AIoT RAN reader AIOTF needs to have sufficient information for AIoT RAN reader selection.

The first missing step is that the AIOT RAN reader needs to initiate the Nx transport layer association establishment with the AIOTF. It is proposed that the AIOT RAN reader is locally configured with the AIOTFs, to which the Nx transport layer association is established, therefore there is no need to specify AIOTF selection functionality.

Proposal 1:	The AIOT RAN initiates the establishment of the Nx transport layer association with the AIOTF (e.g. using NGAP) when the AIOT RAN is instantiated. The AIOT RAN is locally configured with the AIOTF(s) to which Nx transport layer association is established.

During the Nx transport layer association establishment, the AIOT RAN reader has to provide to the AIOTF information needed for the AIOT RAN reader selection. One needed information is the supported AIoT radio service area. Another information can be whether the AIoT RAN is dedicated to a specific AIoT customer, it means to specific AIoT AF ID or service ID. The details of the needed information can be discussed during the normative phase and together with RAN3 WG.

Proposal 2:	During the Nx transport layer association establishment, the AIOT RAN provides to the AIOTF information to assist the AIOT RAN reader selection (e.g. supported AIoT service area).

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

#### 8.1.2.2	AIOT RAN and the AIOTF communicate directly

When a AIOT RAN and the AIOTF communicate directly:

-	The AIOTF communicates with a AIOT RAN via a direct interface reference point Nx.

-	Figure 8.1.2.2-1 below shows the aspects related to Topology 1 (direct path) architecture in reference point representation architecture with other NFs removed.

Figure 8.1.2.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2.2-2 below shows the aspects related to Topology 1 (direct path) architecture in reference point representation with other NFs removed.

Figure 8.1.2.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

NOTE 1:	NGAP used over Nx reference point will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 2:	The protocol stack used between the AIoTF and the AIOT RAN will be concluded by RAN WG3.

-	The AIOT RAN initiates the establishment of the Nx transport layer association with the AIOTF (e.g. using NGAP). The AIOT RAN is locally configured with the AIOTF(s) to which Nx transport layer association is established.

-	During the Nx transport layer association establishment, the AIOT RAN provides to the AIOTF information to assist the AIOT RAN reader selection (e.g. supported AIoT service area).

NOTE X:	The detailed information exchanged between the AIOT RAN and the AIOTF during the Nx transport will be specified together with the RAN WG3 during the normative phase.

-	Figure 8.1.2.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the AIOT RAN and AIoTF.

Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

NOTE 3:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be determined by RAN WG3.

# * * * * End of changes * * * *