3GPP TSG-SA2 Meeting #166AH-e	S2-2500548

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3853 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving an EN about List of satelliteID(s) and S&F Monitoring List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper address following EN about the "List of satelliteID(s)" and/or "S&F Monitoring List" concluded in the study. Editor's note: How to handle the specification of the "List of satelliteID(s)" and "S&F Monitoring List" as per conclusions in TR 23.700-29 [96] is FFS. First, one single list is enough. The lists commonly indicates to the UE that the UE may re-attempt or exchange data with these satellite in this list. We propose to use S&F reject cause to indicate the UE should follow the S&F monitoring list provided by the MME, while it does not restrict the UE to access only to the satellites in the S&F monitoring list. (e.g. UE can still access to the full CN satellite which my already have subscription data or terrestrial network) In this way, UE can access any of satellite not part of S&F Monitoring list, and helping the UE avoid being rejected by the network consecutively. We also added below NOTE which was discussed in SA2#166 meeting, to prevent the attached UE to be detched when receive the NAS reject due to S&F operation. NOTE 4: If the UE is rejected with a cause indicating the NAS rejection is due to S&F operation, the UE’s EMM state shall remain unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the handling of S&F Monitoring list. It is clarified that the satellites included in S&F Monitoring list supports S&F operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not support "List of satelliteID(s)" and/or "S&F Monitoring List" concluded in TR 23.700-29. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[4]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[5]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[6]	3GPP TS 23.203: "Policy and charging control architecture".

[7]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[8]	3GPP TS 43.129: "Packet-switched handover for GERAN A/Gb mode; Stage 2".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[11]	3GPP TS 43.022: "Functions related to MS in idle mode and group receive mode".

[12]	3GPP TS 25.304: "UE procedures in idle mode and procedures for cell re-selection in connected mode".

[13]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[14]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[15]	3GPP TS 43.051: "GERAN Overall description - Stage 2".

[16]	3GPP TS 25.401: "UTRAN overall description".

[17]	IETF RFC 1034 (1987): "Domain names – concepts and facilities" (STD 13).

[18]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[19]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[20]	Void.

[21]	Void.

[22]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[23]	3GPP TS 44.064: "Mobile Station - Serving GPRS Support Node (MS-SGSN); Logical Link Control (LLC) Layer Specification".

[24]	3GPP TS 23.251: "Network Sharing; Architecture and functional description".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	IETF RFC 768: "User Datagram Protocol".

[27]	3GPP TS 23.221: "Architectural requirements".

[28]	3GPP TS 23.008: "Organization of subscriber data".

[29]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL) Phase X; Stage 2".

[30]	3GPP TS 23.236: "Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes".

[31]	IETF RFC 3588: "Diameter Base Protocol".

[32]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[33]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[34]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[35]	IETF RFC 4960: "Stream Control Transmission Protocol".

[36]	3GPP TS 36.413: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[37]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[38]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[39]	Void.

[40]	3GPP TS 33.102: "3G Security; Security architecture".

[41]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

[42]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS Protocol (BSSGP)".

[43]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[44]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[45]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[46]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[47]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[48]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[49]	3GPP TS 22.042: "Network Identity and Time Zone (NITZ) service description; Stage 1".

[50]	Void.

[51]	3GPP TS 32.240: "Charging architecture and principles".

[52]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[53]	3GPP TS 24.285: "Allowed Closed Subscriber Group (CSG) List; Management Object (MO)".

[54]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[55]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[56]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[57]	3GPP TS 23.271: "Functional stage 2 description of LCS".

[58]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[59]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[60]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[61]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3".

[62]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[63]	IETF RFC 3810: "Multicast Listener Discovery Version 2 (MLDv2) for IPv6".

[64]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[65]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[66]	3GPP TS 22.368: "Service Requirements for Machine-Type Communications (MTC); Stage 1".

[67]	3GPP TS 22.011: "Service Accessibility".

[68]	3GPP TS 22.153: "Multimedia priority service".

[69]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[70]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[71]	3GPP TS 23.002: "Network Architecture".

[72]	3GPP TS 23.007: "Restoration procedures".

[73]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[74]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[75]	3GPP TS 23.380: "IMS Restoration Procedures".

[76]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[77]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[78]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[79]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[80]	3GPP TS 22.101: "Service aspects; Service principles".

[81]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[82]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[83]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[84]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[85]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[86]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[87]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access -E-UTRA); Medium Access Control -MAC) protocol specification".

[88]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[89]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[90]	3GPP TS 38.401: "NG-RAN; Architecture description".

[91]	3GPP TS 29.674: "Interface between the UE radio Capability Management Function (UCMF) and the Mobility Management Entity (MME); Stage 3".

[92]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[93]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[94]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[95]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[96]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

* * * * Start of 2nd Change * * * *

4.13.9	Support of Store and Forward Satellite Operation

4.13.9.1	General

The Store and Forward Satellite Operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, some network functionality needs to be deployed on the satellite payload.

Example deployments are described in Annex O.

Figure 4.13.9.1-1: Store and Forward (S&F) Satellite operation

NOTE 1:	In the Figure 4.13.9.1-1, the NTN GW belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network. This is depicted in Figure 4.13.9.1.1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g. location L1 in Figure 4.13.9.1.1) the UE sends the data to the satellite. Subsequently (e.g. location L2 in Figure 4.13.9.1.1), the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step and the process is repeated.

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	Some deployments cannot support all system features, for example they cannot support user plane establishment and user plane data transfer, when operating in S&F Mode in the split MME deployment described in Annex O.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served in S&F mode.

When an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

-	If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer, and/or optionally a S&F Monitoring List when accepting or rejecting a NAS procedure. The S&F Monitoring List is a list of satellites (identified by satellite ID(s)) supporting S&F operation for the same PLMN with which the UE may use to attempt to retry the NAS procedure, in the future. The Satellite IDs are based on the SIB information broadcast by eNB.

NOTE 3:	How the MME determines the S&F Wait Timer is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

The S&F Monitoring List sent to the UE during a NAS procedure can be updated by an MME when the UE performs a NAS procedure in the future.

If the S&F Monitoring List was provided to the UE,

- the UE may (re)attempt a NAS procedure on any of the satellites in the S&F Monitoring List in the current Tracking Area List;

- if the UE has received a S&F reject cause, the UE’s EMM state should remain unchanged.

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The MME may indicate to UE the Uplink S&F estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 4:	The Uplink S&F estimated Delivery time is the estimated/expected time required to deliver the data to the ground network element from the time the data is sent by the UE.

The MME may trigger a Update Location procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Update Location procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE is not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject a UE Attach request for S&F Mode.

NOTE 5:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE 6:	The absence of timestamp in the Update Location Request can occur e.g. when the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.10 of TS 23.682 [74].

* * * * End of Changes * * * *