3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e 	S2-2500610

E-meeting, 20 – 24 January 2025	(revision of S2-25xxxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Further clarification on Local offloading traffic handling at I-SMF

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract: this paper discusses the open issue related to the traffic local offloading handling at I-SMF.

1. Introduction

For Local Offloading traffic handling at I-SMF, there are two ENs in the existing TS23.502 clause 4.23.6.2 and TS23.548 clause 6.10.3.1. Also, how to configure the N4 rule at the UPF controlled by the I-SMF need some more clarification.

In this paper we try to resolve those 2 EN and clarify how the N4 rule is generated.

2. Discussion

Issue 1: Local offloading policy transferred to I-SMF

Editor's note:	It is FFS whether SMF transfers transparently Local Offloading Policy to I-SMF.

This EN indeed has been resolved at S2#166 meeting. It has been decided that the local offloading policy is transparently forwarded to the I-SMF. So this EN can be removed directly.

Proposal 1: Remove the EN on whether SMF transfers transparently Local Offloading Policy to I-SMF directly.

Issue 2: Usage of the AF traffic influence information at the I-SMF

Editor's note:	whether the AF traffic influence information is transferred to I-SMF is FFS.

In HR-SBO session, the AF traffic influence information is transferred to the V-SMF as defined in clause 6.1.6.2.73 of TS29.502. That information help V-SMF determine how to do the L-PSA selection. For the I-SMF based local offloading management it has also been agreed that the AF traffic influence information is sent to the I-SMF if it is available in the TR conclusion.

It is I-SMF that handle the DNS message and determines how to select UL-CL/L-PSA per the received information, e.g. EDI/local offloading policy. The AF traffic influence information indeed is also used to assist the UL-CL/L-PSA selection. It is more suitable to have all information for ULCL/L-PSA selection in one place to have a consistence decision and avoid any potential conflict decision. For example, if the AF traffic influence and DNS message handling are related to the same application flow, then the UL-CL/L-PSA selection can be done per DNS message handling and not need select two L-PSA in different DNAI.

In the existing ETSUN procedure, the DNAI of interest is sent to I-SMF per the AF traffic influence information and the DNAI supported by I-SMF. This is used for I-SMF to determine ULCL/L-PSA UPF selection. If the AF traffic influence information are sent to I-SMF, the I-SMF can directly determine whether ULCL/L-PSA need be inserted and in which place based on that information. Hence the DNAI of interest sent by SMF is not needed.

Proposal 2: SMF transfers the received AF traffic influence information to I-SMF.

Proposal 3: SMF does not need send the DNAI of interest to I-SMF.

Issue 3: How to configured UL-CL/L-PSA controlled by I-SMF

At S2#166 there are no consensus to let the I-SMF generates the N4 rule directly for the UPF controlled by the I-SMF. Hence the existing ETSUN mechanism, i.e. SMF generates the N4 rule, is assumed to be still used. However how the SMF is aware which traffic(s) are offloaded, i.e. how to fill the packet filter set(s) in the PDR, are still unclear.

In the ETSUN case the I-SMF notifies the selected DNAI to the SMF. Per the received AF traffic influence information the SMF determines which traffic(s) are offloading at the notified DNAI. So the SMF fills the related packet filter set as the offloaded traffic transferred at the DNAI is only per AF traffic influence information.

In the I-SMF based local offloading management case, the traffic to be offloaded can be the result of the DNS message handling, i.e. not from the AF traffic influence information. In this case the SMF is not aware which traffic is to be offloaded. If it is handled same as the ETSUN case, the SMF need per this DNAI information check all related information, e.g. EDI, AFTI. If we do like this, following  issue need be considered:

- Unnecessary filter Setting: The filter to be added may include the traffic which is not delivering but can be transferred via the same DNAI.

- EDI information maintenance: the SMF need subscribe and maintain the latest EDI information for the indicated DNN/S-NSSAI. This add burden to the SMF.

On the other hand more simple way is why not let the I-SMF provides the related SDF information of the traffic directly? That is simple and easy way.

Obsrevation 1: the SDF of traffic to be offloaded is better not provided by the SMF but be provided by the I-SMF.

To generate the N4 rule two options on how I-SMF provides the SDF information:

- Option 1:  when the I-SMF notifies the SMF of insertion of the UL-CL/L-PSA, I-SMF also provides the related SDF to SMF. Per the received SDF the SMF generates the related N4 rule.

- Option 2:  when the I-SMF notifies the SMF of insertion of the UL-CL/L-PSA, no additional information is provided. Different comparing to ETSUN case is that the in the PDR, the SMF does not fill the related packet filter set, i.e. the SMF generates the related N4 rule with the blank packet filter set in the PDR. Before the I-SMF configures the received N4 rule at the L-PSA, I-SMF fills the related packet filter set per the traffic to be offloaded. This is similar as TEID filling by the I-SMF, e.g. AN part tunnel TEID.

Comparing above two options, the option 2 has less change to the existing specification and more flexible to let I-SMF fill the packet filter set per traffic offloaded. Option 1 can give concrete information on the relation between the SDF and DNAI. Hence two options are listed for further discussion.

Proposal 4: Two options are possible on packet filter provided by I-SMF:

Option 1:  	The I-SMF notifies the SDF (Service Data Flow Filter) of the offloaded traffic to the SMF. Per the received SDF, the SMF generates related N4 information and sends to the I-SMF for configuration of UL CL and L-PSA(s)

Option  2:  	When I-SMF notifies the SMF of insertion of the UL-CL/L-PSA,  SMF does not fill the related packet filter set in the generated N4 rule for L-PSA controlled by the I-SMF. Those packet filter sets within the PDR are filled by the I-SMF per traffic to be offloaded directly.

3. Proposal

It is proposed to discuss and agree above Proposal. The related accompanied paper (S2-2500684/2500685) are also proposed on the concrete changes.