3GPP TSG-SA WG2#166AHE	S2-2500713

E-meeting, 20-24 January 2025	(was)

Source:	Samsung

Title:	Topic #10, Conclusions on AIoT KI#2 for Subscription Management

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes to update conclusions on Subscription Management.

1 Discussion

During SA2#166 meeting, interim conclusions on Subscription Management. In the conclusions, there are editor’s notes which need to be resolved for subscription management.

Editor’s note:	Where to store the AIoT device subscription is FFS.

Editor’s note:	Items within the subscription data for 3rd party AF need to be figured out later.

In order to make the AIoT devices be ready for the service by the operator, the subscription data for the AIoT device need to be stored in UDM of the operator’s network so that the data can be referred to or managed during the service operation. A 3rd Party Service Provider(AF) may use on-line group subscription management service via NEF.

Items within the subscription data for 3rd party AF may include the following:

-	AF ID: ID of 3rd Party Service Provider (AF)

-	List of Usage ID(s):	ID of TASK of Ambient IoTs. It may have NAI format (host part + realm part)

-	List of Ambient IoT ID(s):	List of each Ambient IoT Device ID(s) which take part in common TASK identified by Usage ID

-	Authentication/Authorization information for AF:	Information for EAP-AKA between 5GC and AF

-	List of Credentials of Ambient IoTs: Credential information of Ambient IoTs

-	TASK Information:	Description of TASK identified by Usage ID

2 Proposal

It is proposed to capture the following changes to TR 23.700-13.

START OF CHANGES

8.2.2	Subscription Management

The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network.

Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network:

-	It is used by the AIOTF to check whether the device is subscribed.

NOTE 1:	Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier.

-	It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data.

-	It contains at least a permanent Ambient IoT Device ID, etc.

Editor’s note:	SA WG2 will align security related materials in subscription data with SA WG3 decision later.

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the Subscription data or related data of an Ambient IoT Device is.

For an operator to make the AIoT devices to be ready for the service, the operator needs to store the subscription data for the AIoT device in UDM of the operator’s network so that the data can be referred to or managed during the service operation. A 3rd Party Service Provider(AF) may use on-line group subscription provisioning/management service via NEF.

Subscription data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service.

-	it is stored at UDM within 5G network.

-	Items within the subscription data for 3rd party AF may include the following:

-	AF ID: ID of 3rd Party Service Provider (AF)

-	List of Usage ID(s):	ID of TASK of Ambient IoTs. It may have NAI format (host part + realm part)

-	List of Ambient IoT ID(s):	List of each Ambient IoT Device ID(s) which take part in common TASK identified by Usage ID

-	Authentication/Authorization information for AF:	Information for EAP-AKA between 5GC and AF

-	List of Credentials of Ambient IoTs: Credential information of Ambient IoTs

-	TASK Information:	Description of TASK identified by Usage ID

UE Subscription data is enhanced to include an indication that whether the UE is allowed to operate as a UE reader.

END OF CHANGES