3GPP TSG-SA WG2#166AHE	S2-2500714

E-meeting, 20-24 January 2025	(was)

Source:	Samsung

Title:	Topic #8, Conclusions on AIoT KI#3 for enabling temporarily disabled AIoT devices

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes conclusions on enabling temporarily disabled AIoT devices.

1 Discussion

During SA2#166 meeting, interim conclusions on AIoT Services are agreed. In the conclusions, there is an editor’s note which needs to be resolved on how to disable a device temporarily and how to enable the device.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

In order to support enabling temporarily disabled AIoT devices, the devices need to receive signals from the network for the enabling. However, receiving all the signals may burden the temporarily disabled devices, so we need idle period information on which time slot the temporarily disabled device can receive signals or not. Also, a temporarily disabled device may return a enablement code to the network when it receives a command to disable the device temporarily. The network keeps the code and may add it to the message when the network sends a command to enable the disabled device. When the temporarily disabled device receives a command to enable it, the device compares the enablement code from the command message and the code it has. When they are matched, the device enables its capability to transmit RF.

2 Proposal

It is proposed to capture the following changes to TR 23.700-13.

START OF CHANGES

8.3	Interim Conclusion on Key Issue #3

8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

Editor's note:	Additional conclusions are FFS.

8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device. In order to support enabling temporarily disabled AIoT devices, the devices need to receive signals from the network for the enabling. However, receiving all the signals may burden the temporarily disabled devices, so we need idle period information on which time slot the temporarily disabled device can receive signals or not. Also, a temporarily disabled device may return a enablement code to the network when it receives a command to disable the device temporarily. The network keeps the code and may add it to the message when the network sends a command to enable the disabled device. When the temporarily disabled device receives a command to enable it, the device compares the enablement code from the command message and the code it has. When they are matched, the device enables its capability to transmit RF.

NOTE:	The security aspect of Enable and Disable is to be concluded by SA WG3.

8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

-	New NEF services (e.g. Nnef_AIoT) to support the AIoT services will be defined.

Editor's note:	NEF event exposure aspect is FFS.

8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines A-IoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the A-IoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

Editor’s note:	If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, a TASK ID etc. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

8.3.5	Information provided by the AF

The AF provides one or some of the following information:

-	Information about AIoT services:

-	Service type indicating e.g. inventory or command (e.g. read or write).

-	Information to be used for AIoT reader selection:

-	At least one of the UE reader ID or the target area information.

-	Optionally, Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

-	Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]:

-	Approximate number of AIoT devices.

-	Approximate D2R message size.

NOTE:	Information to be used for resource allocation will be aligned with RAN WGs during the normative phase.

END OF CHANGES