3GPP TSG-WG SA2 Meeting #166 -Ad Hoc-e	S2-2500762

# 20-24 Jan 2025, Emeeting		(revision of S2-2412248)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0513 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on multi-hop U2N Relay Discovery with Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution is a resubmition of not handled S2-2412248 from last meeting. It is not clear on how to update the Accumulated QoS for PC5 link during multi-hop U2N Relay Discovery with Model A, when a 5G ProSe Intermedidate UE-to-Network Relay can not determine the QoS between itself and previous hop, e.g. if the 5G ProSe Intermedidate UE-to-Network Relay dose not have a existing PC5 link with other Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that when a 5G ProSe Intermedidate UE-to-Network Relay can not determine the QoS of its hop, it removes the Accumulated QoS for PC5 link in the Discovery Announcement to next hop. - Fix typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Accumulated QoS for PC5 link may be sent to next hops during Discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.2.5.2	Procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.3.2.5.2-1 presents the procedures for 5G ProSe multi-hop UE-to-Network Relay Discovery using Model A discovery.

Figure 6.3.2.5.2-1: 5G ProSe multi-hop UE-to-Network Relay discovery procedure with Model A

0.	The 5G ProSe Remote UE, the 5G ProSe Intermedidate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay are (pre-)configured or provisioned with the related information to support multi-hop UE-to-Network Relay discovery, as specified in clause 5.1.4. If the 5G ProSe Remote UE, 5G ProSe Intermedidate UE-to-Network Relay and 5G ProSe UE-to-Network Relay belong to different PLMNs, their HPLMNs ensure that consistent configuraitons are provided.

1.	If a 5G ProSe UE-to-Network Relay is successfully registered and authorized by the 5GS and has the configurations to operate multi-hop UE-to-Network Relay in the current region, it can start the discovery procedure. Otherwise, it shall not perfrom the 5G ProSe multi-hop UE-to-Network Relay discovery, i.e. not sending any announcement message.

2.	The 5G ProSe UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message. To support multi-hop UE-to-Network Relay discovery, the 5G ProSe UE-to-Network Relay sets the Hop-Count to 1, sets the Announcer Info to its own User Info ID, and sets the RSC according to the configuration as in clause 5.1.4.

3.	The 5G ProSe Intermedidate UE-to-Network Relay creates a corresponding discovery entry based on the received 5G ProSe UE-to-Network Relay Discovery Announcement message, if the discovery message meets the AS layer criteria, and the Hop-Count is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

The discovery entry includes the following information:	RSC, (optional) Root Relay Info, Announcer Info, Hop-Count, (optional) Accumulated QoS for PC5 link, (optional) NCGI, (optional) RRC Container.

The Announcer Info is set to the Announcer Info of the received message. The Root Relay Info is set to the Root Relay Info of the reived message. The Hop-count is set to the value of the received message. The Accumulated QoS for PC5 link, if present in the received discovery message, will be updated based on the QoS of the PC5 link between the 5G ProSe Intermedidate UE-to-Network Relay and the sender of the message, if the 5G ProSe Intermedidate UE-to-Network Relay can determine the QoS between 5G ProSe Intermedidate UE-to-Network Relay and the sender of the message, e.g. there exists a PC5 connection between them. Otherwise the 5G ProSe Intermedidate UE-to-Network Relay removes the Accumulated QoS for PC5 link in discovery entry.

If an discovery entry already exist, i.e. with the same RSC and (optional) Root Relay Info, and if the Hop-Count of the received message is less than the Hop-Count stored in the discovery entry, the 5G ProSe Intermedidate UE-to-Network Relay updates the discovery entry accordingly. The Announcer Info and Hop-Count are set to the Annoncer Info and Hop-Count of the newly received discovery message.

NOTE:	Based on implementation, a 5G ProSe Intermedidate UE-to-Network Relay may decide to update the discovery entry based on the Accumulated QoS for PC5 link instead of the Hop-Count, or a combination of the two information elements.

4a.	5G ProSe Intermedidate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. The Hop-Count value will be incremented by 1, and the Announcer Info is set to the User Info of the 5G ProSe Intermedidate UE-to-Network Relay.

When received by the 5G ProSe Remote UE, the information in the 5G ProSe UE-to-Network Relay Discovery Announcement message will be used for relay (re)selection, if the Hop-Count is less or equal to the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message. The 5G ProSe Remote UE can also take the Accumulated QoS for PC5 link into account. For relay reselection, the Root Relay Info may be used to prioritize a relay offering the connection to the same 5G ProSe UE-to-Network Relay.

4b.	5G ProSe Intermedidate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. This message may reach another 5G ProSe Intermedidate UE-to-Network Relay.

5.	The 5G ProSe Intermedidate UE-to-Network Relay drops the received message if the Hop-Count is not less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

6.	The 5G ProSe Intermedidate UE-to-Network Relay sets a locally configured timer for each of the discovery entry based on implementation. If the 5G ProSe Intermedidate UE-to-Network Relay does not have an active PC5 link with the relay identfied by the Announcer Info, and it does not receive any new announcement message from that relay, the 5G ProSe Intermedidate UE-to-Network Relay removes the corresponding discovery entry and stops sending related 5G ProSe UE-to-Network Relay Discovery Announcement message.

Based on configuration, the 5G ProSe Intermedidate UE-to-Network Relay may need to establish a Layer-2 link with the relay identified by the Announcer Info in the discovery entry before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.

# * * * * End of changes * * * *