3GPP TSG SA WG2 Meeting #166AH-e	S2-2500772

# 20 - 24 January, 2025, Elbonia				    									(Revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1563 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on how DCSF fetch DC AS URL if not pre-configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-1-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution tries to clarify how the DCSF fetch DC AS URL for downloading Avatar List ID if the URL is not pre-configured in DCSF and solve the remaining EN accordingly. It is proposed to clarify that the DC AS URL can be included in DCSF service specific data and is derived by DCSF when needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the DCSF may derive DC AS URL from DCSF service specific data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.1, AC.11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change****

## AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The Avatar Representations may be downloaded together. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR, and transferred from the DC AS to the DCSF, and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR, and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, URLs, the associated data channel applications).

Editor's note:	Whether the personal asset information (e.g. identities of accessories, garments) can be provided with Avatar IDs and Avatar Representation is FFS.

When the Avatar ID List is downloaded through bootstrap data channel, a DC AS URL is used by DCSF to connect the corresponding DC AS to fetch the Avatar ID List. The DC AS URL(s) may be pre-configured in the DCSF, or derived by DCSF from DCSF service specific data. When the downloading of Avatar ID List is requested through bootstrap data channel, the DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar IDs may be provided from the BAR to MF, and it may be transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

Editor's note:	Whether the Avatar Representation can be downloaded through bootstrap data channel is FFS.

**** Second Change****

### AC.11.3.1	Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1: Procedures of Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1 depicts a typical call flow procedure of avatar ID list download procedure through bootstrap data channel. The main steps in the call flow are as follows:

NOTE:	The avatar ID list can be downloaded via application data channel or pre-configured in the UE locally as described in clause AC.11.1.

1.	IMS session and bootstrap data channel have been established.

2.	The UE1 downloads application list through established bootstrap data channel.

3.	The UE1 selects and downloads avatar communication application through established bootstrap data channel.

4.	The DCSF checks the requested DC application is avatar communication application, and select the corresponding DC AS based on local configuration. The DCSF gets DC AS URL by local configuration or derives it from service specific data.

5-6.	The DCSF downloads avatar communication from DCAR.

7. The DCSF requests DC AS to get UE1's avatar ID list using DC AS URL.

8.	The DC AS get UE1's avatar ID list from BAR.

9-10.	The BAR returns UE1's avatar ID list to the DC AS, the DC AS returns it to the DCSF.

11.	The DCSF sends the avatar ID list together with the avatar application to the UE1.

**** End of Changes ****