3GPP TSG SA WG2 #166-Ad-Hoc-e	            S2-2500794

20 - 24 January, 2025, Electronic meeting 	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1565 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: EN resolutions for Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to SP-241512 (approved in SA#106), the following ENs need to be resolved: In WT8 Enhancements of IMS architecture and procedures to support IMS based Avatar communication - Usage of URL of DC AS for downloading the avatar ID list is to be specified; - How the MF interacts with BAR for direclty downloading avatar representation is FFS; - Whether and how to download Avatar representation via bootstrap DC is FFS. Also, the step 5 to 8 of Figure AC.11.3.2.2-1 about how the receiving UE downloads the Avatar Representation of the user of the sending UE should be defined after align with SA3 decision. This is described in the following EN of clause AC.11.3: Editor's note: Whether and which user identity(ies) should be used by the user of the sending UE (UE#1) and/or the receiving UE (UE#2) for downloading of the Avatar Representations in receiving UE rendering mode will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - How the DCSF gets the DC AS URL for Avatar ID List download when it is requested through bootstrap data channel is clarified. The DCSF gets from the DCSF service specific data or from its local configuration. - It is clarified that MF receives a Replacement HTTP URL representing the Avatar ID List from DCSF, and uses this to replace the root URL of the Avatar ID when the Avatar Representation is requested through data channel. This makes the Avatar Representation request message to be transferred from MF to DC AS via MDC2 interface. By this way, the MF can directly download the Avatar Representation from BAR if the Replacement HTTP URL points BAR. Also, the MF can download the Avatar Representation when it is requested through bootstrap data channel as well as application data channel. - The step 5 to 8 of Figure AC.11.3.2.2-1 are revised according to the EN. - Detail procedures are clarified in overal. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.3.2, AA.2.5.2.2, AC.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification: Description of additional media specification information needed for data channel media stream from application layer, which includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies how the media stream should be processed.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Replacement HTTP URL per stream ID allocated by the application layer representing the Avatar ID List for the IMS subscriber via the MDC2 interface.

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation to perform network rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE:	DC Media Specification and AR Media Specification are optional depending on the scenario and media type.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new media) and includes the media type according to SDP. Each entry may optionally contain Avatar Media Specification, DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media. The Avatar Media Specification contains media processing instruction for MF on how to perform network rendering for avatar communication.

# * * * * Second change * * * *

AA.2.5.2.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The MediaID value is set by the consumer.

NOTE 1:	Different Termination IDs associated with the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Replacement HTTP URL per stream ID allocated by the application layer representing the Avatar ID List for the IMS subscriber via the MDC2 interface.

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation to perform network rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	Avatar Media Specification are optional depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID, and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

# * * * * Third change * * * *

AC.11	Support of Avatar Communication

AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The Avatar Representations may be downloaded together. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR, and transferred from the DC AS to the DCSF, and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR, and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, URLs, the associated data channel applications).

Editor's note:	Whether the personal asset information (e.g. identities of accessories, garments) can be provided with Avatar IDs and Avatar Representation is FFS.

When the downloading of a data channel application is requested through bootstrap data channel, the DCSF checks DC AS address when it determines that the requested data channel application is for avatar communication. The DC AS address for certain data channel application for avatar communication may be configured to DCSF, or retrieved from DCSF service specific data in HSS/UDM. The DC AS may provide its address to HSS/UDM by using External Parameter Provisioning as described in clause 4.15.6 of TS 23.502 [94]. The DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

During the data channel application and the Avatar ID List downloading procedure, the DCSF provides a Replacement HTTP URL representing the Avatar ID List to IMS AS and the IMS AS transfers it to MF, so that the MF stores this and applies this when the MF is requested to retrieve Avatar Representations. When the MF receives a request of downloading an Avatar Representation with an Avatar ID and the user identity through an established data channel, the MF replaces the root URL of the Avatar ID with the Replacement HTTP URL representing the Avatar ID List. Then the request is transferred to the media point of DC AS via MDC2 interface.

NOTE:	DCSF may get the Replacement HTTP URL representing the Avatar ID List from DCSF service specific data, or from DC AS, or from local configuration.

NOTE:	If the Replacement HTTP URL representing the Avatar ID List points directly to the BAR, the MF can download the Avatar Representation directly from BAR. The MF does not need to know whether it points to DC AS or BAR.

The UE may exchange an Avatar ID and/or an Avatar Representation with the IMS DC network through bootstrap data channel (e.g. if there is only one data channel application to be used for avatar communication and then the UE may decid to download Avatar Representations before ADC setup for faster session setup, if Avatar ID or Avatar Presentation needs to be added/modified/deleted from BAR) and/or through application data channel.

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar IDs may be provided from the BAR to MF, and it may be transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

AC.11.2	Architecture

Figure AC.11.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Representation, its Avatar ID, and the associated information as described in clause AC.11.1. One or more Avatar Representation(s), is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR provides Avatar Representation and the associated information to the MF directly or after URL redirection from the DC AS.

DCSF:

-	If the Avatar ID List is downloaded through bootstrap data channel, the DCSF may fetch the Avatar ID List from the DC AS, and send to MF via MDC1.

MF:

-	The MF supports the UE downloading the Avatar ID List and associated information through bootstrap data channel or application data channel.

-	The MF supports the UE downloading the Avatar Representation through bootstrap data channel or application data channel.

-	The MF may retrieve the Avatar Representation and the associated information from BAR via MDC2.

-	The MF may support Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS may provide the DC AS URL(s) to DCSF via DC3/N33 or DC4 for downloading of Avatar ID List through bootstrap data channel.

-	The DC AS retrieves the Avatar ID List from BAR(s) via MDC2.

-	The DC AS sends the Avatar ID List either to DCSF via MDC3 for downloading through bootstrap data channel, or to MF via MDC2 for downloading through application data channel.

-	The DC AS may retrieve the Avatar Representation and the associated information from BAR and sends to MF via MDC2.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

AC.11.3	Procedure

Editor's note:	Which network entity will authenticate the UE will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision.

Editor's note:	Whether and which user identity(ies) should be used by the user of the sending UE (UE#1) and/or the receiving UE (UE#2) for downloading of the Avatar Representations in receiving UE rendering mode will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision.

AC.11.3.1	Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1: Procedures of Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1 depicts a typical call flow procedure of avatar ID list download procedure through bootstrap data channel. The main steps in the call flow are as follows:

NOTE:	The avatar ID list can be downloaded via application data channel or pre-configured in the UE locally as described in clause AC.11.1.

1.	IMS session and bootstrap data channel have been established by appling step 1 to step 20 of the procedure specified in clause AC.7.1.

2.	As specified in step 21-2 of clause AC.7.1, the UE1 downloads application list through the established bootstrap data channel.

3.	As specified in step 21-3 of clause AC.7.1, the UE1 sends an application request message to MF to request a data channel application, which is the data channel application for avatar communication, through the established bootstrap data channel. The MF forwards the message to the DCSF.

4. The DCSF retrieves the requested data channel application for avatar communication from DCAR.

5.	The DCSF checks whether the requested data channel application is for avatar communication, and if it is for avatar communication then checks the corresponding DC AS address (e.g. from DCSF service specific data, or local configuration).

6. The DCSF requests to DC AS to get the Avatar ID List for the user of UE1 by using DC AS URL identified in step 5.

7. The DC AS requests to BAR to get the Avatar ID List for the user of UE1.

8.	The DC AS receives the Avatar ID List for the user of UE1 from BAR.

9. The DCSF receives the Avatar ID List for the user of UE1 from the DC AS.

10. The DCSF provides Replacement HTTP URL representing the received Avatar ID List to the IMS AS by invoking Nimsas_MediaControl service, and the IMS AS provides it to the MF by invoking Nmf_MediaResourceManagement service. The MF stores the Replacement HTTP URL representing the Avatar ID List and applies this when the MF is requested to retrieve Avatar Representation(s).

NOTE:	DCSF may get the Replacement HTTP URL representing the Avatar ID List from DCSF service specific data in step 5, or from DC AS in step 9, or from local configuration.

11.	The DCSF sends the Avatar ID List for the user of UE1 together with the data channel application for avatar communication to the UE1.

NOTE:	If UE decides to request an Avatar Representation before an application data channel (e.g. for faster session setup), the UE may send an Avatar Representation request to the MF through bootstrap data channel, then the MF replaces the root URL with the Replacement HTTP URL set in step 10 so that the request message is transferred to the corresponding DC AS.

AC.11.3.2	UE centric procedure

The UE centric mode includes two scenarios:

-	Sending UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should download the avatar representation from BAR, if the Avatar representation is not locally available. The sending UE performs avatar animation and sends the animated video stream to the peer UE.

-	Receiving UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should sends the avatar ID to the peer UE. The peer UE downloads the avatar representation from BAR based on the received avatar ID, performs the avatar animation, and displays the animated video stream.

NOTE 1:	It is assumed that both UEs support IMS data channel and at least one UE support avatar animation capability among the two UEs in a avatar communication session.

NOTE 2:	UEs could transmit additional data using application data channel and/or audio/video to each other to support the UE centric rendering. The usage of those data is out of scope of this specification.

AC.11.3.2.1	Sending UE centric procedure

Figure AC.11.3.2.1-1: Procedures of Sending UE centric IMS Avatar communication

Figure AC.11.3.2.1-1 depicts a typical call flow procedure of sending UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads the data channel application for avatar communication and the Avatar ID List as described in clause AC.11.3.1. The UE1 executes the data channel application for avatar communication, and determines whether the Avatar Represenation identified by the Avatar ID which selected by the user needs to be downloaded from the IMS DC network. If the Avatar Representation is locally available on the UE1, then step 2-6 may be skipped.

2.	If the Avatar Representation needs to be downloaded, the UE1 may decide to establish a P2A application data channel between UE1 and DC AS for avatar representation transmission. The P2A application data channel(s) are established between UE1 and DC AS as described in clause AC.7.2.2.

3.	The UE1 sends a request message to MF to request an Avatar Representation by providing the Avatar ID and the user identity of the UE1 (e.g. IMPU, IMSI, or MSISDN) through the application data channel. The MF replaces the root URL of the Avatar ID with Replacement HTTP URL representing the Avatar ID List which is stored during Avatar ID List downloading procedure as described in step 10 of clause AC.11.3.1. By this replacement, the Avatar Representation request message is transferred to the media point of DC AS via MDC2 interface.

4. The DC AS authenticates and authorizes the request from UE1, and requests the Avatar Representation from BAR by using Avatar ID.

5. The BAR responds to the DC AS with the Avatar Representation identified by the requested Avatar ID.

6.	The DC AS responds to MF with the Avatar Representation, and then MF sends it to UE1 through application data channel.

7.	UE1 performs avatar animation.

8.	UE1 transmits a video stream of the animated Avatar Representation over RTP to UE2.

AC.11.3.2.2	Receiving UE centric procedure

Figure AC.11.3.2.2-1: Procedures of Receiving UE centric IMS Avatar communication

Figure AC.11.3.2.2-1 depicts a typical call flow procedure of receiving UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads the data channel application for avatar communication and the Avatar ID List as described in clause AC.11.3.1. The UE1 executes the data channel application for avatar communication, and determines whether the Avatar Represenation identified by the Avatar ID which selected by the user needs to be downloaded from the IMS DC network.

2.	If the downloaded data channel application for avatar communication supports receiving UE centric mode, the UE1 may decide to establish a P2A2P application data channel between UE1 and DC AS, and DC AS and UE2.

3.	The UE1 decides to use receiving UE centeric mode based on its status such as power, signal, computing power, internal storage, etc., to request to peer UE to perform avatar animation.

4.	The UE1 performs avatar animation negotiation with the DC AS and UE2 through the established P2A2P application data channel. The negotiation includes usage of the Avatar ID of the user of UE1 for the UE2,the avatar animation capabilities (e.g. 2D/3D capturing, feature extraction, 2D/3D rendering, transcoding from/to text or gesture) and avatar animation data formats supported by both UE1 and UE2.

Editor's note:	The negotiation procedure needs to be further defined in SA WG4, and cooperation with SA WG4 is needed.

8.	The DC AS responses with UE1's avatar representation data to the MF, and the MF sends it back to UE2.

5. The Avatar Representation identified by the Avatar ID of the user of UE1 which is decided to use in step 4 is downloaded to UE2.

Editor's note:	Whether the Avatar ID of the user of UE1 is provided to the UE2 and the corresponding procedure of downaloding Avatar Representation of the user of UE1 to the UE2 will be aligned with by SA WG3 decision.

6.	The UE1 sends the avatar animation data (e.g. facial expression) to UE2 through the established application data channel and/or audio/video through RTP between UE1 and UE2, based on the negotiation perfomed in step 4.

10.	The UE2 performs avatar animation with the Avatar Represenation of the user of UE1 and the avatar animation data received from the UE1.

AC.11.3.3	Network centric procedure

Figure AC.11.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.11.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

Editor's note:	The detail of network rendering need to be further specified.

Editor's note:	Alignment with SA WG4 may be needed.

1.	IMS session and bootstrap data channel have been established. UE1 downloads the data channel application for avatar communication and the Avatar ID List as described in clause AC.11.3.1. The UE1 executes the data channel application for avatar communication, and determines whether the Avatar Represenation identified by the Avatar ID which selected by the user needs to be downloaded from the IMS DC network.

2.	If the downloaded data channel application for avatar communication supports network centric mode, the UE1 may decide to establish a P2A2P application data channel UE1 and DC AS, and DC AS and UE2.

3.	The UE1 decides to use network centric mode based on its status such as power, signal, computing power, internal storage, etc., to request to network to perform avatar animation..

4.	The avatar animation negotiation between the UE1 and the DC AS, and between the DC AS and the UE2 is performed thorugh the established P2A2P application data channel. The negotiation includes the avatar animation capabilities (e.g. 2D/3D capturing, feature extraction, 2D/3D rendering, transcoding from/to text or gesture) and avatar animation data formats supported by UE1, MF, DC AS, and UE2.

Based on the avatar animation negotiation result, the UE1 may use a separate P2A2P application data channel, which is used for avatar animation data transmissition between the UE1 (and/or UE2) and the IMS DC network. The UE1 initiates the separate P2A2P application data channel(s) if newly needed. With the avatar media processing specification received from the DC AS, the DCSF provides it to the IMS AS by invoking Nimsas_MediaControl service, and the IMS AS provides it to the MF by invoking Nmf_MediaResourceManagement service.

NOTE:	DCSF may retrieve the avatar media processing specification from DC AS using the DC AS URL used for Avatar ID retrieval as described in clause AC.11.3, or the separate URL indicated by the DCSF service specific data or local configuration. The procedure of avatar media processing specification retrieval is not specified in this specification.

Based on the avatar animation negotiation result, the media re-negotiation between UE1, MF, and UE2 in terms of achoring the audio/video streams to the MF.

Editor's note:	The avatar animation negotiation procedure and parameters need to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5. The UE1 sends a request message to MF to download the Avatar Representation by providing the Avatar ID and the user identity of the UE1 (e.g. IMPU, IMSI, or MSISDN) through the application data channel. The MF replaces the root URL of the Avatar ID with Replacement HTTP URL representing the Avatar ID List which is stored during Avatar ID List downloading procedure as described in step 10 of clause AC.11.3.1. By this replacement, the Avatar Representation request message is transferred to the media point of DC AS via MDC2 interface.

6.	The MF downloads the Avatar Representation of the user of UE1 thorugh DC AS. The MF maintains the downloaded Avatar Representation in the context.

7.	The UE1sends the avatar animation data (e.g. facial expression, FOV) to the MF through the established P2A2P  application data channel stream and/or the audio/video streams based on the avatar animation negotiation result in step 4.

8. [Conditional] The UE2 may send the avatar animation data (e.g. FOV) to the MF through the established P2A2P application data channel and/or audio/video streams through established RTP channels based on the avatar animation negotiation result in step 4.

9.	The MF performs avatar animation with the Avatar Representation of the user of UE1 and the avatar animation data which is received from application data channel stream and/or audio/video streams. The MF performs appropriate type of media processing (e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar) according the avatar animation formats, decided during avatar animation negotiation in step 4.

10.	The MF transmits the video stream of the animated Avatar Representation of the user of UE1 over RTP to UE2.

11. The MF transmits the video stream of the animated Avatar Representation of the user of UE1 over RTP to UE1 if required.

After successful setup of a network centric avatar communication, the UE or the network may initiate avatar animation re-negotiation based on the status of the UE/network. In this case, the UE may start from the step 3 of the above procedures.

The above procedures also apply for the transition from audio/video communication between two users to avatar communication. In an ongoing audio/video communication, one of UEs decides to switch the communication to avatar communication, the UE interacts with the IMS Core and peer UE to establish (and/or add) the bootstrap data channel and/or application data channel for the avatar communication as the procedures described in clause AC.11.3.1 and the above procedures. After successful setup of data channels for avatar communications, the UEs start to exchange the avatar animation data and the animated Avatar Representation through data channels and/or audio/video channel, and stop to exchange of the original video media between two UEs.

Editor's note:	The detail of transition from avatar communication to audio/video communication, and avatar animiation re-negotiation procedures are to be further specified.

# * * * * End of changes * * * *