3GPP TSG-WG SA2 Meeting #166AH	S2-2500815

# Elbonia, 20th – 24th Jan, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6012 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the subscription aspects for network energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For KI#2 of EnergySys (TR23.700-66), in SA2#166 meeting, it is agreed that the AMF forwards the Energy Saving indicator to the NG-RAN and whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS. This document is proposed to resolve this EN. In the current specification, there is no principle described for the handling of UEs with this indicator, in particular, when and how often RAN energy saving shall apply. Without such principle, the AMF forwards the Energy Saving indicator for all these UEs to NG-RAN and NG-RAN may often apply energy saving actions for these UEs. This can easily lead to situations where some UEs are consistently impacted and thus unequally treated compared to other UEs. The document proposes a conditional forwarding of the Energy Saving indicator of a UEbased on EIF control for the two aspects: - Whether RAN energy saving actions shall be enabled? - Whether the specific UE shall participate in RAN energy saving? If EIF determines that RAN energy saving is enabled and that certain UEs can participate, the EIF forwards the Energy Saving indicator for those UEs to the AMF together with an associated validity period. The EIF may forward it to the AMF directly or via PCF if AM Policy control is used by the operator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to resolve EN related to the energy saving indicator of the UE in the Subscription information aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete principle of the subscription aspects related to the Energy Saving indicator of UE for network energy saving. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The EIF receives the Energy Saving indicator in the UE Subscription data from the UDM.

The EIF determines whether energy saving operations are enabled (or not) and which UEs, from those with Energy Saving indicator in their subscription data, can participate and for how long.

NOTE 1:	The EIF decision about enabling energy saving operations in a certain area or the whole network is up to operator configuration.

NOTE 2:	The EIF decision about the participation of a UE in energy saving operations in the NG-RAN is up to operator configuration. The EIF could keep track of which UE participated in energy saving operations in the past and apply certain strategies to avoid that the some UEs are frequently participating while other UEs rarely do.

If the EIF decides to enable energy saving operations and has selected one or more UEs, the EIF forwards the Energy Saving indicator for those UEs to their responsible AMFs and PCFs together with a validity period. The validity period  defines the period of time for which energy saving operations is apllied (i.e. the AMF send the indication to the new NG-RAN to which the UE is connected due to handover and the PCF applies energy saving policies).

If the AMF receives an Energy Saving indicator for a UE together with a validity period, the AMF forwards the Energy Saving indicator to the NG-RAN that UE is connected to until the end of the duration is reached.

If the NG-RAN receives an Energy Saving indicator for a UE, the NG-RAN can apply energy saving operations for that UE.

If the PCF receives an Energy Saving indicator for a UE together with a validity period, the PCF can apply energy saving policies for that UE.

NOTE 3:	The energy saving policies in the PCF (e.g. traffic restrictions, RFSP index) are up to operator configuration.

Editor's note:	The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS.

Editor's note:	Whether the Energy Saving indicator is also stored in the UDR is FFS.

# * * * * End of changes * * * *