3GPP TSG-SA2 Meeting #166 Ad Hoc-e	S2-2500821

# Online, 20th Jan 2024 – 24th Jan 2025				                             (revision of S2-25xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1567 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedure of Avatar Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on last SA2 #166 meeting discussion, KI#8 avatar communication has some remaining issues to be solved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Update detail procedure of the avatar communication network centric call flow. 2.Remove the ENs that have been solved. 3.Some editorial correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.1, AC.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

## AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The Avatar Representations may be downloaded together. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR, and transferred from the DC AS to the DCSF, and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR, and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, URLs, the associated data channel applications).

For downloading of Avatar ID List through bootstrap data channel, the DC AS URL(s) may be pre-configured in the DCSF, or fetched by DCSF from data channel specific data in HSS. When the downloading of Avatar ID List is requested through bootstrap data channel, the DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar IDs may be provided from the BAR to MF, and it may be transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

Editor's note:	Whether the Avatar Representation can be downloaded through bootstrap data channel is FFS.

# * * * * Second change * * *

## AC.11.3	Procedure

Editor's note:	Which network entity will authenticate the UE will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision.

Editor's note:	Whether and which user identity(ies) should be used by the user of the sending UE (UE#1) and/or the receiving UE (UE#2) for downloading of the Avatar Representations in receiving UE rendering mode will be decided by SA WG3 and the procedure will be aligned with SA WG3 decision.

### AC.11.3.1	Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1: Procedures of Avatar ID List Download through Bootstrap Data Channel

Figure AC.11.3.1-1 depicts a typical call flow procedure of avatar ID list download procedure through bootstrap data channel. The main steps in the call flow are as follows:

NOTE:	The avatar ID list can be downloaded via application data channel or pre-configured in the UE locally as described in clause AC.11.1.

1.	IMS session and bootstrap data channel have been established.

2.	The UE1 downloads application list through established bootstrap data channel.

3.	The UE1 selects and downloads avatar communication application through established bootstrap data channel.

4.	The DCSF checks the requested DC application is avatar communication application, and selects the corresponding DC AS based on local configuration.

5-6.	The DCSF downloads avatar communication from DCAR.

7. The DCSF requests DC AS to get UE1's avatar ID list using DC AS URL.

8.	The DC AS get UE1's avatar ID list from BAR.

9-10.	The BAR returns UE1's avatar ID list to the DC AS, the DC AS returns it to the DCSF.

11.	The DCSF sends the avatar ID list together with the avatar communication application to the UE1.

### AC.11.3.2	UE centric procedure

The UE centric mode includes two scenarios:

-	Sending UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should download the avatar representation from BAR, if the Avatar representation is not locally available. The sending UE performs avatar animation and sends the animated video stream to the peer UE.

-	Receiving UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should send the avatar ID to the peer UE. The peer UE downloads the avatar representation from BAR based on the received avatar ID, performs the avatar animation, and displays the animated video stream.

NOTE 1:	It is assumed that both UEs support IMS data channel and at least one UE support avatar animation capability among the two UEs in an avatar communication session.

NOTE 2:	UEs could transmit additional data using application data channel and/or audio/video to each other to support the UE centric rendering. The usage of those data is out of scope of this specification.

#### AC.11.3.2.1	Sending UE centric procedure

Figure AC.11.3.2.1-1: Procedures of Sending UE centric IMS Avatar communication

Figure AC.11.3.2.1-1 depicts a typical call flow procedure of sending UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application. If the avatar representation is locally available on the UE1, then step 2-6 can be skipped.

2.	P2A application data channel(s) are established between UE1 and DC AS which is used for avatar representation transmission.

3.	UE1 downloads Avatar representation from DC AS using avatar ID and UE1's identity (e.g., IMSI or MSISDN) through the established application data channel.

4-5.	The DC AS should authenticate and authorize the UE1's request and download Avatar representation using avatar ID from the BAR, the BAR responses with avatar representation data.

Editor's note:	Whether MF download the avatar representation directly from BAR or via DC AS is FFS.

6.	The DC AS responses with the avatar representation data to MF, and then MF sends it back to UE1.

7.	UE1 performs avatar animation.

8.	UE1 transmits the animated video stream over RTP to UE2.

#### AC.11.3.2.2	Receiving UE centric procedure

Figure AC.11.3.2.2-1: Procedures of Receiving UE centric IMS Avatar communication

Figure AC.11.3.2.2-1 depicts a typical call flow procedure of receiving UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	P2A2P application data channel(s) are established between UE1/UE2 and DC AS.

3.	UE1 decides to request peer UE to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	UE1 performs avatar animation negotiation with the DC AS and UE2. The negotiation includes usage of the UE1's avatar ID and the animation data types (e.g. text and/or facial expression) supported by UE1.

Editor's note:	The negotiation procedure needs to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	UE2 downloads avatar representation from DC AS using the avatar ID received from UE1 through the established application data channel in step 2.

6-7.	The DC AS should authenticate and authorize the UE2 request and download avatar representation using UE1's avatar ID from the BAR, the BAR responses with UE1's avatar representation data. The DC AS should authorize the UE2 download request based on the negotiation result from step 4.

8.	The DC AS responses with UE1's avatar representation data to the MF, and the MF sends it back to UE2.

9.	UE1 may send metadata (e.g. facial expression) to UE2 through the established application data channel or audio/video through RTP channel between UE1 and UE2.

10.	UE2 performs avatar animation with the UE1's avatar representation based on the metadata received from UE1.

### AC.11.3.3	Network centric procedure

Figure AC.11.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.11.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

Editor's note:	Alignment with SA WG4 may be needed.

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	Application data channel(s) are established between UE1/UE2 and DC AS.

3.	UE1 decides to request network to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	The avatar animation negotiation is finished between the DC AS and UE1.

Editor's note:	The avatar animation negotiation procedure and parameters need to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	If the negotiation result is successful in step 4, the UE1 initiates new P2A2P application data channels, which are used for avatar data transmission between the UE1/UE2 and the network.

6-7.	During the P2A2P application data channel establishment procedure, the IMS AS notifies the DCSF via Nimsas_SessionEventControl_Notify of the media change request event. The DCSF checks the app id in the ADC establishment request is Avatar communication, then the DCSF fetch DC AS URL in DCSF service specific data.

8-9.	The DCSF retrieves the avatar media specification data from DC AS using DC AS URL.

10-11.	The DCSF instructs the MF via IMS AS how to establish the data channel and corresponding avatar media processing specification, via Nimsas_MediaControl_MediaInstruction.

12.	The IMS AS sends the MediaControl Instruction response to the DCSF.

13-17.	Steps 11-23 of clause AC.7.2.3 applies.

18.	The application data channel(s) are established between UE1, MF and UE2.

19.	The IMS AS initiates the media re-negotiation between UE1, UE2 and MF, which anchors UE1 and UE2's audio/video to MF.

20.	The MF downloads UE1's avatar representation using the URL in the avatar media specification data sent from DCSF in step 10.

21-22.	The UE1/UE2 may send avatar metadata (e.g. facial expression, FOV) through the P2A2P ADC established in step 5 and/or audio/video through established RTP channels in step 15.

23.	MF performs avatar animation based on the avatar metadata received from application data channel and/or audio/video. MF should support different media processing according the avatar type, e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar.

24-25.	MF sends the animated video stream to UE2, and sends the animated video stream to UE1 if required.

The above procedures also apply for the transition from audio/video communication between two users to avatar communication. In an ongoing audio/video communication, one of UEs decides to switch the communication to avatar mode, it will interact with the IMS Core and peer UE to establish application data channel for the avatar communication. Then two UEs will exchange avatar media; and the original video media between two UEs may be stopped.

# * * * * End of changes * * * *