3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500833

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0657 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on LMF requesting ML model for LMF-based AI/ML Positioning from the NWDAF containing MTLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - CR0597R13 is not been fully implemented, missing some descriptions on LMF discovering/selecting NWDAF and requesting ML model from NWDAF. This needs to be implemented, together with the following correction: - LMF provides Vendor ID instead of ML Model Interoperability indications to discover the NWDAF(s) from the NRF. Also the descriptions are modified to be more concise or clear. - LMF shall include an indication that support of ML Model training for LMF-based AI/ML Positioning is required in the request for NWDAF discovery. - LMF shall indicate LMF-based AI/ML Positioning when requesting ML Model(s) from NWDAF. Other parameters for ML model requesting are also to be clarified, to resolve the following EN in CR0597R13: Editor's Note: Whether and which additional parameters are considered when LMF requesting model positioning is FFS. - LMF may request multiple ML models, e.g. each with a different Accuracy level of Interest. The following EN can be removed: Editor's note: Whether multiple model identifiers can be provided is FFS. - The output information by the NWDAF for ML model provisioning also needs to be clarified. - Description of step 6 of Figure 6.22.53-1 needs to be modified as step 6 is not the same as step 5. - The description that "LMF may also trigger the ML model retraining in the training entity based on the result of model performance result" is obscure and needs to be clarified. - Editorial modifications are needed, e.g. changing “AI/ML model” to “ML model”, adding clause 5.18.0 for the hanging paragraphs under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implement the missing part of CR0597R13 with corrections and editorial modifications. - Add that LMF includes an indication that support of ML Model training for LMF-based AI/ML Positioning is required in the request for NWDAF discovery. - Clarify the input parameters provided by the LMF for ML model requesting, and remove the following ENs: Editor's Note: Whether and which additional parameters are considered when LMF requesting model positioning is FFS. Editor's note: Whether multiple model identifiers can be provided is FFS. - Clarify the output information provided by the NWDAF for ML model provisioning. - Update the ML model requesting procedure in clause 6.22.5 accordingly, and also modify description of step 6. - Clarify that the LMF or NWDAF containing MTLF may trigger ML model retraining based on model performance monitoring result. - Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the LMF requests ML model from NWDAF for LMF-based AI/ML positioning is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, 5.18.0 (new), 6.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE x:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training is described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

-	In the NF discovery request, the LMF includes an indication that support of ML Model training for LMF-based AI/ML Positioning is required, and in addition:

-	The LMF may include the positioning case information (e.g. UE assisted LMF-based AI/ML Positioning, NG-RAN assisted LMF-based AI/ML Positioning, or both).

-	The LMF may provide an Area of Interest and the Vendor ID.

-	The LMF may provide the ML Model accuracy checking capability to discover a NWDAF containing MTLF capable of ML model performance monitoring for LMF-based AI/ML Positioning.

NOTE y:	Other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37] such as Analytics ID, FL capability type and related time period, S-NSSAI or any roaming capabilities are not included by LMF.

Editor's Note:	The types of LMF-based AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

The LMF requests the NWDAF containing MTLF to provide a ML Model for LMF-based AI/ML Positioning as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The LMF provides the following input parameters in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request:

-	Indication of requesting ML Model for LMF-based AI/ML Positioning.

-	Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), 	Positioning case information, Target of ML Model Reporting, ML Model Target Period, Time when model is needed, Requested representative ratio, Inference Input Data information and ML Model Monitoring Information.

-	If vendor specific information is needed to allow running the ML Model, then the ML Model Interoperability Information.

-	If the LMF supports multiple AI/ML Models, indication of supporting for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_MLModelProvision_Subscribe) Notification Target Address (+ Notification Correlation ID).

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The NWDAF containing MTLF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning, and optionally, the following parameters:

-	ML Model Filter Information and/or Target of ML Model Reporting, if the ML Model provisioning request includes multiple ML Model Filter Information and/or Target of ML Model Reporting;

-	Indication of whether the ML Model identifier is updated (e.g. retained ML model).

-	Validity period, Spatial validity, ML Model representative ratio, Training Input Data Information, ML Model accuracy Information.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and available in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF.

Editor´s note:	Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

For ML model training, performance monitoring and UE positioning calculation using LMF-based AI/ML Positioning, the LMF checks with UDM the user consent status before collecting UE related data, see clause 6.22.3 and clause 6.22.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model. When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF or the NWDAF containing MTLF may also trigger the ML model retraining based on the model performance monitoring result.

* * * Next Change * * * *

6.22.5	Retrieval of trained ML Model from NWDAF for LMF-based AI/ML positioning

The LMF may retrieve a trained ML Model from the NWDAF, and perform UE positioning using the trained ML model after receiving a UE positioning request.

Figure 6.22.5-1: Retrieval of trained ML model(s) for LMF-based AI/ML positioning from NWDAF

1.	The LMF determines to request trained ML model(s) from the NWDAF for LMF-based AI/ML positioning, e.g. based on local policy.

2.	The LMF discovers an NWDAF that can provide trained ML model(s) for LMF-based AI/ML positioning, by invoking the Nnrf_NFdiscovery_Request service operation including an indication that support of ML Model training for LMF-based AI/ML Positioning is required, ML Model Filter Information, optionally Vendor ID, ML Model Accuracy checking capability and other parameters as described in clause 5.2 of TS 23.288 [37] and clause 5.18.

3.	The LMF receives from the NRF a list of candidates NWDAF instances that match the attributes provided in the Nnrf_NFDiscovery_Request, as specified in clause 5.2.7.3 of TS 23.502 [3].

4.	The LMF selects a NWDAF out of the list of candidates NWDAF instances, and then requests a trained ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including an indication of requesting for trained ML Model(s) for LMF-based AI/ML positioning and other parameters as described in clause 6.2A of TS 23.288 [37] and clause 5.18.

5.	The NWDAF provides one or more trained ML Models for LMF-based AI/ML positioning to the LMF including the ML Model Identifier, ML Model Information and other parameters as described in clause 6.2A of TS 23.288 [37] and clause 5.18.

6.	If the LMF subscribes to trained ML Model(s) for LMF-based AI/ML positioning from the NWDAF, based on the Nnwdaf_MLModelProvision_Subscribe request received in step 4, the NWDAF provides information of the (re-)trained ML model to the LMF by invoking Nnwdaf_MLModelProvision_Notify service operation as described in clause 6.2A of TS 23.288 [37] and clause 5.18. The LMF updates the trained ML Model(s) accordingly.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

* * * End of Changes * * * *