TSG SA WG2 Meeting #166Ad Hoc	S2-2500847

E-meeting, Jan. 20 – 24, 2025

Source:	Lenovo, Tencent, Tencent Cloud, Ericsson

Title:	Discussion on the remaining issues of expedited data transfer scheme

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract of the contribution: discussion on remaining issues for Expedited data transfer scheme agreed in KI#5

1	Background

During SA2#166 meeting, Expedited data Transfer (also known as data boosting) feature was discussed extensively. However, the 23.501 CR S2-2412944 still remains open due to time limitation and there is one issue left as Editor’s Note in this CR. In this paper, the remaining Editor’s Note is discussed.

Editor’s Note: Whether to explicitly provide the Expedited Transfer Indication value to FALSE for the non-expedited media stream is FFS.

Besides, the QoS rule configuration issue is also discussed.

Another point of discussion is whether Reflective QoS must be enabled for both media streams expedited and non-expedited. In current 23.501 CR S2-2412944 text, it is left for operator configuration to enable the Reflective QoS control or not in the non-expedited media flow.

2	Discussion

## 2.1	Editor’s Note

It is assumed during the discussion in last SA2 meeting that AF should provide two SDFs to enable expedited data transfer with associated ETI values. As captured in the Editor Note, there’re mainly two options as follows,

- Op#1a: Two SDFs associated with different Expedited Transfer Indication (ETI) values are provided by AF and used by PCF to generate two PCC rules. The PCF sets the PCC rule for the expedited media stream (EIT=TRUE) with Reflective QoS Control enabled and precedence value in the range 70-99. The PCF sets the PCC rule for the non-expedited media stream with Reflective QoS Control disabled and precedence value above 80.

- Op#1b: Two SDFs associated with different Expedited Transfer Indication (ETI) values are provided by AF and used by PCF to generate two independent PCC rules with Reflective QoS Control enabled.

- OP#2: Two SDFs are configured with two different QoS requirements, and only the expedited one is configured with ETI. PCF sets the PCC rule for the expedited media stream with Reflective QoS Control enabled and precedence value in the range 70-99. The PCF sets the PCC rule for the non-expedited media stream with Reflective QoS Control disabled and precedence value above 80.

- The following Table 1 gives a detailed analysis and comparison on the behaviours of the related network functions, including AF, PCF, SMF and UPF.

Table 1. Network Function behaviour comparison for 2 options.

|  | OP#1a: two SDFs associated with different ETI values, reflective QoS set for expedite media only OP#1b: two SDFs associated with different ETI values, reflective QoS set for both media flows | OP#2: Two SDFs are configured with two different QoS requirements, and only the expedited one is configured with ETI and reflective QoS |
| --- | --- | --- |
| AF | AF provides two media flows associated with different QoS requirements, one QoS requirement is associated with Expedited Transfer Indication (ETI) set to TRUE and the other one is set to FALSE. | AF provides two media flows associated with different QoS requirements, only one QoS requirement is associated with ETI. |
| PCF | Generate one PCC rule per media flow. No need to specifically associate two SDFs. OP#1a: PCF takes into account ETI value to enable/disable RQ and set the precedence value for each media flow. OP#1b: PCF enables RQ for both media flows with ETI present | Generate one PCC rule per media flow. Set the PCC rule associated with ETI with precedence value in the range 70-99 and RQ enabled. Set the PCC rule associated with no ETI with RQ disabled and precedence above 80 if there is another media with same IP-5 tuple and ETI set. |
| SMF | Generate two PDR rules for two PCC rules. No need to associate two PCC rules | Generate two PDR rules for two PCC rules. No need to associate two PCC rules |
| UPF | Performs PDR mapping based on both IP-5 tuple and ETI value. | Performs PDR mapping based on order determined by the precedence value, e.g., match the PDR with IP-5 tuple and ETI first. If not matched, then match the PDR with only IP-5 tuple instead. |

In order to compare the two options, the following two issues are considered.

- Issue#1: Whether PCF/SMF needs to associate two SDFs?

  - For Op#1(a&b), thanks to the different ETI values for different SDFs, no need to associate two PCC/PDR rules at PCF/SMF side. In Op#1a the PCF must set the precedence of the SDF with ETI=false to a value above 80. For Op#2, PCF needs to allocate higher precedence to the SDF with ETI set to TRUE, and lower precedence to the SDF with ETI not present. New logic needs to be added in Op#2 so that the PCF associates the media with no ETI with another media that contains same IP-5 tuple but ETI set, in order to assign the PCC rule precedence value.

Observation 1: no association is needed for the 2 SDFs in option Op#1a and Op#1b, but SDF association is needed in Op#2.

Observation 2: Op#1a and Op#2 require the PCF to set the precedence of the SDF for non-expedited transfer to a value above 80. Op#1b has no dependency on the precedence value assigned by the PCF.

- Issue#2: whether to explicitly provide the ETI value to false for the non-expedited media stream?

  - In order to support Expedited data Transfer, a new IE for Expedite Transfer can be defined on the N6 path. It is assumed that the ETI IE is always there on the N6 path regardless of the values. Otherwise, a new IE needs to be defined to indicate the existence of the ETI IE, which is wasteful. That is, ETI IE exists even for the SDF with normal QoS in Op#2, the value of which is actually FALSE. In Op#2, UPF first attempts to match the PDR with IP-5 tuple and ETI, and if not matched then only with IP-5 tuple. While for Op#1(a&b), UPF needs to perform PDR mapping including both IP-5 tuple and ETI value even for the SDF with normal QoS.

Observation 3: In both cases, PDR matching with ETI needs to be attempted first. The logic of Op#1 is slightly simple but PDR mapping at UPF when first attempt fails is slightly complex than Op#2.

## 2.2	QoS rule configuration

2.2.1  Reflective QoS scheme

In TS 24.501, the reflective QoS scheme is described in 6.2.5.1.

- 6.2.5.1.4.3 Creating a derived QoS rule by reflective QoS in the UE If the UE receives a DL user data packet marked with a QFI and an RQI, the DL user data packet belongs to a PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, and the UE does not have a derived QoS rule with the same packet filter for UL direction as the packet filter for UL direction derived from the DL user data packet as specified in subclause 6.2.5.1.4.2, then the UE shall create a new derived QoS rule as follows: a) the QFI of the derived QoS rule is set to the received QFI; b) the precedence value of the derived QoS rule is set to 80 (decimal); and c) the packet filter for UL direction of the derived QoS rule is set to the derived packet filter for UL direction; and the UE shall start the timer T3583 associated with the derived QoS rule with the RQ timer value last received during the UE-requested PDU session establishment procedure of the PDU session (see subclause 6.4.1) or the network-requested PDU session modification procedure of the PDU session (see subclause 6.4.2). If the RQ timer value was received neither in the UE-requested PDU session establishment procedure of the PDU session nor in any network-requested PDU session modification procedure of the PDU session, the default standardized RQ timer value is used.

In short, UE shall create a new derived QoS rule when it receives a DL user data packet marked with a QFI and an RQI if it does not have a derived QoS rule with the same packet filter for UL direction. And the precedence value of the derived QoS rule is set to 80 (decimal).

- 6.2.5.1.4.4 Updating a derived QoS rule by reflective QoS in the UE If the UE receives a DL user data packet associated with a QFI and an RQI, the DL user data packet belongs to a PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, and the UE has a derived QoS rule with the same packet filter for UL direction as the packet filter for UL direction derived from the DL user data packet as specified in subclause 6.2.5.1.4.2: a) the UE shall re-start the timer T3583 associated with the derived QoS rule with the RQ timer value last received during the UE-requested PDU session establishment procedure of the PDU session (see subclause 6.4.1) or the network-requested PDU session modification procedure of the PDU session (see subclause 6.4.2). If the RQ timer value was received neither in the UE-requested PDU session establishment procedure of the PDU session nor in any network-requested PDU session modification procedure of the PDU session, the default standardized RQ timer value is used; and b) if the QFI value associated with the DL user data packet is different from the QFI value stored for the derived QoS rule, the UE shall replace the QFI value stored for the derived QoS rule with the new QFI value for the derived QoS rule.

In short, UE shall replace the QFI value stored for the derived QoS rule when it receives a DL user data packet marked with a different QFI and an RQI if it has a derived QoS rule with the same packet filter for UL direction.

- 6.2.5.1.4.5 Deleting a derived QoS rule in the UE Upon expiry of timer T3583 associated with a derived QoS rule, the UE shall remove the derived QoS rule. Upon release of the PDU session, the UE shall remove the derived QoS rule(s) associated with the PDU session. If the network accepts the request from the UE to revoke the usage of reflective QoS and sets the value of the RQ timer to "deactivated" or zero, the UE shall remove the derived QoS rule(s) associated with the PDU session. Upon inter-system mobility from WB-N1 mode to NB-N1 mode or from NR connected to 5GCN to NB-N1 mode, the UE shall remove the derived QoS rule(s) associated with the PDU session that is kept active. When a derived QoS rule is deleted, the timer T3583 associated with the derived QoS rule shall be stopped.

In short, UE shall remove the derived QoS rule upon the associated timer T3583 expiry.

##### 6.2.5.1.3 UL user data packet matching For PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall attempt to associate the UL user data packet with: a) the QFI of a signalled QoS rule associated with the PDU session which has a set of packet filters containing a packet filter for UL direction matching the UL user data packet or containing a packet filter for both UL and DL directions matching the UL user data packet; or b) the QFI of a derived QoS rule associated with the PDU session which has the packet filter for UL direction matching the UL user data packet; by evaluating the QoS rules in increasing order of their precedence values until the UL user data packet is associated with a QFI or all QoS rules are evaluated

In short, UE evaluating the QoS rules, e.g., signaled QoS rule and derived QoS rule, based on the increasing order of their precedence values.

2.2.2  QoS rule configuration

Take Op#1a as an example, assume that AF provides two media flows associated with different QoS requirements, one QoS requirement is associated with Expedited Transfer Indication set to TRUE and the other one is set to FALSE (which is without ETI for Op#2). For UE supports Reflective QoS, PCF generates two PCC rules for the two SDFs and sends to SMF. SMF generates two PDRs, PDR#1 with QFI=10 for the SDF with Expedited Transfer Indication set to FALSE, PDR#2 with QFI=5 for the SDF with Expedited Transfer Indication set to TRUE. Besides, Reflective QoS is enabled for PDR#2. Then the following steps may happen,

- Step 1: DL packet is transmitted with “Expedited Transfer Indication” being set to FALSE. UPF maps the DL packet with PDR#1 and markes it with QFI=10. For UL packet, it is assumed that SMF signals QoS rule with QFI=10 and precedence value larger than 80 (decimal) to the UE. UE will also mark the UL packet with QFI=10. That is, PCF will marke sure that the precedence value for the SDF with Expedited Transfer indication set to FALSE should be set larger than the derived QoS rule. As a consequence, the PCF cannot use precedence values below 80 (decimal), reducing the possible precedence values to the range 81-99.

In Op#1b the precedence value assigned by the PCF has no impact, as both PCC rules have Reflective QoS control enabled and the UE will mark the UP packet with QFI=10 as soon as a DL packet with QFI=10 and RQI=1 is received.

- Step 2: DL packet is transmitted with “Expedited Transfer Indication” being set to TRUE. UPF maps the DL packet with PDR#2 and marks it with QFI=5 and RQI=1. Based on Observation 1, UE creates a new derived QoS rule for the data flow with QFI=5 and precedence value 80 (decimal). UE will also mark the UL packet with QFI=5 as the derived QoS rule is evaluated prioir to the signaled one and matched.

In Op#1b the UE updates the UE-derive QoS rule and marks the UL packet with QFI=5, regardless of the precedence value, as soon as a DL packet with QFI=5 and RQI=1 is received.

- Step 3: DL packet is transmitted with “Expedited Transfer Indication” being set to FALSE. UPF maps the DL packet with PDR#1 and marks it with QFI=10. For UL packet, if the timer T3583 has not expired, the derived QoS rule still works and UE marks the UL data of the QoS flow with QFI=5. If the timer T3583 expires, UE will delete the derived QoS rule and marks the UL data with QFI=10 based on the signaled QoS rule. As a consequence, Expedited Transfer in the UL will not terminate as soon as the AF stops sending packets with ETI=true, because the UE will apply the UE derived QoS rule until RQ Timer for the PDU session expires. However, the dismatch for the DL and UL is the outcome of the legacy reflective QoS scheme, which is not introduced by data boosting scheme. If we assume that ETI is set to true for each DL packet during the data boosting period. Then as long as SMF configures UE with the an appropriate small non-zero RQ timer, the dismatch for the DL and UL will be reduced.

In Op#1b the UE updates the UE-derive QoS rule and marks the UL packet with QFI=10, as soon as a DL packet with QFI=10 and RQI=1 is received, regardless of the RQ Timer value.

Observation 4: In Op#1a and Op#2 it is a requirement for the PCF to generate PCC rule with precedence value of the SDF with Expedited Transfer indication set to FALSE (or without ETI) larger than that of the derived QoS rule, i.e., 80 (decimal).

For Op#1b, SMF does not need to send QoS rule to UE in advance. UE first generates the derived QoS rule with QFI=10 in step 1. Based on TS 24.501 6.2.5.1.4.4, UE replaces the QFI value stored for the derived QoS rule (i.e., QFI=10) with the new QFI value (i.e., QFI=5) for the derived QoS rule in step 2, which enables data boosting. In step 3, UE replaces the QFI=5 with QFI=10 for the derived QoS rule, which disables data boosting. In this way, Op#1b does not need to wait timer T3583 to be expired. However, UE needs to check more packets with RQI=1 in Op#1b for SDF with ETI=FALSE than that in Op#1a and Op#2, which imposes higher burden on UE.

Observation 5: In Op#1b the UE sets the correct QoS for UL packets (same as for DL packets) as soon as the AF stops sending DL packets with ETI=true and starts sending DL packets with ETI=false. However, UE needs to check more packets with RQI=1 than that in Op#1a and Op#2.

Observation 6: in Op#1a and Op#1b the UE does not set the correct QoS for UL packets until the RQ Timer expires, which can be alleviated by setting appropriate small non-zero RQ timer by SMF.

3	Summary

The followings are observations and proposals,

Observation 1: no association is needed for the 2 SDFs in option Op#1a and Op#1b, but SDF association is needed in Op#2.

Observation 2: Op#1a and Op#2 require the PCF to set the precedence of the SDF for non-expedited transfer to a value above 80. Op#1b has no dependency on the precedence value assigned by the PCF.

Observation 3: In all options, PDR matching with ETI needs to be attempted first The logic of Op#1 is slightly simple but PDR mapping at UPF when first attempt fails is slightly complex than Op#2.

Observation 4: In Op#1a and Op#2, it is a requirement for the PCF to generate PCC rule with precedence value of the SDF with Expedited Transfer indication set to FALSE (or without ETI) larger than that of the derived QoS rule, i.e., 80 (decimal).

Observation 5: In Op#1b the UE sets the correct QoS for UL packets (same as for DL packets) as soon as the AF stops sending DL packets with ETI=true and starts sending DL packets with ETI=false. However, UE needs to check more packets with RQI=1 than that in Op#1a and Op#2.

Observation 6: in Op#1a and Op#1b the UE does not set the correct QoS for UL packets until the RQ Timer expires, which can be alleviated by setting appropriate small non-zero RQ timer by SMF.