3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500854

# Online, 20th Jan – 24th Jan, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6016 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of description on support of N3GPP device behind UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-1-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some related descriptions need to be corrected in the current text of clause 5.52.2: 1. The QoS differentiation will not be enabled in the case where the UE doesn’ t bind a Non-3GPP Device Identifier to the non-3GPP device when the traffic of the non-3GPP device requires differentiated QoS. 2. Non-3GPP Device Identifiers are linked to specific QoS in 5GC and thus it is necessary for the UE to distinguish different non-3GPP devices based on their available information or the QoS allocated to the non-3GPP device may be incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the necessity of UE binding the Non-3GPP Device Identifier to a non-3GPP device of which the traffic requires differentiated QoS from “may” to “shall”. - The UE identifies a non-3GPP device based on the information available for the non-3GPP device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of UE binding a Non-3GPP Device Identifier to a non-3GPP device is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.52.2	Traffic identification

When a non-3GPP device is connecting to the UE and if the traffic of the non-3GPP device requires differentiated QoS, the UE shall bind a Non-3GPP Device Identifier to the non-3GPP device. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific, e.g, based on the information available for the non-3GPP device (MAC address, QoS requirements, device types, etc).

NOTE 2:	At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information.

# * * * * End of changes * * * *