SA WG2 Meeting #S2-166AH	S2-2500867

20 - 24 Jan, 2025, Emeeting

Title:	LS Response on slice mapping between the HPLMN and EHPLMN

Response to:	C4-245430

Release:	Rel-18

Work Item:	TEI18, 5GS_Ph1, 5GProtoc17

Source:	SA2

To:	CT4

Cc:	CT1

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR6022(R19)，23.501 CR6037(R18)

1. Overall Description:

SA2 thanks for CT4 for the LS (C4-245430) on slice mapping between the HPLMN and EHPLMN. In the LS CT4 mentioned that the following assumptions has been reached:

1. For non-roaming case of NSAC for number of UEs, as indicated in SA2 answer, the operator may deploy only one NSACF which serves both EHPLMN S-NSSAI and HPLMN S-NSSAI. In this case, the AMF may send one request to this NSACF, carrying the serving PLMN (i.e. EHPLMN) S-NSSAI together with the serving PLMN ID and the corresponding mapped HPLMN S-NSSAI together with the HPLMN ID.

2. For non-roaming case of NSAC for number of PDU sessions, I-SMF shall provide the S-NSSAI of the serving PLMN (i.e. EHPLMN) to the NSACF and A-SMF shall provide the S-NSSAI of the home PLMN to the NSACF.

CT4 further asked 3 questions regarding the following NOTE in clause of 5.15.11.2.1 in TS 23.501.

NOTE 2:	I-SMF does not interact with NSCAF.

SA2 would like to provide the answers as following:

Q1: Could the restriction of interaction between I-SMF and NSACF be removed in the case of different S-NSSAI values are used for EHPLMN and HPLMN?

SA2 Answer: Yes. SA2 agreed the CRs as attached.

Q2: If the answer of Q1 is yes, are the CT4’s assumptions still conflicting with any other stage2 principles?

SA2 Answer: SA2 did not discuss to include two PLMN IDs (i.e. EHPLMN ID and HPLMN ID) in the same Nnsacf_NSAC_NumOfUEsUpdate service operation. See clause 5.2.21.2.2 of TS 23.502, only one PLMN ID is included and the PLMN ID is the Serving PLMN ID of the UE.

Q3: If the answer of Q1 is no, what is the correct way to handle the EHPLMN NSAC in above scenario from stage 2 perspective?

SA2 Answer: See answer to Q1 above.

2. Actions:

To CT4

ACTION:

SA2 kindly asks CT4 to take the information above into account.

3. Date of Next SA2 Meetings:

SA2#167	Feb 17-21, 2025 					Athens, GR

SA2#168	Apr 7-11, 2025 						Goteborg, SE