SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500884

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6027 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting the input and output of SMF and resolving the EN on roaming. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the 4.2.18.1, there is EN on whether Raoming is supporting is FFS. There is no requiremment and solution for this. It proposes to remove this EN directly. In the subscribe service operation, the consumer (e.g. EIF) provides the UE ID, DNN/S-NSSAI, IP-5 tuples. The producer (e.g. SMF), notify the result which does not contain input provided by consumer. It proposes to extract the consumer input from the table Table 5.51.2.2-1. Editorial change, move the “” sentence from clause 5.51.2.1 to 5.51.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN on roaming. Modify the table and Editorial change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not resolved. The content in the table is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.18.1, 5.51.2.1, 5.51.2.2, 5.51.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.2.18.1	Non-roaming architecture

Figure 4.2.18-1 depicts the non-roaming reference architecture using service-based interfaces. The figure depicts only the NFs enhanced to support the Energy Efficiency and Energy Saving functionality.

Figure 4.2.18-2 depicts the non-roaming reference architecture, using the reference point representation showing how various network functions interact with each other. The figure depicts the direct interface of the Energy Information Function (EIF) with other NFs only.

Figure 4.2.18-1: Non-roaming architecture for Energy Efficiency and Energy Saving

Figure 4.2.18-2: Non-roaming architecture for Energy Efficiency and Energy Saving in reference point representation

NOTE 1:	The reference points between NF and NRF is not shown in the figure 4.2.18-2.

Editor's note:	The interface between the EIF and OAM may need coordination with SA WG5.

NOTE 2:	In this release, the Roaming is not supported.

Editor's note:	The reference point name will be changed later.

Next CHANGE

### 5.51.2	Energy Consumption Information collection, calculation and exposure

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, PDU Session and/or QoS flow granularity and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) subject to operator's policy.

Next CHANGE

#### 5.51.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. PDU Session and/or QoS flow) from UPF (via SMF).

Node level energy consumption is reported PLMN-wide at a configurable period T.

The serving SMFs are retrieved from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

EIF provides UE ID, DNN/S-NSSAI, IP 5-tuple which is shown in Table 5.51.2.2-1 to retrieve the information from SMF. And the information collected from SMF by EIF, is shown in Table 5.51.2.2-2.

Table 5.51.2.2-1: Information to SMF for retrieve the information

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |

The data volume reporting period from the SMFs is the period T with PLMN-wide configuration.

Table 5.51.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time period. |
| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB serving the UE | The gNB which serving the UE. |
| Time period | Time period of the collected information. |

Table 5.51.2.2-3 provides the list of information received from OAM. EIF requests the OAM information with providing the serving gNB ID(s) and (I-)UPF ID(s), which provided by SMFs. The reporting period from the OAM is the period T which is the same as data volume reporting period from SMFs.

Table 5.51.2.2-3: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time period T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time period T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time period T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

Next CHANGE

#### 5.51.2.3	Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularities (UE, PDU Session and/or QoS flow), based on input parameters from Tables 5.51.2.2-2 and 5.51.2.2-3 and gets the results. Some example formulas to support the above calculation are described in Annex T.

END OF CHANGES