3GPP TSG-SA2 Meeting #166 Ad Hoc-e	S2-2500895

# Online, 20th Jan 2024 – 24th Jan 2025				                             (revision of S2-25xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1573 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IMS DC figures to align with other annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per Annex AC.2.1 of TS 23.228, NEF is used in IMS DC architecture to exposure IMS network capabilities to the 3rd party AS/AF. As per TS 29.500, the Service Based Architecture shall support the NF Service Framework that enable the use of NF servicecs as specified in clause 7.1 of 3GPP TS 23.501, which includes that NF services may communicate directly between NF Service consumers and NF Service Producers, or indirectly via an SCP. The principle also applies to IMS SBA. But IMS SBA shown in Figure AA.1.1-1 does not align with them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NEF and SCP in Figure AA.1.1-1 to align with other annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1, AA.1.2, AC.2.1 and AC.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *  First Change * * * *

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE 1:	The DCSF and MRF supports NF registration and discovery, but they do not provide services in this release of the specification.

NOTE 2:	Nimsas services are defined in the context of IMS Data Channel, see clause AA.2.4.

NOTE 3:	If an SCP is deployed it can be used for indirect communication between NFs and NF services as described in Annex E of 3GPP TS 23.501 [93].

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 3:	In this Release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

# * * * *  Second Change * * * *

## AA.1.2	Reference point to support SBA in IMS

Following reference points are realized by service-based interfaces in IMS:

N5:	Reference point between the PCF and an AF.

NOTE:	P-CSCF acts as an AF from PCF point of view. N5 Reference point is defined in TS 23.501 [93].

N70:	Reference point between an SBI capable I/S-CSCF and an SBI capable HSS.

N71:	Reference point between an SBI capable IMS AS and an SBI capable HSS.

DC1:	Reference point between an SBI capable IMS AS and DCSF.

DC2:	Reference point between an SBI capable IMS AS and MF.

DC3:	Referencec point between DCSF and NEF.

N72:	Reference point between DCSF and an SBI capable HSS.

Nxx:	Reference point between an SBI capable IMS AS and NEF.

Nyy:	Reference point between an SBI capable HSS and NEF.

# * * * *  Third Change * * * *

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

# * * * *  Forth Change * * * *

## AC.2.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

-	DC4:	Reference point between the DCSF and DC Application Server.

-	DC5:	Reference point between the DCSF and DCAR.

-	N72/Sc:	Reference point between the DCSF and HSS.

The following reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71/Sh:	Reference point between the IMS AS and HSS.

The following reference points are updated to support IMS events and IMS network capabilities exposure:

-	N33:	Reference point between the NEF and the 3rd party AF/AS.

The following reference points are defined for IMS events and IMS network capabilities exposure:

-	Nxx:	Reference point between the IMS AS and NEF.

-	Nyy:	Reference point between the HSS and NEF.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function MF and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function MF and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

# * * * *  End of Changes * * * *