3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500930

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: AF Authorization for Ambient IoT Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the support of AF Authorization for Ambient IoT Services via subscription Management for 3rd party AF.

1. Discussion

Follow the conclusion in clause 8.2.2 of TR 23700-13, this proposal introduces the support of AF authorization data in UDM in order enable authorize the Ambient IoT Services from AF.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

5.6	AF Authorization for Ambient IoT Services

The network exposes the Ambient IoT services to the AF. The authorization data for the AF is required to allow the network to perform authorization of the AF request targeting for Ambient IoT service, in addition to the authorization at NEF based on local configuration or operator policy.

The ADM holds authorization data via OAM configuration for every AF client that may initiate Ambient IoT service requests, the AF ID is the primary key for authorization data in the ADM. The AIOTF checks whether the external AF client that initiates the Ambient IoT Service has authorization data and retrieves, the AIOTF performs validation on the following items if requested by the AF:

-	whether the AF is authorized for the target area,

-	whether the AF is authorized for the requested service operation,

-	whether the AF is authorized for the requested target AIoT device(s)

The table 5.6-1 below describes information storage structures that are mandatory (M), conditional (C) or optional (O) for the AF authorization data.

Table 5.6-1: AF Authorization Data

| AF Authorization Data | Status | Description |
| --- | --- | --- |
| AF ID | M | Identifier used to identify the AF |
| Allowed area | M | Indicate the allowed area for the indicated AF. |
| Allowed service operations | O | Indicated the allowed service operation (s) for the indicated AF, e.g., inventory, read, write, permanent disable. |
| Allowed target AIoT devices | O | Indicate the allowed list of AIoT Device ID for the indicated AF. |

# * * * * End of changes * * * *