3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500932

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Subscription Management for Ambient IoT Device

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the support of Subscription Management for Ambient IoT Device.

1. Discussion

Following the conclusion in clause 8.2.2 of TR 23.700-13, this proposal introduces the support of Subscription Data Management for Ambient IoT Device.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

5.5	AIoT Device Subscription Data Management

The AIoT Device may or may not have operator’s subscription data in the network. If the AIoT Device is managed by the network, then the subscription data for an AIoT Device is required in the network, otherwise the AIoT device doesn’t have the operator’s subscription data in the network, whereas the corresponding profile (e.g., Device ID or credentials) is stored in the AAA server external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the subscription data or corresponding profile of an AIoT Device is.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription in the network and retrieves the subscription data. The subscription data for AIoT Device is different with UE subscription data as defined in TS 23.502 clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that supports management of AIoT Device’s subscription data, and this ADM network entity is independent from the UDM managing the UE subscription. The AIoT Device ID is the primary key for AIoT device subscription data in the ADM.

Editor’s note: Whether ADM support ID security mechanism pending on SA3 decision.

The table 5.5-1 below describes information storage structures for AIoT device subscription data.

Table 5.5-1: AIoT Device Subscription Data

| Field | Description |
| --- | --- |
| Disabled flag | Indicate the disabled status information (e.g., current status, status change timestamp etc) for an AIoT device to be permanently disabled |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

# * * * * End of changes * * * *