3GPP TSG-WG SA2 Meeting #166-AH-e	S2-2501002

Electronic Meeting, 20th Jan – 24th Jan, 2025

Source:	Xiaomi

Title:	Discussion on AL-FEC awareness at NG-RAN

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract: This paper discusses how to support the AL-FEC awareness at NG-RAN in this release.

1. Introduction

Normative work for AL-FEC awareness at NG-RAN will be based on the following principles:

-	When requesting or updating QoS for a DL flow, an AF may provide to PCF/NEF - together with PDU Set QoS parameters - the content ratio for the flow. An AF may provide either PSIHI or content ratio for a flow.

-	If a PCF receives the content ratio from an AF, then the PCF may include the content ratio in PCC rules that it provides to the SMF.

-	The SMF provides content ratio to NG-RAN when establishing/modifying a QoS flow.

-	If NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows:

-	If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding.

NOTE 1:	The principles concluded for AL-FEC awareness at RAN (as above) is subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2.

NOTE 2:	RAN can, e.g. primarily discard obsolete AL-FEC PDUs of each PDU Set.

NOTE 3:	There is no difference in terms of how many PDUs NG-RAN will be discarded for a given QoS flow. In other words, NG-RAN will not discard more PDUs if content ratio information is provided for a QoS flow compared to when no content ratio information is provided for the same QoS flow. This scheme merely allows NG-RAN to select the PDUs to be discarded to ensure that the original content can still be recovered at the receiver.

NOTE 4:	Since there is no difference how many PDUs will be discarded, there are no impacts to existing congestion control algorithms.

In Rel-19, SA2 defined the content ratio as the ratio of PDUs of a PDU Set that are needed at the UE to be able to reconstruct the original content. In TR 23.700-70 the conclusion for KI#1_AL-FEC states:

- The LS replies from RAN2 on AL-FEC awareness at NG-RAN:

In S2-2500043/R2-2411002:

Within RAN2, there is no consensus on the usefulness of using AL-FEC content ratio information for discarding DL PDUs by NG-RAN during congestion. It is up to NG-RAN how/whether to use content ratio information, should it be provided.

In S2-2409567/R2-2405781:

Questions for RAN2:

- Can NG-RAN determine whether a PDU was successfully delivered over an unacknowledged mode data bearer? If so, does NG-RAN get this information sufficiently early to decide whether or not to drop subsequent AL-FEC packets?

Answer:	RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer.

- Provide feedback on the impact on NG-RAN to support dynamic redundancy ratios, i.e., a different ratio of PDUs that need to be successfully transferred to the UE for different PDU Sets within the same QoS flow?

Answer:	So far RAN2 has not yet had sufficient time to analyse the proposal in detail. Based on initial analysis, RAN2 thinks that it may have negative impact on RAN performance due to intentional dropping of packets. Further analysis is required to fully assess those impacts. In addition, RAN2 thinks that it may introduce additional complexity for RAN on both UL and DL. For DL, while no impact on RAN2 specifications is foreseen, it may have impact on network implementation complexity. For UL, there might be potential impact on RAN2 specifications.

In S2-2409610/S4-241785:

To SA2, SA4 would like to make the following addition on the operation of AL-FEC codes:

For RLC UM and MDS AL-FEC codes, there is no need to have 100% reliability in determining the successful delivery of individual PDUs, as long as the probability that as many PDUs as the source PDUs are successfully delivered per PDU Set is high enough. The same concept can be applied to near MDS AL-FEC codes.

In S2-2407404/S4-241368:

- SA2 understands that different AL-FEC mechanisms exist (e.g., maximum-distance separable (MDS) schemes like RaptorQ and Reed-Solomon, FlexFEC, etc.) and is discussing for which AL-FEC mechanisms to enable AL-FEC awareness at RAN. Can SA4 identify commonly used AL-FEC mechanisms (not necessarily 3GPP defined), which should be supported for AL-FEC awareness at RAN from SA4's perspective?

SA4 Answer:

There are two types of AL-FEC: MDS or near-MDS codes, and non-MDS codes. Both MDS codes and non-MDS codes are commonly used in the industry. For example, the MTSI service specified in 3GPP TS 26.114 supports FlexFEC. From SA4’s perspective, both types of codes may be supported. For non-MDS codes, e.g., FlexFEC and ULPFEC, the exact dependency between the source packets and the parity packets needs to be known to be able to identify a sufficient set of packets for the reconstruction of the original, uncoded information. Please refer to the SA4 TR 26.822 Clause 6.5 for more details on the AL-FEC codes.

- Does SA4 see a need (from a general application perspective) to support both static and dynamic redundancy ratios (i.e., the ratio of AL-FEC information) for AL-FEC awareness at RAN?

SA4 Answer:

Both static and dynamic redundancy ratios are used by the industry and may be supported. There are AL-FEC schemes in which the redundancy ratio is fixed based on code construction. Furthermore, dynamic redundancy ratios may be more beneficial in practice since applications often appeal to dynamic AL-FEC control in combination with retransmissions and congestion control mechanisms to ensure robust and efficient adaptation to network conditions.

- Does SA4 see a need for the application layer to distinguish RAN’s intentionally dropped obsolete FEC packets from congestion related drops, and related to this, the need for specific application behaviour, e.g., to reduce the sending rate? The background to this question is the following:

SA4 Answer: For the current congestion control mechanisms for real-time communication of which SA4 is aware, SA4 sees a need for the application layer to distinguish RAN’s intentionally dropped obsolete FEC packets from congestion related drops, and the need, depending on the operating point, for reducing the sending rate. In the RFC’s for real-time communication SA4 have reviewed, all packet losses should be considered as a signal of network congestion. When the RAN is enabled by a sender application to intentionally drop obsolete FEC packets, the RAN should indicate the action of intentionally dropping obsolete FEC packets to the application, so that the application can respond to intentional drops of obsolete FEC packets differently than to congestion related drops. 3GPP SA4 recommends notifying IETF about this system behaviour.

- Other companies argued that there is no need for reducing the sending rate when NG-RAN discards obsolete AL-FEC PDUs as long as NG-RAN can still meet the QoS characteristics of the other QoS flows in the same cell (i.e., because there is no fairness issue in this case).

SA4 Answer:

  - When the network is in congestion, SA4 sees a need for reducing the sending rate in general.

  - When the network is not in congestion, if the RAN discards obsolete AL-FEC PDUs, SA4 thinks that it may be okay to not reduce the sending rate. However, in this uncongested case SA4 sees that there is a need for the RAN to inform the application if packets are intentionally discarded. Note, the congestion control also needs to consider packet losses and congestion on N6 (e.g., paths through the Internet).

  - SA4 will study under FS_5G_RTP_Ph2 the benefits for the applications and the implications specifically with congestion control when the NG-RAN discards obsolete AL-FEC PDUs when the network is not in congestion.

The LS replies from SA4 on AL-FEC awareness at NG-RAN:

- Enhancements to provide AL-FEC awareness, content ratio over Control plane for NG-RAN.

NOTE:	Whether to do enhancements for AL-FEC awareness at RAN are subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2.

SA approved the XRM_Ph2 WID in SP-241385, which includes the following

The progress including conclusions and replies about AL-FEC awareness at NG-RAN are summarized as above. This paper discusses the approach to support the AL-FEC awareness at NG-RAN in Rel-19.

2. Discussion

As summarized in the introduction about the progress including conclusions and replies about AL-FEC awareness at NG-RAN in Rel-19, there are several observations and proposals, as follows:

Observation 1: From SA4 perspective,

- For the two types of AL-FEC mechanisms, both MDS codes and non-MDS codes are commonly used in the industry.

- For the two types of AL-FEC redundancy ratios, both static and dynamic redundancy ratios are used by the industry and may be supported.

- SA4 sees a need for the application layer to distinguish RAN’s intentionally dropped obsolete FEC packets from congestion related drops.

- When the network is in congestion, SA4 sees a need for reducing the sending rate in general.

- For RLC UM and MDS AL-FEC codes, there is no need to have 100% reliability in determining the successful delivery of individual PDUs.

Observation 2: From RAN2 perspective,

- There is no consensus till now on the usefulness of using AL-FEC content ratio information for discarding DL PDUs by NG-RAN during congestion.

- It is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications, in case if the content ratio information provided to the gNB.

- With respect to RLC UM, RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered.

- For DL, while no impact on RAN2 specifications is foreseen.

Observation 3: From SA2 perspective,

- The content ratio as the ratio of PDUs of a PDU Set is defined and can be provided over Control plane to NG-RAN, for RLC AM support in this release, if the content ratio provided by AF.

- When the PDU Set content ratio provided to the NG-RAN, the PSIHI will not be provided and the Per Set discarding based on PSIHI by the NG-RAN may not be performed.

Proposal 1: Provide the PDU Set content ratio over Control plane to NG-RAN for RLC AM supporting, if the content ratio provided by AF. It’s up to NG-RAN whether and how to use it.

Proposal 2: Clarify that the PSIHI will not be provided and the Per Set discarding based on PSIHI by the NG-RAN may not be performed, when the PDU Set content ratio provided to the NG-RAN.

Proposal 3: Send the LS to NG-RAN to notify the progress including the provision of PDU Set content ratio, and the effects on the PSIHI and Per Set discarding based on PSIHI by the NG-RAN.

3. Conclusion

It is proposed to agree the above proposals.