3GPP TSG-WG SA2 Meeting #166-AH-e	S2-2501003

Electronic Meeting, 20th Jan – 24th Jan, 2025

Source:	Xiaomi

Title:	Discussion on one direction PDU Set support in Alternative QoS Profile

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract: This paper discusses how to provide and determine AQPs with one direction PDU set support.

1. Introduction

The following conclusion principles are agreed for key issue #1_AQP.

1.	The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

2.	Based on the Alternative Service Requirements from AF, the PCF generates PCC rule with Alternative QoS parameters in a prioritized order, which can include UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER) and the corresponding reference to each Alternative QoS parameter Set.

3.	SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN. When the QoS profile includes UL and/or DL PDU Set QoS parameter, the Alternative QoS Profile shall include the corresponding PDU Set QoS parameters; otherwise, the Alternative QoS profile does not include the corresponding PDU Set QoS parameters(s) when the QoS profile doesn't include the UL and/or DL PDU Set QoS parameter.

4.	If QoS notification is determined as "GFBR cannot be guaranteed", the NG-RAN determines the AQP that can be used as a reference to the currently fulfilled QoS considering the priority order: If the PDU Set QoS handling is applied in the corresponding direction, the PSDB and PSER in the corresponding direction and GFBR in the AQP are used by the NG RAN to determine the AQP that can be used as a reference to the currently fulfilled QoS.

5.	During PDU Session Establishment/Modification procedures, the SMF/PCF may update the QoS profile and/or Alternative QoS Profiles to include/exclude the PDU Set QoS parameters based on the PDU Set Based Handling Support Indication from the NG-RAN, and/or the QoS requirement from the AF.

In TR 23.700-70 the conclusion for KI#1 states:

There are however no further details mentioned about when the PDU Set QoS parameters only in one direction provided with the PDU QoS parameters together in the QoS profile and the Alternative QoS profiles, principles or priorities of the fulfilled QoS selection in the AQPs.

This paper discusses approaches for the one direction PDU Set support in Alternative QoS profiles.

2. Discussion

For the AQP applied with the PDU Set QoS parameters, there are no further details mentioned about:

- If the PDU Set QoS parameters cannot be guaranteed, when AQP applied with the PDU Set QoS parameters in one direction (e.g., DL), whether the fulfilled DL PDU Set QoS parameters in AQP will be determined primarily by NG-RAN, rather than the fulfilled DL PDU QoS parameters (e.g., PDB and/or PER) in the applied QoS Profile will be selected?

- If the PDU Set QoS parameters cannot be guaranteed, when AQP applied with the PDU Set QoS parameters in one direction (e.g., DL), and the PDU QoS parameters applied and guaranteed in another direction (e.g. UL) without the PDU Set QoS parameters, how to determine or guarantee the UL QoS when the fulfilled DL PDU Set QoS parameters in AQP determined and selected by NG-RAN? Whether is it up to the operator policy, or same UL QoS parameters provided in the different AQPs when AQP applied with the PDU Set QoS parameters in DL?

- If the PDU Set QoS parameters provided and applied in one direction (e.g., DL), and QoS parameters provided and applied in another direction (e.g., UL), when both directions cannot be guaranteed, which part will have higher priority to determine the new fulfilled AQPs? E.g., QoS parameters in UL or the PDU Set QoS parameters in DL?

Observation 1: When the PDU Set QoS parameters provided with the PDU QoS parameters together in the QoS profile and the Alternative QoS profiles, the QoS guarantee is independent in different directions.

Observation 2: When AQP applied with the PDU Set QoS parameters in one direction (e.g., DL), and the PDU Set QoS parameters cannot be guaranteed, one of the profiles (AQPs with highest priority) with the DL PDU Set QoS parameters may be selected to fulfil the DL PDU Set QoS requirements by the NG-RAN, or the DL PDU QoS parameters applied in the same profile if applicable, or another profile (AQPs with highest priority) may be selected to fulfil the DL PDU QoS parameters by the NG-RAN.

Proposal 1: The priority between the PDU Set QoS parameters and the PDU QoS parameters in the same direction can be indicated to the NG-RAN, e.g. the profile with highest priority of AQPs with the DL PDU Set QoS parameters can be selected to fulfil the DL PDU Set QoS by the NG-RAN, rather than the DL PDU QoS parameters applied.

Observation 3: For the PDU Set QoS parameters provided and applied in one direction, and QoS parameters provided and applied in another direction, when both directions cannot be guaranteed, the NG-RAN can try to fulfil the QoS profile with switching the QoS in the profile, or can determine the new profile considering the QoS parameters firstly, or the PDU Set QoS parameters in another direction firstly.

Observation 4: When AQP applied with the PDU Set QoS parameters in one direction, and applied with the legacy PDU QoS parameters in another direction, principle or priority of the fulfilled QoS selection should be indicated when the QoS cannot be guaranteed in one direction, or both directions.

Proposal 2: Same priority between the PDU Set QoS parameters and the PDU QoS parameters, for both one direction and two directions when the QoS cannot be guaranteed.

3. Conclusion

It is proposed to agree the above proposals.