3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2501016

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1317 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Updates to VFL preparation procedures e.g to address trusted AF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  | Date: |  | 2025-01-08 |
| Category: | B |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - It is ffs how to document trusted AF as VFL server case case. While there are similarities with procedures for NWDAF as VFL server, there are also differences, in particular related to the trigger for the procedure and the fact that only NWDAFs can be discovered as VFL clients. - For the VFL preparation procedure, whether and how NEF does pre-work of sample IDs intersection before the VFL server determines the final sample IDs is FFS. It is suggested not to support such sample intersection to allow for separate sessions at the NEF for each VFL client. - It is FFS whether trusted AF can act as VFL clients when an untrusted AF acts as VFL Server. It is suggested not to standardize such scenarios. Interactions between AFs can be left outside 3GPP standards - For UEs as samples, additional discussion is needed on whether UE needs to be registered or not and whether the NWDAF as VFL server can check whether UEs are registered before interacting with VFL clients. Unregistered UEs are not suitable for samples as no data related to them can be collected. It is preferable that the VFL server checks whether UEs are registered at the UDM instead of each VFL client performing such checks. An NWDAF and trusted AF can directly interact with the UDM. An untrusted AF can use the Nnef_MemberSelection service to check whether UEs are registered. |  |  |  |  |  |
| Summary of change: |  | - Add procedures for trusted AF as VFL server into Clause for untrusted AF as VFL server but work with cross-references for the description of related steps. - Remove EN about sample intersection at NEF. - Remove EN about interactions between AFs - The VFL server checks whether UEs are registered at the UDM. An NWDAF and trusted AF can directly interact with the UDM. An untrusted AF can use the Nnef_MemberSelection service . |  |  |  |  |  |

| Consequences if not approved: | 1.Trusted AF as VFL server case is not documented. 2. Unresolved issue about sample intersection at NEF remains. 3. Unresolved issue about interactions between AFs remains. 4. Unresolved issue about samßple selection remains and there is a risk of unnnecessary signalling as each client might perform checks at the UDM for each UE. |  |  |  |
| --- | --- | --- | --- | --- |
| Unresolved |  |  |  |  |
| Clauses affected: | 6.2H.2.2.1, 6.2H.2.2.2 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | x | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | x | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

##### 6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

0.	If UEs are to be used as samples, the NWDAF as VFL server may check at the UDM whether the related UEs are registered.

1.	The NWDAF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. The NWDAF as a VFL Server also provides, the suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training, the suggested list of sample IDs that will be used in training.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client ID checks the list of sample IDs suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept out of the sample IDs suggested by the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client accepts. The VFL Clients provides the list of supported Feature IDs to the VFL Server, if available, alternatively the VFL clients may not want to reveal Feature IDs or VFL Server may know the supported FeatureIDs for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_VFLTraining_Response or and each AF VFL Client invokes Naf_VFLTrainingRequest_Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process.

4.	The VFL server determines the final list of samples considering the samples that all selected VFL clients support, if used the Feature ID per VFL client and VFL Interoperability Information to be used for training and provide them to the selected VFL Clients at the start of the training phase, as described in clause 6.2H.2.3.1.

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when AF is the VFL Server

0.	If UEs are to be used as samples, the AF server may check whether the related UEs are available. A trusted AF as VFL server may check at the UDM whether the related UEs are registered. An untrusted AF may use the Nnef_MemberSelection service to ask the NEF to reduce the list of candidate UEs based on filter criteria it provides.

Steps 1 to 5 apply when the VFL server is an untrusted AF.

1-	The untrusted AF as VFL server sends VFL preparation requests to each of the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training.

2-	The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The NEF send VFL preparation request to the candidate internal NWDAF ID using Nnwdaf_VFLTraining_Request service with the same information as provided in step 1 in clause 6.2H.2.2.1.

3-	Same as step 2 in clause 6.2H.2.2-1.

4-	Same as step 3 in clause 6.2H.2.2-1.

5-	The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

Steps 6 to 8 apply when the VFL server is a trusted AF.

6-8		Same as steps 1 to 3 in clause 6.2H.2.2-1, with the exception that trusted AF is VFL server and only NWDAF are applicable as VFL clients.

9-	Same as step 4 in clause 6.2H.2.2-1.