3GPP SA WG2 Meeting #166-AdHoc-e	S2-2501023

# Electronic Meeting, 20-24 January 2025								                  rev of S2-250xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6054 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for non-3GPP connection information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.502, non-3GPP device connection information is defined to used in PDU session modification proceudure. But in TS23.501, the definition of non-3GPP device connection information is missing. So propose to align the discription for PDU session modification with TS 23.502 by using non-3GPP device connection information. Also bring the EN from TS 23.502. but it should be a NOTE, not a EN, because it doesn’t need further update when stage 3 finishes the work. Editor's note: Details on how non-3GPP device connection information is included within PDU Session Modification Request are to be defined in stage 3. So propose to change as NOTE: NOTE: Details on how non-3GPP device connection information is included within PDU Session Modification Request are to be defined in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Rephase the description for PDU session modification proceudre with “non-3GPP device connection information” - To add the NOTE from TS 23.502 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The term “non-3GPP device information” is misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the UE sends the non-3GPP device connection information (e.g., non-3GPP Device Identifier and user plane information) to SMF in PDU Session Modification procedure:

-	For Ethernet PDU Session Type, the non-3GPP device information includes the Non-3GPP Device Identifier and optional MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier

-	For IPv4 PDU Session Type, the non-3GPP device information includes the Non-3GPP Device Identifier and optional  the IP Address and/port ranges associated with the Non-3GPP Device Identifier.

-	For IPv6 PDU Session Type, the non-3GPP device information includes the Non-3GPP Device Identifier and optional  the IPv6 Address/prefix(sub) that is associated with the Non-3GPP Device Identifier.

Editor's note:	Whether the UE sends the Non-3GPP Device Identifier and user plane information (e.g. IP Address) in PDU session establishment is FFS.

NOTE:	Details on how non-3GPP device connection information is included within PDU Session Modification Request are to be defined in stage 3.

* * * End of Change * * *