3GPP TSG-WG SA2 #166-Ad Hoc-e	S2-2501036

# Jan. 20 – 24, 2025, eMeeting													(revision of S2-2412660)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1388 | rev | 6 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Clarify the ambiguity of QoS requirements of media-flow vs. data-flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, Ericsson(?), Huawei(?), HiSilicon(?), Samsung, Nokia, NTT DOCOMO, Xiaomi, vivo, Lenovo], China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The XRM_Ph2 KI#4 standardizes the support of the differentiated QoS handling for multiplexed media flows. - The TS 23.501 clause 5.37.11 clearly states the relationship between and differentiates the usage of media flow vs. data flow, as quoted following: The AF may provide individual QoS requirements for each individual RTP media flow within the multiplexed data flows and for the (S)RTCP that controls the (S)RTP transmission, by including the (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements within the flow description in the "AF session with the required QoS" procedure specified in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3]. The policy control related to multiplexed media flows identification is specified in clause 6.1.3.22 of TS 23.503 [45]. It also indicates explicitly the QoS requirement is per individual media-flow based. - The TS 23.501 clause 3.1 (Definitions) defines the nonmenclature ‘(S)RTP Multiplexed Media Identification Information’ as quoted following: (S)RTP Multiplexed Media Identification Information: Component of the Packet filter information used to identify different media flows that are transported in (S)RTP, or the (S)RTCP that controls the (S)RTP transmission, when (S)RTP, (S)RTCP and other associated protocols are multiplexed into a single IP traffic flow as specified in IETF RFC 5761 [203], IETF RFC 5764 [204], IETF RFC 7983 [205], IETF RFC 8872 [206], IETF RFC 9143 [207] and IETF RFC 9443 [208]. which champions explicitly the usage of media flow(s). - In the SA2#166 held in Nov. 2024 in Orlando, USA, the KI#4 23.503 baseline document (S2-2412660) has been extensively discussed. This document recommended to add the following paragraph in a to-be-added new clause (so-named as the clause 6.1.3.27.x): In order to support differentiated QoS for multiplexed media flows multiplexed on the same end-to-end transport layer connection, the AF may request for differentiated QoS handling for multiplexed media flows by including (S)RTP Multiplexed Media Identification Information (see TS 23.501 [2] clause 5.7.6.2) in the Flow Description and specific QoS requirements per media flow as described in clause 6.1.3.27.3. Just like what is in TS 23.501, this paragraph (targeting for 23.503) also adopts the usage of media flow(s). Further, it references the 23.503 clause 6.1.3.27.3 for per media-flow based QoS requirements. - However, currently the TS 23.503 clause 6.1.3.27.3 only specifies the QoS information and service requirements for each data flow, without mentioning the newly-introduced term ‘media flow’. This leads to the possible ambiguity of QoS requirements for media flows or for data flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a ‘NOTE Y1’ to clarify the ambiguity of QoS requirements for media flows or for data flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Having ambiguity of QoS requirements for media flows or for data flows |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change in this CR will be the add-on to a new clause in 23.503 6.1.3.27.x that was originally proposed to be added by CR: S2-2412660. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

##### 6.1.3.27.x	Traffic identification and differentiated QoS for multiplexed media flows

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.x of TS 23.501 [2].

When the UE indicates the support for IP Packet Filter with (S)RTP Multiplexed Media Identification Information to SMF at PDU Session establishment,  the SMF provides this UE capability information to PCF.

In order to support differentiated QoS for multiplexed media flows multiplexed on the same end-to-end transport layer connection, the AF may request for differentiated QoS handling for multiplexed media flows by including (S)RTP Multiplexed Media Identification Information(see TS 23.501 [2] clause 5.7.6.2) in the Flow Description and specific QoS requirements per media flow as described in clause 6.1.3.27.3. The PCF determines the PCC Rules and QoS requirements for the media flows under consideration of the UE support for  IP Packet Filter with (S)RTP Multiplexed Media Identification Information as follows:

- When the AF request includes uplink direction and the UE capability indicates support for IP Packet Filter  with (S)RTP Multiplexed Media Identification Information, or the AF request is for downlink only, PCF generates different PCC rules for the corresponding media flows. Each PCC Rule includes the QoS requirements that apply to the corresponding media flow identified by (S)RTP Multiplexed Media Identification Information in the IP Packet Filter set.

- When the AF request includes uplink direction and the UE has not indicated support for IP Packet Filter with (S)RTP Multiplexed Media Identification Information, the PCF may notify the AF that the differentiated QoS handling for multiplexed media flows cannot be fulfilled.

NOTE Y:	It is assumed the PSA UPF supports the IP Packet Filter with (S)RTP Multiplexed Media Identification Information for multiplexed media.

NOTE Y1:	The Flow Description and specific QoS requirements are per media flow, which is different from those for per data flow in clause 6.1.3.27.3.

# * * * * End of changes * * * *