SA WG2 Meeting #166 Adhoc-e	S2-2501041

# E-meeting, 20 – 24 January 2025	(was S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3881 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition between S&F Mode and Normal Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exception sheet in SP-241918 for 5GSAT_Ph3-ARC includes for S&F operation the following: - (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode Changes are thus needed to define the transitioning in both a UE and a satellite. In addition, the conclusions of TR 23.700-29 in clause 8.2 state that: The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. …… - Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network. NOTE 7: The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5]. With a dedicated USIM for S&F mode, it is not necessary for the UE to detach for one mode when attaching for the other mode. Consequently, UE transitions between S&F and normal modes can be based on whether the UE uses the same USIM for both modes (when the UE can only be attached for one mode at any time) or whether the UE uses different USIMs (when the UE can be attached for both modes at the same time). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UE and satellite requirements for transition between S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE or satellite may not be able to successfully or correctly transition from normal mode to S&F mode or the reverse which could prevent network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.x (new), Annex O.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.x	Transition between S&F mode and normal mode

In order for a UE to switch between S&F mode and normal mode, a UE that cannot support simultaneous S&F and normal operation shall detach locally, as defined in TS 24.301 [46], for either mode when previously attached for this mode and after successfully completing a new attach for the other mode. A UE that can support simultaneous S&F and normal operation shall not detach locally for either mode when previously attached for this mode and after successfully completing a new attach for the other mode.

NOTE:	To support simultaneous S&F and normal operation where a UE can be dual attached for normal mode and S&F mode, an HPLMN operator needs to use a different second IMSI for S&F mode on the HPLMN side. The different second IMSI may be configured in a second USIM in the UE or may be configured in a serving ground PLMN and mapped from a common IMSI used by the UE for S&F mode and normal mode.

When a satellite supporting normal mode loses its only or last feeder link, support for normal mode shall cease and any UE accessing the satellite is treated as entering a coverage gap. Depending on operator policy and regulatory requirements, the satellite, if S&F capable, may immedately or at a later time start support for S&F mode as described in clause 4.13.9.1.

When a satellite that is supporting S&F mode obtains a feeder link, support for S&F mode may cease immediately, at a later time or not at all depending on operator policy and regulatory requirements. Shortly before support for S&F mode will cease, the onboard MME should complete ongoing NAS procedures for S&F mode and may perform an S1 release or detach for any UE in ECM Connected state for S&F mode. Depending on operator policy, regulator requirements and satellite capability, the satellite may then immedately or at a later time start support for normal mode.

# * * * * Next change * * * *

## O.3.2	Architecture and Principles of Operation

An example architecture of Model B is shown in Figure O.3.2-1. Each satellite contains the functionally of an eNB plus a full EPC that can include an MME, SGW, PGW, HSS, E-SMLC, SMSC etc. Each satellite further includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE. There is also store and forward functionality on the ground that can be part of, or connected to, an NTN Gateway and that contains the proxy functionality.

NOTE:	This architecture does not support normal roaming. However, a UE may have an HPLMN different to the serving EPC in a satellite and the S&F Function.

Figure O.3.2-1: Example Architecture of Model B

Editor's note:	Security aspects of the usage of the S&F Proxy are FFS.

For Model B, the signalling and procedures used between a UE and satellite to support UE access and UE services in S&F operation are the same as used between a UE and serving PLMN for normal satellite access except for the differences described in clause 4.13.9. A UE thus sees the onboard eNB and EPC as being equivalent to a serving PLMN for normal operation. Some PLMN services cannot be available for S&F operation due to subscription restriction for a UE or lack of support by the onboard eNB and/or EPC.

A UE accesses and attaches to a satellite for Model B as described in clause 4.13.9. The onboard EPC can obtain the UE location and verify that the UE is allowed to access the PLMN that was selected by the UE. The UE can then establish PDN connections to the onboard EPC and utilise them for the services that are supported in S&F operation.

Depending on what is supported by the onboard eNB and EPC, the UE can perform mobile originated (MO) transactions such as sending SMS, and sending data (e.g. using IP or non-IP protocols) using User Plane or Control Plane CIoT EPS optimisation. Each MO transaction is transferred by the onboard EPC to the onboard endpoint proxy which stores transaction data and signalling (e.g. SMS, data,) and associated protocol and remote endpoint data. The onboard endpoint proxy also returns responses to the UE at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeouts and enable a user (if participating) to be aware of the one way communication status.

Shortly before satellite coverage will be lost, the onboard EPC can detach the UE.

After the UE loses coverage and when the satellite obtains a feeder link to a ground based portion of the serving PLMN selected by the UE for the Attach, the satellite (or the onboard endpoint proxy) transfers data for the UE (e.g. the IMSI and last known UE location), and all of the stored data and signalling for the UE MO transactions, to an S&F function for the serving PLMN. The S&F function might contain a proxy that stores the data and signalling for the UE MO transactions, performs or instigates an update location with the UE’s HPLMN if the S&F Function is not part of this HPLMN, and then forwards the data and signalling for each MO transaction to the associated remote endpoint. The proxy can also receive and store data and signalling for mobile terminated (MT) transactions that can be returned by the remote endpoints to the UE.

The S&F function and proxy can simulate continuous reachability of the UE. The MT transaction data and signalling for the UE (e.g. the IMSI) received and stored by the proxy are transferred to one or more satellites that are expected to later provide coverage to the UE.

When MT data is transferred to the satellite(s), the onboard endpoint proxy subscribes to the reachability of the UE and optional PDN connectivity status related information for data transmission, e.g. T8 destination address for non-IP data, UE IP address for IP data. When the UE accesses and attaches to the satellite, the endpoint proxy sends MT data to the UE based on the notification from onboard EPC, e.g. sends non-IP data to the T8 destination address or replaces the target IP address for IP data to UE's IP address and sends to PGW.

Transition between S&F mode and normal mode can be supported as defined in clause 4.13.9.x, e.g. using a second USIM in the UE containing a second IMSI for S&F mode.

# * * * * End of changes * * * *