3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500105r01

# E-meeting, Jan 20 – 24, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0523 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Multi-hop Relay Reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - If a new U2N Relay is used after the relay reselection, IP address/prefix changes may happen. In this case, the current enhancement of Layer-2 link modification procedure for multi-hop U2N Relay is insufficient as the Remote UE IP Info allocated by old Root U2N Relay is reported to the new Root U2N Relay, leading to disruption of IP communication. - As described in clause 6.5.3, “The lower layer (e.g. PC5-RRC layer) indicates to an upper layer… together with … the path information, and …”, however the path information could not be provided by the lower layer. - Typo in 6.4.3.8.1, “Remote UE triggers the link modification procecedure to...” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that Intermediate Relay notifies its child relay(s) and Remote UE(s) about the link modification when it decides that the Root U2N Relay is changed. Clarify that Remote UE requests for new IP address/prefix based on the notification mentioned above. - Clarify that “the path information” is retrived from the link context based on PC5 Link Identifier rather than provided by lower layer. - Fix typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - IP address may not be correctly managed after relay reselection. - Misleading that the PC5-RRC layer stores the path information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.8.3, 6.5.3, 6.4.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.8.3	Relay reselection and mobility support

There are two types of situations for parent relay reselection, i.e. for a single 5G ProSe Remote UE and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs.

For a single 5G ProSe Remote UE, when the connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold, the 5G ProSe Remote UE triggers the reselection of the relays that offers the same service, identified by the same RSC.

Figure 6.4.3.8.3-1: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays without Root Relay change

Figure 6.4.3.8.3-1 depicts the Relay reselection operation for this situation. When the 5G ProSe Remote UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Intermediate UE-to-Network Relay-1, the 5G ProSe Remote UE performs relay reselection by using the relay discovery procedure in clause 6.3.2.5. The 5G ProSe Remote UE discovered the 5G ProSe Intermediate UE-to-Network Relay-2 that offers the same RSC. In case there are multiple 5G ProSe Intermediate UE-to-Network Relays discovered, the 5G ProSe Intermediate UE-to-Network Relay that offers connection to the same 5G ProSe UE-to-Network Relay is prioritized.

5G ProSe Remote UE establishes the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2 as specified in clause 6.4.3.8.1. After the connection establishment, the Intermediate UE-to-Network Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe UE-to-Network Relay regarding the 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is used, it switches the connection/routing context for the 5G ProSe Remote UE from the Layer-2 link with the 5G Intermediate UE-to-Network Relay-1 to that of the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2.

The 5G ProSe Remote UE tears down the link with the 5G ProSe Intermediate UE-to-Network Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Intermediate UE-to-Network Relay-1 to update the 5G ProSe UE-to-Network Relay to remove its association with 5G ProSe Remote UE.

If the same 5G ProSe UE-to-Network Relay is selected after the relay reselection, the 5G ProSe Remote UE can keep its IP address/prefix (in case of Layer-3 UE-to-Network relay).

If a new 5G ProSe UE-to-Network Relay is used after the relay reselection, the procedures as that described in clause 6.4.3.8.1 will be executed. In this case, IP address/prefix changes may happen (in case of Layer-3 UE-to-Network relay).

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure, when its connection with a parent Relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold.

If the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselection, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path. After receving the re-selection indication the 5G ProSe Remote UE may trigger the reselection of the relays as described above.

During the transmition of re-selection indication to 5G ProSe Remote UE, after receving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above.

Intermediate U2N Relay0

Intermediate U2N Relay1

Intermediate U2N Relay2

Intermediate U2N Relay3

Remote UE1

U2N Relay1

U2N Relay2

Figure 6.4.3.8.3-2: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays with Root Relay change

When the 5G ProSe Intermediate Relay-1 has lost the connection or has an imminent connection loss/deteriorated connection with its parent 5G ProSe Intermediate Relay or the 5G ProSe UE-to-Network Relay, the 5G ProSe Intermediate Relay-1 performs relay reselection by using the relay discovery procedure in clasue 6.3.2.5. For Layer-3 UE-to-Network relay, if the 5G ProSe Intermediate Relay-1 decides a new 5G ProSe UE-to-Network Relay is used after the relay reselection, IP address/prefix changes may happen. In this case, the Intermediate UE-to-Network Relay-1 notifies its child relay(s) and/or Remote UE(s) about the change of Root 5G ProSe UE-to-Network Relay before sending Remote User ID and Remote UE Info to its parent Relay. The 5G ProSe Intermediate Relay-1 sends Link Modification Request message to its child relay(s) and Remote UE(s) including an indication that the Root UE-to-Network Relay is changed. The nofitication triggers the Remote UE to request for a new IP address/prefix from the new Root 5G ProSe UE-to-Network Relay.

# * * * * Second change * * * *

### 6.5.3	5G ProSe UE-to-Network Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE keeps performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-Network Relay for relay reselection.

The 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 are used to discover available 5G ProSe UE-to-Network Relays for 5G ProSe UE-to-Network Relay reselection.

For 5G ProSe Multi-hop UE-to-Network Relay, relay reselection is triggered if a 5G ProSe UE-to-Network Relay or Intermediate UE-to-Network Relay is no longer available, if a connection with the previous or next relay is lost, or if the link quality drops below a configured threshold. The path failure (connection lost or link quality drop) may be detected by a lower layer (e.g. PC5-RRC layer). The lower layer (e.g. PC5-RRC layer) indicates to an upper layer (e.g. PC5-S signalling layer) about the detected failure, together with the information on the detected failure/eventand the PC5 Link Identifier for the unicast link (which can be used to retrieve the corresponding UE ID(s) and path information from the link context).

For 5G ProSe Multi-hop UE-to-Network Relay reselection is performed as described in clause 6.4.3.8.3 for model A discovery and clause 6.4.3.9.3 for model B discovery.

# * * * * Third change * * * *

##### 6.4.3.8.1	Layer-2 Link establishment and management

Each of the 5G ProSe Intermediate UE-to-Network Relay establishes a Layer-2 Link with its immediate parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation.

Layer-2 link management procedures as defined in clause 6.4.3.6 for 5G ProSe UE-to-Network Relay operation are used. When establishing the Layer-2 link with its parent relay (either a 5G ProSe UE-to-Network Relay or another 5G ProSe Intermediate UE-to-Network Relay), the 5G ProSe Intermediate UE-to-Network Relay takes the role of the 5G ProSe Remote UE and its parent relay takes the role of 5G ProSe UE-to-Network Relay. The Layer-2 link is established per RSC.

The 5G ProSe Remote UE uses the procedure defined in clause 6.4.5.6 to establish a Layer-2 Link with the selected 5G ProSe Intermediate UE-to-Network Relay. The Intermediate UE-to-Network Relay takes the role of of 5G ProSe UE-to-Network Relay in the procedure.

NOTE:	In the case of Layer-3 UE-to-Network Relay operation, only the root 5G ProSe UE-to-Network Relay needs to establish the PDU session with the network.

When an Intermediate UE-to-Network Relay has a child Intermediate UE-to-Network Relay or a 5G ProSe Remote UE connected, it updates the Layer-2 link with its parent relay, using the procedure defined in clause 6.4.3.4, with the following enhancements:

-	The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info, hop-count) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clause 6.5.1.1. The hop-count corresponds to the PC5 hops of the Remote UE from the network.

The Remote UE triggers the link modification procedure to its parent relay after obtaining the IP address/prefix from the 5G ProSe UE-to-Network Relay.

An Intermediate UE-to-Network Relay triggers a link modification procedure towards its parent relay if a new Remote User ID is received from its child relay during the link modification procedure.

The 5G ProSe UE-to-Network Relay's Remote UE reporting procedure includes all the Remote UE information reported by its child relays.

# * * * * End of changes * * * *